3GPP TSG-CT WG1 Meeting #124-e	C1-203914

# Electronic meeting, 2-10 June 2020													was C1-203685

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 0174 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding mcdata id in signalling payload for sender of the data in MCData media plane (Session) communication. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCImp-MCDATA-CT |  |  |  |  |  | Date: |  |  | 2020-05-22 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No procedure to convey the data sender information while in MCData media plane (session) communications. Once the communication is established any participants can send the data and the information about sender is missing in the SDS, FD, SDS Disposition notification and FD Disposition notification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Included the new field to carry the data sender information and which is applicable only when sending data in MCData media plane (session) communications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In a group MCData communication using media plane, the recipients of the data will not be able to identify from whom the data is being received and which is important for MCData client to present it to user. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.1.2.1, 15.1.3.1, 15.1.5.1, 15.1.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1 : IEI value of ‘Sender MCData user ID’ information element is modified from ‘80’ to ‘51’ as it was overlapping with IEI value ‘8-‘ of ‘SDS disposition request type’ information element. Improved Consequences text added. |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

#### 15.1.2.1	Message definition

This message is sent by the UE to other UEs when sending an SDS data payload. This message provides the signalling content related to the SDS data payload. For the contents of the message see Table 15.1.2.1-1.

Message type:	SDS SIGNALLING PAYLOAD

Direction:			UE to other UEs (can be via network)

# Table 15.1.2.1-1: SDS SIGNALLING PAYLOAD message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | SDS signalling payload message identity | Message type 15.2.2 | M | V | 1 |
|  | Date and time | Date and time 15.2.8 | M | V | 5 |
|  | Conversation ID | Conversation ID 15.2.9 | M | V | 16 |
|  | Message ID | Message ID 15.2.10 | M | V | 16 |
| 21 | InReplyTo message ID | InReplyTo message ID 15.2.11 | O | TV | 17 |
| 22 | Application ID | Application ID 15.2.7 | O | TV | 2 |
| 8- | SDS disposition request type | SDS disposition request type 15.2.3 | O | TV | 1 |
| 7D | Extended application ID | Extended application ID 15.2.24 | O | TLV-E | 3-x |
| 7E | User location | User location 15.2.25 | O | TLV-E | 4-x |
| 51 | Sender MCData user ID | MCData user ID 15.2.15 | O | TLV-E | 4-x |

#### 15.1.3.1	Message definition

This message is sent by the UE to other UEs when sending an FD data payload. This message provides the signalling content related to the FD data payload. For the contents of the message see Table 15.1.3.1-1.

Message type:	FD SIGNALLING PAYLOAD

Direction:			UE to other UEs (via the network)

# Table 15.1.3.1-1: FD SIGNALLING PAYLOAD message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | FD signalling payload message identity | Message type 15.2.2 | M | V | 1 |
|  | Date and time | Date and time 15.2.8 | M | V | 5 |
|  | Conversation ID | Conversation ID 15.2.9 | M | V | 16 |
|  | Message ID | Message ID 15.2.10 | M | V | 16 |
| 21 | InReplyTo message ID | InReplyTo message ID 15.2.11 | O | TV | 17 |
| 22 | Application ID | Application ID 15.2.7 | O | TV | 2 |
| 9- | FD disposition request type | FD disposition request type 15.2.4 | O | TV | 1 |
| A- | Mandatory download | Mandatory download 15.2.16 | O | TV | 1 |
| 78 | Payload | Payload 15.2.13 | O | TLV-E | 3-x |
| 79 | Metadata | Metadata 15.2.17 | O | TLV-E | 3-x |
| 7D | Extended application ID | Extended application ID 15.2.24 | O | TLV-E | 3-x |
| 51 | Sender MCData user ID | MCData user ID 15.2.15 | O | TLV-E | 4-x |

#### 15.1.5.1	Message definition

This message is sent by the UE to another other UE to share SDS disposition information. For the contents of the message see Table 15.1.5.1-1.

Message type:	SDS NOTIFICATION

Direction:			UE to other UEs (can be via network)

# Table 15.1.5.1-1: SDS NOTIFICATION message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | SDS notification message identity | Message type 15.2.2 | M | V | 1 |
|  | SDS disposition notification type | SDS disposition notification type 15.2.5 | M | V | 1 |
|  | Date and time | Date and time 15.2.8 | M | V | 5 |
|  | Conversation ID | Conversation ID 15.2.9 | M | V | 16 |
|  | Message ID | Message ID 15.2.10 | M | V | 16 |
| 22 | Application ID | Application ID 15.2.7 | O | TV | 2 |
| 7D | Extended application ID | Extended application ID 15.2.24 | O | TLV-E | 3-x |
| 51 | Sender MCData user ID | MCData user ID 15.2.15 | O | TLV-E | 4-x |

#### 15.1.6.1	Message definition

This message is sent by the UE to another other UE to share FD disposition information. For the contents of the message see Table 15.1.6.1-1.

Message type:	FD NOTIFICATION

Direction:			UE to other UEs (via the network)

# Table 15.1.6.1-1: FD NOTIFICATION message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | FD notification message identity | Message type 15.2.2 | M | V | 1 |
|  | FD disposition notification type | FD disposition notification type 15.2.6 | M | V | 1 |
|  | Date and time | Date and time 15.2.8 | M | V | 5 |
|  | Conversation ID | Conversation ID 15.2.9 | M | V | 16 |
|  | Message ID | Message ID 15.2.10 | M | V | 16 |
| 22 | Application ID | Application ID 15.2.7 | O | TV | 2 |
| 7D | Extended application ID | Extended application ID 15.2.24 | O | TLV-E | 3-x |
| 51 | Sender MCData user ID | MCData user ID 15.2.15 | O | TLV-E | 4-x |

* * * * * * * END CHANGES * * * * * * *