3GPP TSG-CT WG1 Meeting #124-e	C1-203848

Electronic meeting, 2-10 June 2020	(was C1-203175)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.483 | CR | 0072 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add PointCoordinate, Speed, Heading nodes in 5.2.48W |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MONASTERY2 |  |  |  |  |  | Date: |  |  | 2 June 2020 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the node structure for PolygonArea and EllipsoidArcArea, the node after Corner and Center is PointCoordinate in Figure 5.1.5 and in other areas of TS 24.483. However, the PointCoordinate node is missing in the 5.2.48W… subclauses. Also, the Speed and Heading nodes in Figure 5.1.5 and other parts of TS 24.483 are present, but are missing in the 5.2.48W… subclauses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The PointCoordinate, Speed, and Heading nodes are added in the correct subclauses in the 5.2.48W… subclause structure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incconsistent specification of the node structures for EnterSpecificArea and ExitSpecificArea as shown in Figure 5.1.5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.48B4A19, 5.2.48B4A20, 5.2.48B4A22, 5.2.48B4A23, 5.2.48B4A39, 5.2.48B4A40, 5.2.48B4A42, 5.2.48B4A43, 5.2.48B4B19, 5.2.48B4B20, 5.2.48B4B22, 5.2.48B4B23, 5.2.48B4B39, 5.2.48B4B40, 5.2.48B4B42, 5.2.48B4B43, 5.2.48W6A3A (new), 5.2.48W6A4, 5.2.48W6A5, 5.2.48W6A7A (new), 5.2.48W6A8, 5.2.48W6A9, 5.2.48W6A12A (new), 5.2.48W6A12B (new), 5.2.48W6A12C (new), 5.2.48W6A12D (new), 5.2.48W6A12E (new), 5.2.48W6A12F (new), 5.2.48W6A15A (new), 5.2.48W6A16, 5.2.48W6A17, 5.2.48W6A19A (new), 5.2.48W6A20, 5.2.48W6A21, 5.2.48W6A24A (new), 5.2.48W6A24B (new), 5.2.48W6A24C (new), 5.2.48W6A24D (new), 5.2.48W6A24E (new), 5.2.48W6A24F (new), 5.2.48W6B3A (new), 5.2.48W6B4, 5.2.48W6B5, 5.2.48W6B7A (new), 5.2.48W6B8, 5.2.48W6B9, 5.2.48W6B12A (new), 5.2.48W6B12B (new), 5.2.48W6B12C (new), 5.2.48W6B12D (new), 5.2.48W6B12E (new), 5.2.48W6B12F (new), 5.2.48W6B15A (new), 5.2.48W6B16, 5.2.48W6B17, 5.2.48W6B19A (new), 5.2.48W6B20, 5.2.48W6B21, 5.2.48W6B24A (new), 5.2.48W6B24B (new), 5.2.48W6B24C (new), 5.2.48W6B24D (new), 5.2.48W6B24E (new), 5.2.48W6B24F (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: - Removed subclauses with no changes. - Added subclauses where existing text specifies only "Minimum" and "Maximum" for Speed and Heading, instead of "MinimumSpeed", "MaximumSpeed,"MinimumHeading", and "MaximumHeading". - Specified the heading range as "0-359". - Specified the speed as a "non-negative integer in units of kilometers/hour". - Changed the status field for "MinimumSpeed", "MaximumSpeed,"MinimumHeading", and "MaximumHeading" from Optional to Required, since they are required if the parent node exists. |  |  |  |  |  |  |  |  |

* * * * * FIRST CHANGE * * * * *

### 5.2.48B4A19	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MinimumSpeed

Table 5.2.48B4A19.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48B4A20	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MaximumSpeed

Table 5.2.48B4A20.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

* * * * * NEXT CHANGE * * * * *

### 5.2.48B4A22	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MinimumHeading

Table 5.2.48B4A22.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48B4A23	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MaximumHeading

Table 5.2.48B4A23.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

* * * * * NEXT CHANGE * * * * *

### 5.2.48B4A39	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MinimumSpeed

Table 5.2.48B4A39.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48B4A40	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MaximumSpeed

Table 5.2.48B4A40.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

* * * * * NEXT CHANGE * * * * *

### 5.2.48B4A42	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MinimumHeading

Table 5.2.48B4A42.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48B4A43	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MaximumHeading

Table 5.2.48B4A43.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForAffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

* * * * * NEXT CHANGE * * * * *

### 5.2.48B4B19	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MinimumSpeed

Table 5.2.48B4B19.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48B4B20	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MaximumSpeed

Table 5.2.48B4B20.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

* * * * * NEXT CHANGE * * * * *

### 5.2.48B4B22	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MinimumHeading

Table 5.2.48B4B22.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48B4B23	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MaximumHeading

Table 5.2.48B4B23.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/EnterSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

* * * * * NEXT CHANGE * * * * *

### 5.2.48B4B39	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MinimumSpeed

Table 5.2.48B4B39.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48B4B40	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MaximumSpeed

Table 5.2.48B4B40.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

* * * * * NEXT CHANGE * * * * *

### 5.2.48B4B42	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MinimumHeading

Table 5.2.48B4B42.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48B4B43	/<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MaximumHeading

Table 5.2.48B4B43.1: /<x>/<x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/MCPTTGroupList/<x>/Entry/RulesForDeaffiliation/ListOfLocationCriteria/<x>/Entry/ExitSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

* * * * * NEXT CHANGE * * * * *

### 5.2.48W6A3A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForActivation/EnterSpecificArea/PolygonArea/Corner/PointCoordinateType

Table 5.2.48W6A3A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/PolygonArea/Corner/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/PolygonArea/Corner/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48W6A4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForActivation/EnterSpecificArea/PolygonArea/Corner/PointCoordinateType/Longitude

Table 5.2.48W6A4.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/ PolygonArea/Corner/PointCoordinateType/Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/PolygonArea/ CornerPointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6A5	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForActivation/EnterSpecificArea/PolygonArea/Corner/PointCoordinateType/Latitude

Table 5.2.48W6A5.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/ PolygonArea/Corner/PointCoordinateType/Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/PolygonArea/ Corner/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

* * * * * NEXT CHANGE * * * * *

### 5.2.48W6A7A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

Table 5.2.48W6A7A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48W6A8	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForActivation/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude

Table 5.2.48W6A8.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

* * * * * NEXT CHANGE * * * * *

### 5.2.48W6A9	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForActivation/EnterSpecificArea/
EllipsoidArcArea/Center/PointCoordinateType/Latitude

Table 5.2.48W6A9.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

* * * * * NEXT CHANGE * * * * *

### 5.2.48W6A12A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/Speed

Table 5.2.48W6A12A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/Speed

| /<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 5.2.48W6A12B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/Speed/MinimumSpeed

Table 5.2.48W6A12B.1: 	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48W6A12C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/Speed/MaximumSpeed

Table 5.2.48W6A12C.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48W6A12D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/Heading

Table 5.2.48W6A12D.1: 	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/Heading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 5.2.48W6A12E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/Heading/MinimumHeading

Table 5.2.48W6A12E.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48W6A12F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/Heading/MaximumHeading

Table 5.2.48W6A12F.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/EnterSpecificArea/Heading/MaximmHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

* * * * * NEXT CHANGE * * * * *

### 5.2.48W6A15A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/ExitSpecificArea/PolygonArea/
Corner/PointCoordinateType

Table 5.2.48W6A15A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/PolygonArea/Corner/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/PolygonArea/Corner/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the type of the coordinates. |  |  |  |  |

### 5.2.48W6A16	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForActivation/ExitSpecificArea/PolygonArea/Corner/PointCoordinateType/Longitude

Table 5.2.48W6A16.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/ PolygonArea/Corner/PointCoordinateType/Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/PolygonArea/ Corner/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6A17	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForActivation/ExitSpecificArea/PolygonArea/
Corner/PointCoordinateType/Latitude

Table 5.2.48W6A17.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/ PolygonArea/Corner/PointCoordinateType/Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/PolygonArea/ Corner/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

* * * * * NEXT CHANGE * * * * *

### 5.2.48W6A19A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

Table 5.2.48W6A19A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48W6A20	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForActivation/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude

Table 5.2.48W6A20.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6A21	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForActivation/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude

Table 5.2.48W6A21.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

* * * * * NEXT CHANGE * * * * *

### 5.2.48W6A24A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/
LocationCriteriaForActivation/ExitSpecificArea/Speed

Table 5.2.48W6A24A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/Speed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 5.2.48W6A24B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/Speed/MinimumSpeed

Table 5.2.48W6A24B.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48W6A24C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/Speed/MaximumSpeed

Table 5.2.48W6A24C.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48W6A24D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/Heading

Table 5.2.48W6A24D.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/Heading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 5.2.48W6A24E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/Heading/MinimumHeading

Table 5.2.48W6A24E.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48W6A24F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/Heading/MaximumHeading

Table 5.2.48W6A24E.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForActivation/ExitSpecificArea/Heading/Maximum umHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

* * * * * NEXT CHANGE * * * * *

### 5.2.48W6B3A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/PolygonArea/Corner/PointCoordinateType

Table 5.2.48W6B3A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/PolygonArea/Corner/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/PolygonArea/Corner/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48W6B4	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForDeactivation/EnterSpecificArea/PolygonArea/Corner/PointCoordinateType/Longitude

Table 5.2.48W6B4.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/ PolygonArea/Corner/PointCoordinateType/Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/PolygonArea/ Corner/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6B5	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForDeactivation/EnterSpecificArea/PolygonArea/Corner/PointCoordinateType/Latitude

Table 5.2.48W6B5.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/ PolygonArea/Corner/PointCoordinateType/Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/PolygonArea/ Corner/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

* * * * * NEXT CHANGE * * * * *

### 5.2.48W6B7A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

Table 5.2.48W6B7A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48W6B8	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForDeactivation/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude

Table 5.2.48W6B8.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6B9	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForDeactivation/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude

Table 5.2.48W6B9.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

* * * * * NEXT CHANGE * * * * *

### 5.2.48W6B12A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/Speed

Table 5.2.48W6B12A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/Speed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 5.2.48W6B12B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/Speed/MinimumSpeed

Table 5.2.48W6B12B.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48W6B12C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/Speed/MaximumSpeed

Table 5.2.48W6B12C.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48W6B12D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/Heading

Table 10.2.97B3C18.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/Heading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 5.2.48W6B12E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/Heading/MinimumHeading

Table 5.2.48W6B12E.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48W6B12F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/Heading/MaximumHeading

Table 5.2.48W6B12F.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/EnterSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

* * * * * NEXT CHANGE * * * * *

### 5.2.48W6B15A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/PolygonArea/Corner/PointCoordinateType

Table 5.2.48W6B15A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/PolygonArea/Corner/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/PolygonArea/Corner/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48W6B16	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForDeactivation/ExitSpecificArea/PolygonArea/Corner/PointCoordinateType/Longitude

Table 5.2.48W6B16.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/ PolygonArea/Corner/PointCoordinateType/Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/PolygonArea/ Corner/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6B17	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForDeactivation/ExitSpecificArea/PolygonArea/Corner/PointCoordinateType/Latitude

Table 5.2.48W6B17.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/ PolygonArea/Corner/PointCoordinateType/Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/PolygonArea/ Corner/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of a corner. |  |  |  |  |

-	Values: 0-16777215

* * * * * NEXT CHANGE * * * * *

### 5.2.48W6B19A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

Table 5.2.48W6B19A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get, Replace |  |
|  | This interior node contains the coordinates of the center point of the ellipsoid arc. |  |  |  |  |

### 5.2.48W6B20	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForDeactivation/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude

Table 5.2.48W6B20.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Longitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Longitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the longitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

### 5.2.48W6B21	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/
Entry/LocationCriteriaForDeactivation/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude

Table 5.2.48W6B21.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/ EllipsoidArcArea/Center/PointCoordinateType/Latitude

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/EllipsoidArcArea/Center/PointCoordinateType/Latitude |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the latitudinal coordinate of the center. |  |  |  |  |

-	Values: 0-16777215

* * * * * NEXT CHANGE * * * * *

### 5.2.48W6B24A	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/Speed

Table 5.2.48W6B24A.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/Speed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/Speed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the speed. |  |  |  |  |

### 5.2.48W6B24B	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/Speed/MinimumSpeed

Table 5.2.48W6B24B.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/Speed/MinimumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/Speed/MinimumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48W6B24C	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/Speed/MaximumSpeed

Table 5.2.48W6B24C.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/Speed/MaximumSpeed

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/Speed/MaximumSpeed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum speed. |  |  |  |  |

-	Values: non-negative integer in units of kilometers/hour.

### 5.2.48W6B24D	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/Heading

Table 5.2.48W6B24D.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/Heading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/Heading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get, Replace |  |
|  | This interior node contains the heading. |  |  |  |  |

### 5.2.48W6B24E	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/Heading/MinimumHeading

Table 5.2.48W6B24E.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/Heading/MinimumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/Heading/MinimumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the minimum heading. |  |  |  |  |

-	Values: 0-359

### 5.2.48W6B24F	/<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/Heading/MaximumHeading

Table 5.2.48W6B24F.1: /<x>/<x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/Heading/MaximumHeading

| <x>/OnNetwork/FunctionalAliasList/<x>/Entry/LocationCriteriaForDeactivation/ExitSpecificArea/Heading/MaximumHeading |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get, Replace |  |
|  | This leaf node contains the maximum heading. |  |  |  |  |

-	Values: 0-359

* * * * * END CHANGES * * * * *