3GPP TSG-CT WG1 Meeting #123-e	C1-202083

Electronic meeting, 16-24 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 2033 | rev | - | Current version: | 16.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of certain erroneous Information Element Identifiers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-04-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Information Element Identifiers (IEI) of three 5GMM (Mobility Management) messages are erroneous and need to be corrected. - In the Registration Request message, the newly added IEI for the “N5GC Indication” IE follows a “Type 1” IE, whereas the actual IE is a Type 2 - A dash “-“ is missing in the IEI of Request Type IE in the NAS UL Transport message - The “UE radio capability ID deletion indication” in the Configuation Update Command message is a Type 1 IE. However, its IEI does not follow the rule for Type 1 IEs. |  |  |  |  |  |  |  |  |
| I |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The newly added IEI for the “N5GC Indication” IE is changed from “A1” to “XX” (awaiting the rapporteur’s IE assignment) - A dash “-“ is added to the IEI of Request Type IE in the NAS UL Transport message - The IEI of Radio Capability ID deletion indication is changed from “68” to “X-“ (awaiting Rapporteur’s IE assignment) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Erroneous IEI values will remain in the specification leading to misinterpretation and tratment of the information elements at the receiving side |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6; 8.2.10; 8.2.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 8.2.6	Registration request

#### 8.2.6.1	Message definition

The REGISTRATION REQUEST message is sent by the UE to the AMF. See table 8.2.6.1.1.

Message type:	REGISTRATION REQUEST

Significance:	dual

Direction:		UE to network

Table 8.2.6.1.1: REGISTRATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended Protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Registration request message identity | Message type 9.7 | M | V | 1 |
|  | 5GS registration type | 5GS registration type 9.11.3.7 | M | V | 1/2 |
|  | ngKSI | NAS key set identifier 9.11.3.32 | M | V | 1/2 |
|  | 5GS mobile identity | 5GS mobile identity 9.11.3.4 | M | LV-E | 6-n |
| C- | Non-current native NAS key set identifier | NAS key set identifier 9.11.3.32 | O | TV | 1 |
| 10 | 5GMM capability | 5GMM capability 9.11.3.1 | O | TLV | 3-15 |
| 2E | UE security capability | UE security capability 9.11.3.54 | O | TLV | 4-10 |
| 2F | Requested NSSAI | NSSAI 9.11.3.37 | O | TLV | 4-74 |
| 52 | Last visited registered TAI | 5GS tracking area identity 9.11.3.8 | O | TV | 7 |
| 17 | S1 UE network capability | S1 UE network capability 9.11.3.48 | O | TLV | 4-15 |
| 40 | Uplink data status | Uplink data status 9.11.3.57 | O | TLV | 4-34 |
| 50 | PDU session status | PDU session status 9.11.3.44 | O | TLV | 4-34 |
| B- | MICO indication | MICO indication 9.11.3.31 | O | TV | 1 |
| 2B | UE status | UE status 9.11.3.56 | O | TLV | 3 |
| 77 | Additional GUTI | 5GS mobile identity 9.11.3.4 | O | TLV-E | 14 |
| 25 | Allowed PDU session status | Allowed PDU session status 9.11.3.13 | O | TLV | 4-34 |
| 18 | UE's usage setting | UE's usage setting 9.11.3.55 | O | TLV | 3 |
| 51 | Requested DRX parameters | 5GS DRX parameters 9.11.3.2A | O | TLV | 3 |
| 70 | EPS NAS message container | EPS NAS message container 9.11.3.24 | O | TLV-E | 4-n |
| 74 | LADN indication | LADN indication 9.11.3.29 | O | TLV-E | 3-811 |
| 8- | Payload container type | Payload container type 9.11.3.40 | O | TV | 1 |
| 7B | Payload container | Payload container 9.11.3.39 | O | TLV-E | 4-65538 |
| 9- | Network slicing indication | Network slicing indication 9.11.3.36 | O | TV | 1 |
| 53 | 5GS update type | 5GS update type 9.11.3.9A | O | TLV | 3 |
| 41 | Mobile station classmark 2 | Mobile station classmark 2 9.11.3.31C | O | TLV | 5 |
| 42 | Supported codecs | Supported codec list 9.11.3.51A | O | TLV | 5-n |
| 71 | NAS message container | NAS message container 9.11.3.33 | O | TLV-E | 4-n |
| 60 | EPS bearer context status | EPS bearer context status 9.11.3.23A | O | TLV | 4 |
| 6E | Requested extended DRX parameters | Extended DRX parameters 9.11.3.26A | O | TLV | 3 |
| 6A | T3324 value | GPRS timer 3 9.11.2.5 | O | TLV | 3 |
| 67 | UE radio capability ID | UE radio capability ID 9.11.3.68 | O | TLV | 3-n |
| 35 | Requested mapped NSSAI | Mapped NSSAI 9.11.3.31B | O | TLV | 3-42 |
| 48 | Additional information requested | Additional information requested 9.11.3.12A | O | TLV | 3 |
| 1A | Requested WUS assistance information | WUS assistance information 9.11.3.71 | O | TLV | 3-n |
| XX | N5GC indication | N5GC indication 9.11.3.72 | O | T | 1 |

Next Change

### 8.2.10	UL NAS transport

#### 8.2.10.1	Message definition

The UL NAS TRANSPORT message transports message payload and associated information to the AMF. See table 8.2.10.1.1.

Message type:	UL NAS TRANSPORT

Significance:	dual

Direction:		UE to network

Table 8.2.10.1.1: UL NAS TRANSPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | UL NAS TRANSPORT message identity | Message type 9.7 | M | V | 1 |
|  | Payload container type | Payload container type 9.11.3.40 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Payload container | Payload container 9.11.3.39 | M | LV-E | 3-65537 |
| 12 | PDU session ID | PDU session identity 2 9.11.3.41 | C | TV | 2 |
| 59 | Old PDU session ID | PDU session identity 2 9.11.3.41 | O | TV | 2 |
| 8- | Request type | Request type 9.11.3.47 | O | TV | 1 |
| 22 | S-NSSAI | S-NSSAI 9.11.2.8 | O | TLV | 3-10 |
| 25 | DNN | DNN 9.11.2.1B | O | TLV | 3-102 |
| 24 | Additional information | Additional information 9.11.2.1 | O | TLV | 3-n |
| A- | MA PDU session information | MA PDU session information 9.11.3.31A | O | TV | 1 |
| F- | Release assistance indication | Release assistance indication 9.11.3.46A | O | TV | 1 |

Next Change

### 8.2.19	Configuration update command

#### 8.2.19.1	Message definition

The CONFIGURATION UPDATE COMMAND message is sent by the AMF to the UE. See table 8.2.19.1.1.

Message type:	CONFIGURATION UPDATE COMMAND

Significance:	dual

Direction:		network to UE

Table 8.2.19.1.1: CONFIGURATION UPDATE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Configuration update command message identity | Message type 9.7 | M | V | 1 |
| D- | Configuration update indication | Configuration update indication 9.11.3.18 | O | TV | 1 |
| 77 | 5G-GUTI | 5GS mobile identity 9.11.3.4 | O | TLV-E | 14 |
| 54 | TAI list | 5GS tracking area identity list 9.11.3.9 | O | TLV | 9-114 |
| 15 | Allowed NSSAI | NSSAI 9.11.3.37 | O | TLV | 4-74 |
| 27 | Service area list | Service area list 9.11.3.49 | O | TLV | 6-114 |
| 43 | Full name for network | Network name 9.11.3.35 | O | TLV | 3-n |
| 45 | Short name for network | Network name 9.11.3.35 | O | TLV | 3-n |
| 46 | Local time zone | Time zone 9.11.3.52 | O | TV | 2 |
| 47 | Universal time and local time zone | Time zone and time 9.11.3.53 | O | TV | 8 |
| 49 | Network daylight saving time | Daylight saving time 9.11.3.19 | O | TLV | 3 |
| 79 | LADN information | LADN information 9.11.3.30 | O | TLV-E | 3-1715 |
| B- | MICO indication | MICO indication 9.11.3.31 | O | TV | 1 |
| 9- | Network slicing indication | Network slicing indication 9.11.3.36 | O | TV | 1 |
| 31 | Configured NSSAI | NSSAI 9.11.3.37 | O | TLV | 4-146 |
| 11 | Rejected NSSAI | Rejected NSSAI 9.11.3.46 | O | TLV | 4-42 |
| 76 | Operator-defined access category definitions | Operator-defined access category definitions 9.11.3.38 | O | TLV-E | 3-n |
| F- | SMS indication | SMS indication 9.11.3.50A | O | TV | 1 |
| 6C | T3447 value | GPRS timer 3 9.11.2.5 | O | TLV | 3 |
| 75 | CAG information list | CAG information list 9.11.3.18A | O | TLV-E | 3-n |
| 67 | UE radio capability ID | UE radio capability ID 9.11.3.68 | O | TLV | 3-n |
| XX | UE radio capability ID deletion indication | UE radio capability ID deletion indication 9.11.3.69 | O | TV | 1 |
| 44 | 5GS registration result | 5GS registration result 9.11.3.6 | O | TLV | 3 |
| 1B | Truncated 5G-S-TMSI configuration | Truncated 5G-S-TMSI configuration 9.11.3.70 | O | TLV | 3 |

Next Change