3GPP TSG-CT Meeting #88e	CP-201183

# E-Meeting, 29th June – 1st July 2020	(was C4-203618)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 0359 | rev | 2 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NRF Notifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell, Cisco |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson, Nokia, Nokia Shanghai Bell, Cisco |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc16 |  |  |  |  |  | Date: |  |  | 2020-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There may be a mismatch between the NF Profile data received in a discovery response and the subsequent notifications sent by NRF to subscribing NFs, when such NF Profile is changed. This is particularly harmful when those changes affect an attribute of type array, since the array indexes may differ between those received in the discovery response, and those received in the notification request from NRF. This issue also affects the creation of subscriptions, when the "notifCondition" attribute may be used to refer to specific elements of an array (e.g. a certain service in the nfServices array). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The NRF shall only send profile update notifications by replacing entire arrays, and not sending changes of individual array elements. - When receiving a subscription request that includes "notifCondition" attribute (i.e. a list of monitored/unmonitored attributes) referencing a specific array element (by its array index) it shall apply the condition to all elements of the same array. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NF profile data kept by NF consumers of the NRF may get corrupted, if the set of changes received in a notification are applied to the wrong array index. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.2.3, 6.1.6.2.16, 6.1.6.2.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible corrections (even when OpenAPI is not modified), with impacts on the following APIs: - TS29510_Nnrf_NFManagement.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 2 (to CT#88e): - Changes in A.2 are not needed since they are already present in the TS baseline |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.2.2.3	Accept-Encoding

The NRF should support gzip coding (see IETF RFC 1952 [30]) in HTTP requests and responses and indicate so in the Accept-Encoding header, as described in clause 6.9 of 3GPP TS 29.500 [4].

NF Service Consumers of the NFManagement API should support gzip coding in HTTP requests and responses and they should support gzip coding in the reception of notification requests sent by the NRF.

* * * Next Change * * * *

##### 6.1.6.2.16	Type: SubscriptionData

Table 6.1.6.2.16-1: Definition of type SubscriptionData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| nfStatusNotificationUri | Uri | M | 1 | Callback URI where the NF Service Consumer will receive the notifications from NRF. |
| reqNfInstanceId | NfInstanceId | O | 0..1 | If present, this IE shall contain the NF instance id of the NF service consumer. |
| subscrCond | SubscrCond | O | 0..1 | If present, this attributed shall contain the conditions identifying the set of NF Instances whose status is requested to be monitored. If this attribute is not present, it means that the NF Service Consumer requests a subscription to all NFs in the NRF (NOTE 1). |
| subscriptionId | string | C | 0..1 | Subscription ID for the newly created resource. This parameter shall be absent in the request to the NRF and shall be included by NRF in the response to the subscription creation request. Read-Only: true Pattern: "^([0-9]{5,6}-)?[^-]+$" |
| validityTime | DateTime | C | 0..1 | Time instant after which the subscription becomes invalid. This parameter may be sent by the client, as a hint to the server, but it shall be always sent back by the server (regardless of the presence of the attribute in the request) in the response to the subscription creation request. |
| reqNotifEvents | array(NotificationEventType) | O | 1..N | If present, this attribute shall contain the list of event types that the NF Service Consumer is interested in receiving. If this attribute is not present, it means that notifications for all event types are requested. |
| reqNfType | NFType | O | 0..1 | If included, this IE shall contain the NF type of the NF Service Consumer that is requesting the creation of the subscription. The NRF shall use it for authorizing the request, in the same way as the "requester-nf-type" is used in the NF Discovery service (see Table 6.2.3.2.3.1-1). When the subscription is for a set of NF Instances, the subscription may be accepted by NRF, but it shall only generate notifications from NF Instances whose authorization parameters allow the NF Service Consumer to access their services (NOTE 2). |
| reqNfFqdn | Fqdn | O | 0..1 | If included, this IE shall contain the FQDN of the NF Service Consumer that is requesting the creation of the subscription. The NRF shall use it for authorizing the request, in the same way as the "requester-nf-instance-fqdn" is used in the NF Discovery service (see Table 6.2.3.2.3.1-1). When the subscription is for a set of NF Instances, the subscription may be accepted by NRF, but it shall only generate notifications from NF Instances whose authorization parameters allow the NF Service Consumer to access their services (NOTE 2). |
| reqSnssais | array(Snssai) | O | 0..1 | If included, this IE shall contain the list of S-NSSAIs of the NF Service Consumer that is requesting the creation of the subscription. The NRF shall use it for authorizing the request, in the same way as the "requester-snssais" is used in the NF Discovery service (see Table 6.2.3.2.3.1-1). When the subscription is for a set of NF Instances, the subscription may be accepted by NRF, but it shall only generate notifications from NF Instances whose authorization parameters allow the NF Service Consumer to access their services (NOTE 2). |
| plmnId | PlmnId | O | 0..1 | If present, this attribute contains the target PLMN ID of the NF Instance(s) whose status is requested to be monitored. |
| notifCondition | NotifCondition | O | 0..1 | If present, this attribute contains the conditions that trigger a notification from NRF; this attribute shall only be present if the NF Service Consumer has subscribed to changes on the NF Profile (i.e., reqNotifEvents contains the value "NF_PROFILE_CHANGED", or reqNotifEvents attribute is absent). If this attribute is absent, it means that the NF Service Consumer does not indicate any restriction, or condition, on which attributes of the NF Profile shall trigger a notification from NRF. (NOTE X). |
| reqPlmnList | array(PlmnId) | C | 1..N | This IE shall be included when subscribing to NF services in a different PLMN. When included, this IE shall contain the PLMN ID(s) of the requester NF. |
| NOTE 1: The "subscription to all NFs" may be quite demanding in terms of resources in NRF and also in terms of network traffic of the resulting notifications, so it should be authorized by NRF under very strict policies (e.g. only to a specific requesting NF, as indicated by reqNfType and reqNfFqdn attributes). NOTE 2: The authorization parameters in NF Profile are those used by NRF to determine whether a given NF Instance / NF Service Instance can be discovered by an NF Service Consumer in order to consume its offered services (e.g. "allowedNfTypes", "allowedNfDomains", etc.). NOTE X: If the attributes to be monitored or excluded from monitoring, included as part of the "notifCondition" attribute, refer to a specific element of an array (e.g. they refer to a specifc array index of the "nfServices" attribute of the NFProfile), the NRF shall apply the same condition to all elements of the same array. |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.17	Type: NotificationData

Table 6.1.6.2.17-1: Definition of type NotificationData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| event | NotificationEventType | M | 1 | Notification type. It shall take the values "NF_REGISTERED", "NF_DEREGISTERED" OR "NF_PROFILE_CHANGED". |
| nfInstanceUri | Uri | M | 1 | Uri of the NF Instance (see clause 6.1.3.3.2) associated to the notification event. |
| nfProfile | NFProfile | C | 0..1 | New NF Profile or Updated NF Profile; it shall be present when the notification type is "NF_REGISTERED" or "NF_PROFILE_CHANGED". |
| profileChanges | array(ChangeItem) | C | 1..N | List of changes on the profile of the NF Instance associated to the notification event; it may be present when the notification type is "NF_PROFILE_CHANGED" (see NOTE 1, NOTE X). |
| NOTE 1: If "event" attribute takes the value "NF_PROFILE_CHANGED", then either "nfProfile" or "profileChanges" attributes shall be present, but not both. NOTE X: The NRF shall notify about NF Profile changes affecting attributes of type "array" only as a complete replacement of the whole array (i.e. it shall not notify about changes of individual array elements). |  |  |  |  |

EXAMPLE:	Notification payload sent from NRF when an NF Instance has changed its profile by updating the value of the "recoveryTime" attribute of its NF Profile, and updated any attribute of any of its NF Service Instances:

{

"event": "NF_PROFILE_CHANGED",

"nfInstanceUri": ".../nf-instances/4947a69a-f61b-4bc1-b9da-47c9c5d14b64",

"profileChanges": [

{

"op": "REPLACE",

"path": "/recoveryTime",

"newValue": "2018-12-30T23:20:50Z"

},

{

"op": "REPLACE",

"path": "/nfServices",

"newValue": [ ...new array content... ]

}

]

}

* * * End of Changes * * * *