3GPP TSG-CT WG3 Meeting #110-e	C3-203467

# E-Meeting, 2nd -11th  June 2020						                     (Revision of C3-203097)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.514 | CR | 0225 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Max bitrate of TSN QoS information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-06-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It was agreed that AF may provide max bit rate for the PCF to determine the GBR. (S2-2002730) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the max bitrate for TSC traffic in uplink and downlink. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PCF can’t determine the GBR of PCC rule. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.503 ... CR#0436 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Begin of Change * * * *

4.2.2.24	Provisioning of TSCAI input Information and QoS related data

If the "TimeSensitiveNetworking" feature is supported the AF may provide TSCAI input information and QoS related data to the PCF by the Npcf_PolicyAuthorization_Create service operation to describe the TSC traffic pattern and QoS characteristics for use in the 5G System.

To indicate the TSCAI input information of a TSN stream or aggregated set of TSN streams, the AF may include for the uplink flow direction (egress interface of the UE) in the "tscaiInputUl" attribute and/or for the downlink flow direction (ingress interface of the RAN) the "tscaiInputDl" attribute included in a media subcomponent entry of the "medSubComps" attribute included within a media component:

-	the time period between the start of two bursts in reference to the TSN GM encoded in the "periodicity" attribute; and

-	the arrival time of the first data burst in reference to the TSN GM encoded in the "burstArrivalTime" attribute.

The uplink and/or downlink flow direction of the TSN stream or aggregated set of TSN streams shall be encoded within the corresponding "MediaSubComponent" entries of the "medSubComps" attribute, in the "ethfDescs" attribute.

To indicate the TSC QoS related data of a TSN stream or aggregated set of TSN streams, the AF may include in the "tsnQos" attribute included in a media component entry of the "medComponents" attribute;

-	the maximum burst size encoded in the "maxTscBurstSize" attribute;

-	the maximum time a packet may be delayed encoded in the "tscPackDelay" attribute;

-	the TSC traffic priority in scheduling resources among other TSN streams encoded in the "tscPrioLevel" attribute.

The TSN AF may also include the max bitrates in uplink and downlink within the "marBwUl" attribute and the "marBwDl" attribute of the "medComponents" attribute respectively.

When all the TSN streams or aggregated set of TSN streams gathered in the same media component share the same TSCAI input information, the AF may include the "tscaiInputUl" attribute and/or the "tscaiInputDl" attribute at media component level. When "tscaiInputUl" attribute and/or the "tscaiInputDl" attribute are included at media component level and media subcomponent level, the value at media subcomponent level takes precedence.

The PCF shall reply to the AF as described in subclause 4.2.2.2.

The PCF shall check whether the received TSCAI input container and TSC QoS related data require to create PCC rules to provide the SMF with derived QoS characteristics and the received TSCAI input container. Provisioning of PCC rule(s) to the SMF shall be carried out as specified in 3GPP TS 29.512 [8].

* * * * End of Change * * * *