3GPP TSG-CT Meeting #88e	CP-201279

# E-Meeting, 29th June– 01st July 2020										    Revision of C4-203483

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.673 | CR | 0001 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multiple Coding Formats Support in RAC-Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia, Nokia Shanghai Bell, Qualcomm, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Samsung, Nokia, Nokia Shanghai Bell, Qualcomm, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS |  |  |  |  |  | Date: |  |  | 2020-06-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 3GPP TS 23.502 CR 2231 (S2-2002955) & TS 23.501 CR 2255 (S2-2003484) introduce support for multiple coding formats in RACS operation. This CR adds corresponding changes in Nucmf_UECapabilityManagement Service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of multiple Coding formats in DicEntryCreateData Addition of requested Coding Format in Get operation on Dictionary Entries collection /dic-entries Addition of requested Coding Format in Get operation on Individual Dictionary Entry /dic-entries/{dicEntryId} |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RACS operation as defined in Stage-2 will not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.2.2.2.1, 5.2.2.2.2, 5.2.2.3, 6.1.2.2.2, 6.1.2.4, 6.1.3.2.3.1, 6.1.3.3.3.1, 6.1.6.1, 6.1.6.2.2, 6.1.6.2.3, 6.1.6.2.4, 6.1.6.3.x, 6.1.6.5.2, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS23.502 CR 2231 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR introduces backward compatible new Feature to UECapabilityManagement API |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Updated CR Category, Added supporting companies, changed racFormat to rac-format, and corrected some typos. Rev2 : Updated ContentType for ueRadioCapabilityEPS, removed changes to CreateSubscription, made rac-format conditional, split ueRadioCapability into ueRadioCapabilityEPS & ueRadioCapabilityEPS in DicEntryData (Issues found during rappartuer implementation) |  |  |  |  |  |  |  |  |

****************************************First Change**********************************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[15]	3GPP TS 38.413: "NG Radio Access Network (NG-RAN); NG Application Protocol (NGAP)".

[16]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[x]	3GPP TS 36.413: “Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)”

****************************************Next Change*********************************************

##### 5.2.2.2.1	General

The Resolve service operation shall be used to retrieve UE Radio Access Capability Information from a dictionary entry stored in the UCMF, by a NF service consumer, e.g. an AMF, when:

-	it has received an unknown UE Radio Capability ID, which is either Manufacturer-assigned or PLMN-assigned;

The NF Service Consumer (e.g. AMF) shall retrieve UE Radio Access Capability Information by using the HTTP GET method as shown in Figure 5.2.2.2.1-1.

Figure: 5.2.2.2.1-1 Retrieve UE Radio Access Capability Information

1.	The NF Service Consumer, e.g. an AMF, shall send an HTTP GET request to the resource URI Dictionary Entries collection resource. The query parameters are: a Manufacturer-assigned or a PLMN-assigned UE Radio Capability ID, and the Coding format (e.g. 5GS or EPS) in which UE Radio Access Capability Information needs to be provided.

2a.	On success, "200 OK" shall be returned. The response body shall contain UE Radio Access Capability Information in the requested Coding format stored in the dictionary entry that is matching the UE Radio Capability ID in query, and may contain a PLMN-assigned UE Radio Capability ID if the query parameter is a Manufacturer-assigned UE Radio Capability ID and if the PLMN-assigned UE Radio Capability ID mechanism is used in the network.

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.3.3.2-3 shall be returned.  For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.3.3.2-2, where applicable.

****************************************Next Change*********************************************

##### 5.2.2.2.2	Retrieve a Dictionary Entry

The Resolve service operation may be used to retrieve a dictionary entry stored in the UCMF, by a NF service consumer, e.g. an AMF, when it has the Dictionary Entry ID available.

The NF Service Consumer (e.g. AMF) shall retrieve a Dictionary Entry by using the HTTP GET method as shown in Figure 5.2.2.2.1-1.

Figure 5.2.2.2.1-1 Retrieve a dictionary entry

1.	The NF Service Consumer, e.g. an AMF, shall send an HTTP GET request to an individual Dictionary Entry resource. The query parameter is the Coding Format (e.g. 5GS or EPS) in which UE Radio Access Capability Information needs to be provided.

2a.	On success, "200 OK" shall be returned. The response body shall the dictionary entry information for the given Dictionary Entry ID.

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.3.3.2-3 shall be returned.  For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.3.3.2-3, where applicable.

****************************************Next Change*********************************************

#### 5.2.2.3	Service Operation Assign

The Assign service operation shall be used to request the UCMF to create a new dictionary entry and assign a PLMN-assigned UE Radio Capability ID. The NF service consumer, e.g. an AMF, may consume the service by providing the UE Radio Access Capability Information retrieved from the UE, and Type Allocation Code of PEI of the UE, e.g. when it hasn't received any UE Radio Capability ID.

The NF Service Consumer (e.g. AMF) shall obtain the PLMN Assigned UE Radio Capability ID by using the HTTP POST method as shown in Figure 5.2.2.3.1-1.

Figure 5.2.2.3-1 Create a dictionary entry

1.	The NF Service Consumer shall send a POST request to the resource representing the Dictionary Entries collection resource of the UCMF. The payload body of the POST request shall contain:

-	a representation of the individual dictionary entry resource to be created;

-	the UE Radio Access Capability Information for the current radio configuration of the UE in 5GS format, or EPS format, or both the formats;

-	the Type Allocation Code of the PEI of the UE.

2a.	On success, "201 Created" shall be returned, the payload body of the POST response shall contain the representation describing the status of the request and the "Location" header shall be present and shall contain the URI of the created resource. When PLMN Assigned UE Radio Capability ID is used, the PLMN Assigned UE Radio Capability ID shall be included in DicEntryCreatedData.

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.2.3.2-3 shall be returned.  For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.3.3.2-2, where applicable. A UCMF configured to operate in Mode of Operation A (3GPPP TS 23.501, Clause 5.4.4.1a) shall reject the operation if the request does not contain UE Radio Access Capability Information in both the formats and the UCMF is not able to find match of the received UE Radio Access Capability Information in its database.

****************************************Next Change*********************************************

##### 6.1.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

Multipart messages shall also be supported (see clause 6.1.2.4) using the content type "multipart/related", comprising:

-	one JSON body part with the "application/json" content type; and

-	one or more binary body parts with 3gpp vendor specific content subtypes.

The 3gpp vendor specific content subtypes defined in Table 6.1.2.2.2-1 shall be supported.

Table 6.1.2.2.2-1: 3GPP vendor specific content subtypes

| content subtype | Description |
| --- | --- |
| vnd.3gpp.ngap | Binary encoded payload, encoding NG Application Protocol (NGAP) IEs, as specified in clause 9.3 of 3GPP TS 38.413 [15] (ASN.1 encoded). |
| vnd.3gpp.s1ap | Binary encoded payload, encoding S1 Application Protocol (S1AP) IEs, as specified in clause 9.2 of 3GPP TS 36.413 [x] (ASN.1 encoded). |
| NOTE: Using 3GPP vendor content subtypes allows to describe the nature of the opaque payload (e.g. NGAP or 5GS NAS information) without having to rely on metadata in the JSON payload. |  |

See clause 6.1.2.4 for the binary payloads supported in the binary body part of multipart messages.

****************************************Next Change*********************************************

#### 6.1.2.4	HTTP multipart messages

HTTP multipart messages shall be supported to transfer opaque UE Radio Access Capability Information, in the following service operations (and HTTP messages):

-	Resolve Response (GET Response);

-	Assign Request (POST);

HTTP multipart message shall include one JSON body part and one or two binary body parts comprising the OCTET STRING of UE Radio Capability IE (i.e. excluding the IEI, Criticality and octet string length indicator, see also Annex B of 3GPP TS 29.274 [16]) as specified in clause 9.3.1.74 of 3GPP TS 38.413 [15]) and clause 9.2.1.27 of 3GPP TS 36.413 [x].

The JSON body part shall be the "root" body part of the multipart message. It shall be encoded as the first body part of the multipart message. The "Start" parameter does not need to be included.

The multipart message shall include a "type" parameter (see IETF RFC 2387 [10]) specifying the media type of the root body part, i.e. "application/json".

NOTE:	The "root" body part (or "root" object) is the first body part the application processes when receiving a multipart/related message, see IETF RFC 2387 [10]. The default root is the first body within the multipart/related message. The "Start" parameter indicates the root body part, e.g. when this is not the first body part in the message.

For each binary body part in a HTTP multipart message, the binary body part shall include a Content-ID header (see IETF RFC 2045 [12]), and the JSON body part shall include an attribute, defined with the RefToBinaryData type, that contains the value of the Content-ID header field of the referenced binary body part.

****************************************Next Change*********************************************

###### 6.1.3.2.3.1	GET

This operation retrieves the UE Radio Access Capability Information from a dictionary entry stored in the UCMF, by querying with a Manufacturer-assigned or PLMN-assigned UE Radio Capability ID, or the TAC and the SVN as part of the IMEI/IMEISV of the UE.

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ue-radio-capa-id | UeRadioCapaId | M | 1 | PLMN assigned or Manufacturer assigned UE Radio Capability ID used to retrieve a dictionary entry. |  |
| rac-format | RacFormat | C | 0..1 | Coding format in which UE Radio Access Capability Information needs to be provided. See 6.1.6.3.x |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DicEntryData | M | 1 | 200 OK | The response body contains a dictionary entry that is matching the querying parameter. |
| ProblemDetails | O | 1 | 400 Bad Request | The response body contains the error reason of the request message. |
| ProblemDetails | O | 1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - NO_DICTIONARY_ENTRY_FOUND See table 6.1.7.3-1 for the description of this error. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

****************************************Next Change*********************************************

###### 6.1.3.3.3.1	GET

This operation retrieves an individual dictionary entry resource for the mapping information between UE Radio Capability ID(s) and UE Radio Access Capability information, stored in the UCMF, identified by the dicEntryId.

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| rac-format | RacFormat | C | 0..1 | Coding format in which UE Radio Access Capability Information needs to be provided. See 6.1.6.3.x |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DicEntryData | M | 1 | 200 OK | The response body contains a dictionary entry for the given Dictionary Entry ID. |
| ProblemDetails | O | 1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - NO_DICTIONARY_ENTRY_FOUND See table 6.1.7.3-1 for the description of this error. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

****************************************Next Change*********************************************

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nucmf_UECapabilityManagement service based interface protocol.

Table 6.1.6.1-1: Nucmf_UECapabilityManagement specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| DicEntryData | 6.1.6.2.2 |  |  |
| DicEntryCreateData | 6.1.6.2.3 |  |  |
| DicEntryCreatedData | 6.1.6.2.4 |  |  |
| UeRadioCapaId | 6.1.6.2.5 |  |  |
| CreateSubscription | 6.1.6.2.6 |  |  |
| CreatedSubscription | 6.1.6.2.7 |  |  |
| UcmfNotification | 6.1.6.2.8 |  |  |
| ManAssOpRequestlist | 6.1.6.2.9 |  |  |
| RacFormat | 6.1.6.3.x |  |  |

Table 6.1.6.1-2 specifies data types re-used by the Nucmf_UECapabilityManagement service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the N<NF> service based interface.

Table 6.1.6.1-2: Nucmf_UECapabilityManagement re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| PlmnAssiUeRadioCapId | 3GPP TS 29.571 [14] |  |  |
| ManAssiUeRadioCapId | 3GPP TS 29.571 [14] |  |  |
| TypeAllocationCode | 3GPP TS 29.571 [14] |  |  |
| NfInstanceId | 3GPP TS 29.571 [14] |  |  |
| DateTime | 3GPP TS 29.571 [14] |  |  |

****************************************Next Change*********************************************

##### 6.1.6.2.2	Type: DicEntryData

Table 6.1.6.2.2-1: Definition of type DicEntryData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmnAssiUeRadioCapId | PlmnAssiUeRadioCapId | C | 0..1 | This IE shall include a PLMN Assigned UE Radio Capability ID if allocated in the dictionary entry. |
| manAssiUeRadioCapId | ManAssiUeRadioCapId | C | 0..1 | This IE shall include a Manufacturer Assigned UE Radio Capability ID if available in the dictionary entry. |
| typeAllocationCode | TypeAllocationCode | C | 0..1 | This IE shall contain the Type Allocation Code in corresponding to the UE Radio Access Capability in the dictionary entry. |
| ueRadioCapability5GS | RefToBinaryData | C | 0..1 | This IE shall be included to contain the UE Radio Access Capability Information encoded as OCTET STRING of UE Radio Capability IE as specified in clause 9.3.1.74 of 3GPP TS 38.413 [15], when the message is sent to AMF, otherwise it may be included. |
| ueRadioCapabilityEPS | RefToBinaryData | C | 0..1 | This IE shall be included to contain the UE Radio Access Capability Information encoded as OCTET STRING of UE Radio Capability IE as specified in clause 9.2.1.27 of 3GPP TS 36.413 [x], when the message is sent to MME, otherwise it may be included. |
| NOTE: The information in the dictionary entry which is included as the query parameter(s) in the request message shall not be present. |  |  |  |  |

****************************************Next Change*********************************************

##### 6.1.6.2.3	Type: DicEntryCreateData

Table 6.1.6.2.3-1: Definition of type DicEntryCreateData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| typeAllocationCode | TypeAllocationCode | M | 1 | This IE shall contain the Type Allocation Code in corresponding to the UE Radio Access Capability in the dictionary entry. |
| ueRadioCapability5GS | RefToBinaryData | C | 0..1 | This IE shall be included to contain the UE Radio Access Capability Information encoded as OCTET STRING of UE Radio Capability IE specified in clause 9.3.1.74 of 3GPP TS 38.413 [15]. (NOTE) |
| ueRadioCapabilityEPS | RefToBinaryData | C | 0..1 | This IE shall be included to contain the UE Radio Access Capability Information encoded as OCTET STRING of UE Radio Capability IE specified in clause 9.2.1.27 of 3GPP TS 36.413 [x]. (NOTE) |
| NOTE: At least one of ueRadioCapability5GS or ueRadioCapabilityEPS shall be present. |  |  |  |  |

****************************************Next Change*********************************************

##### 6.1.6.2.4	Type: DicEntryCreatedData

Table 6.1.6.2.4-1: Definition of type DicEntryCreatedData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmnAssiUeRadioCapId | PlmnAssiUeRadioCapId | M | 1 | This IE shall include a PLMN Assigned UE Radio Capability ID if allocated. |

****************************************Next Change*********************************************

##### 6.1.6.3.x	Enumeration: RacFormat

The enumeration RacFormat represents the encoding type of the UE’s Radio Access Capability Information.

Table 6.1.6.3.x-1: Enumeration racFormat

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "5GS" | This value indicates UE Radio Access Capability Information shall be encoded as OCTET STRING of UE Radio Capability IE specified in clause 9.3.1.74 of 3GPP TS 38.413 [15]. |  |
| "EPS" | This value indicates UE Radio Access Capability Information shall be encoded as OCTET STRING of UE Radio Capability IE specified in clause 9.2.1.27 of 3GPP TS 36.413 [x]. |  |

****************************************Next Change*********************************************

##### 6.1.6.5.2	UE Radio Capability Information

UeRadioCapability5GS shall be encoded with the OCTET STRING of UE Radio Capability IE (i.e. excluding the IEI, Criticality and octet string length indicator, see also Annex B of 3GPP TS 29.274 [16]) as specified in clause 9.3.1.74 of 3GPP TS 38.413 [15], using the vnd.3gpp.ngap content-type.

UeRadioCapabilityEPS shall be encoded with the OCTET STRING of UE Radio Capability IE (i.e. excluding the IEI, Criticality and octet string length indicator, see also Annex B of 3GPP TS 29.274 [16]) as specified in clause 9.2.1.27 of 3GPP TS 36.413 [x], using the vnd.3gpp.s1ap content-type.

****************************************Next Change*********************************************

## A.2	Nucmf_UECapabilityManagement API

******Skipped For Clarity******

paths:

/dic-entries:

get:

summary: retrieve a dictionary entry matching query parameters

operationId: RetrieveDictionaryEntry

tags:

- Dictionary Entry (Store)

parameters:

- name: ue-radio-capa-id

in: query

required: true

description: UE Radio Capability ID, either PLMN Assigned or Manufacturer Assigned

schema:

$ref: '#/components/schemas/UeRadioCapaId'

- name: rac-format

in: query

description: Encoding format of RAC Info

schema:

$ref: '#/components/schemas/RacFormat'

responses:

'200':

description: Expected response to a valid request

content:

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/DicEntryData'

binaryDataUeRadioCapability5GS:

type: string

format: binary

binaryDataUeRadioCapabilityEPS:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataUeRadioCapability5GS:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataUeRadioCapabilityEPS:

contentType:  application/vnd.3gpp.s1ap

headers:

Content-Id:

schema:

type: string

******Skipped For Clarity******

post:

summary: Create a dictionary entry in the UCMF

tags:

- a dictionary entry (Document)

operationId: CreateDictionaryEntry

requestBody:

content:

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/DicEntryCreateData'

binaryDataUeRadioCapability5GS:

type: string

format: binary

binaryDataUeRadioCapabilityEPS:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataUeRadioCapability5GS:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

binaryDataUeRadioCapabilityEPS:

contentType:  application/vnd.3gpp.s1ap

headers:

Content-Id:

schema:

type: string

required: true

responses:

'201':

description: Dictionary Created

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nucmf-uecm/<apiVersion>/dic-entries/{dicEntryId}'

required: true

schema:

type: string

content:

application/json:

schema:

$ref: '#/components/schemas/DicEntryCreatedData'

******Skipped For Clarity******

/dic-entries/{dicEntryId}:

get:

summary: Get an individual dictionary entry via dicEntryId

operationId: GetDicEntry

tags:

- dic Entry Id (Document)

parameters:

- name: dicEntryId

in: path

description: the ID of a dictionary entry in the UCMF

required: true

schema:

$ref: '#/components/schemas/DicEntryId'

- name: rac-format

in: query

description: Encoding format of of RAC Info

schema:

$ref: '#/components/schemas/RacFormat'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/DicEntryData'

******Skipped For Clarity******

schemas:

DicEntryData:

type: object

properties:

typeAllocationCode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TypeAllocationCode'

plmnAssiUeRadioCapId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnAssiUeRadioCapId'

manAssiUeRadioCapId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ManAssiUeRadioCapId'

ueRadioCapability5GS:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

ueRadioCapabilityEPS:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

DicEntryCreateData:

type: object

required:

- typeAllocationCode

properties:

typeAllocationCode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TypeAllocationCode'

ueRadioCapability5GS:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

ueRadioCapabilityEPS:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

DicEntryCreatedData:

type: object

required:

- plmnAssiUeRadioCapId

properties:

plmnAssiUeRadioCapId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnAssiUeRadioCapId'

******Skipped For Clarity******

#

# SIMPLE DATA TYPES

#

DicEntryId:

type: integer

minimum: 0

maximum: 4294967295

#

# ENUMERATIONS

#

EventType:

anyOf:

- type: string

enum:

- CREATION_OF_DICTIONARY_ENTRY

- DELETION_OF_PLMN_ASSIGNED_IDS

- type: string

RacFormat:

anyOf:

- type: string

enum:

- 5GS

- EPS

- type: string

****************************************End of  Changes*******************************************