3GPP TSG-CT Meeting #93-e	CP-212084

# E-Meeting, 13th – 15th September 2021									(Revision of C3-214184)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.551 | CR | 0084 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to custom operation partialpull |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | ZTE |  |  |  |  |  |  |  |  |  |
| Work item code: | pfdManEnh |  |  |  |  |  | Date: |  |  | 2021-8-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature) C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per the text below from TS 29.501, the URI of a custom operation is constructed by adding a verb as name for the custom operation at the end of the service URI. 4.4.2 Custom operations URI structure The URI of a custom operation which is not associated with a resource shall have the following structure: {apiRoot}/<apiName>/<apiVersion>/<custOpName> "custOpName" represents the name of the custom operation as defined in clause 5.1.3.2. 5.1.3.2 URI Path Segment Naming Conventions d) For custom operations, the last path segment of the URI via which the operation is invoked shall be a verb, or shall start with a verb. Example 5: …/app_instances/{appInstanceId}/instantiate Example 6: …/sessions/terminate-all However, the definition of the custom operation URI in this specification is not aligned with the principles defined in TS 29.501: - “/applications/partial-pull” is defined as the custom operation URI, but “partial-pull” is not the name of the custom operation. Furthermore, custom operation “Partialpull” is defined in 5.3.2.4.2, but “partialpull” is used as custom operation in Table 5.3.1-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change operation name “Partialpull” to “partialpull” in 5.3.2.4.2, and correct the custom operation URI to “/applications/partialpull”. Other editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Definition of the custom operation URI is not aligned with the principles defined in TS 29.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.3, 5.3.1, 5.3.2.4.1, 5.3.2.4.2, 5.3.2.4.2.2, 5.6.1, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible corrections to the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of C3-214184: - “partial-pull” is replaced with “partialpull“ in the custom opertation URI in subclauses 4.2.2.3 and 5.3.2.4.1; - custom opertation name “Partialpull” is changed to “partialpull“ in subclause 5.3.2.4.1; |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

…

# Proposed changes:

# *** 1st Change ***

4.2.2.3	Retrieval of PFDs by the partial pull

This procedure, as shown in Figure 4.2.2.3-1, is used to retrieve PFDs for an application identifier from the PFDF by the partial pull for requested application identifier(s) if the "PartialPull" feature defined in subclause 5.8 is supported both by NF service consumer and PFDF.

Figure 4.2.2.3-1: Retrieval of PFDs by the partial pull

1.	The NF service consumer (e.g. SMF) shall send an HTTP POST message to the resource "{apiRoot}/nnef-pfdmanagement/v1/applications/partialpull". The NF service consumer shall include one or more ApplicationForPfdRequest data structure in the payload body of the HTTP POST. With the ApplicationForPfdRequest data structure, the NF service consumer shall include the application identifier within the "applicationId" attribute and the timestamp of the PFDs received in the last provisioning for the application identifier within the "pfdTimestamp" attribute. The NF service consumer may also request full set of PFD(s) of an application identifier without including the timestamp if it is not available.

2.	If the PFDF accepted the HTTP POST request, the PFDF shall send to the NF service consumer:

-	the HTTP "200 OK" response (as shown in figure 4.2.2.3-1, step 2a) including one or more PfdDataForApp data structure if the NF service consumer requests the PFD(s) for an application identifier(s) without the timestamp or if the NF service consumer requests PFD(s) for an application identifier(s) with timestamp and the PFDF determines that corresponding PFD(s) is changed since the last request based on the timestamp received in the request for the application identifier. Within the PfdDataForApp data type, the PFDF shall include the application identifier within the "applicationId" attribute, the new timestamp within the "pfdTimestamp" attirbute, the "partialFlag" attribute if applicable and create/update/remove the PFDs as follows:

-	include the full list of the PFD(s) within the "pfds" attribute for the application identifier which is requested without the timestamp;

-	include the full list of the PFD(s) for the application identifier if all the PFD(s) are changed for the application identifier(s) since the last request based on the timestamp;

-	for the application identifier whose the PFD(s) are partially changed:

-	include the new PFD(s) with new PFD identifier(s) within the "pfds" attribute if the new PFD(s) is added for the application identifier and the "partialFlag" attribute set to true;

-	include the new PFD(s) with existing PFD identifier(s) within the "pfds" attribute if the existing PFD(s) is updated for the application identifier and the "partialFlag" attribute set to true;

-	include the existing PFD identifier(s) without any content within the "pfds" attribute if the existing PFD(s) is removed for the application identifier and the "partialFlag" attribute set to true;

-	not include the "pfds" attribute if the PFD(s) is removed for the application identifier.

NOTE 1:	The PFDF does not include the PfdDataForApp data type for the application identifier whose PFD(s) is not updated since last request.

NOTE 2	If the PFDF determines that the PFDs are changed since the last request but cannot determine what changes have been applied to the individual PFD(s) for an application identifier, the PFDF can include the full list of the PFD(s) in the PfdDataForApp data type.

-	the HTTP "204 No Content" response (as shown in figure 4.2.2.3-1, step 2b) if the PFD(s) for all the requested application identifier(s) are not changed since last request.

When the NF service consumer receives the response with "200 OK" status code, the NF service consumer shall

-	remove the all existing PFD(s) (if available) and install all the new provided PDF(s) for an application identifier if full list of PFD(s) is received but "partialFlag" attribute is not received;

-	delete the existing PFD(s) for an application identifier(s) where no PFD(s) is received;

-	for an application identifier(s) where the PFD(s) is provided and "partialFlag" attribute is also provided and set to true:

-		install a new PFD(s) if the new PFD(s) with a new PFD identifier(s) is received;

-	update an existing PFD(s) if a new PFD(s) with the same PFD identifier(s) is received;

-	delete an existing PFD(s) if the same PFD identifier(s) without any content is received;

-	reserve an existing PFD(s) if the PFD identifier(s) is not received.

# *** 2nd Change ***

5.3.1	Resource Structure

Figure 5.3.1-1: Resource URI structure of the Nnef_PFDmanagement API

Table 5.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| PFD of applications | /applications | GET | Nnef_PFDmanagement_Fetch. Retrieve PFDs for one or multiple applications using query parameters. |
|  | /applications/partialpull | partialpull (POST) | Request PFDs for one or multiple application identifier(s) by the partial update. |
| Individual application PFD | /applications/{appId} | GET | Nnef_PFDmanagement_Fetch. Retrieve the PFD for an application. |
| PFD subscriptions | /subscriptions | POST | Nnef_PFDmanagement_Subscribe. Subscribe the notification of PFD changes. |
| Individual PFD subscription | /subscriptions/{subscriptionId} | PUT | Nnef_PFDmanagement_Subscribe. Update a subscription to PFD change notifications. |
| Individual PFD subscription | /subscriptions/{subscriptionId} | DELETE | Nnef_PFDmanagement_Unsubscribe. Delete a subscription to PFD change notifications. |

# *** 3rd Change ***

5.3.2.4.1	Overview

Table 5.3.2.4.1-1: Custom operations

| Operation Name | Custom operation URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| partialpull | /applications/partialpull | partialpull (POST) | Request PFDs for one or multiple application identifier(s) by the partial update. |

# *** 4th Change ***

5.3.2.4.2	Operation: partialpull

# *** 5th Change ***

5.3.2.4.2.2	Operation Definition

This custom operation requests PFDs for one ore multiple application identifier(s) by the partial update..

This operation shall support the request data structures specified in table 5.3.2.4.2.2-1 and the response data structure and response codes specified in table 5.3.2.4.2.2-2.

Table 5.3.2.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(ApplicationForPfdRequest) | M | 1 | Contains the application Identifier(s) for PFDs request. |

Table 5.3.2.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(PfdDataForApp) | M | 0..N | 200 OK | The PFDs for one or more application identifier(s) are returned. |
| n/a |  |  | 204 No Content |  |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. |  |  |  |  |

# *** 6th Change ***

5.6.1	General

This subclause specifies the application data model supported by the API.

Table 5.6.1-1 specifies the data types defined for the Nnef_PFDmanagement service based interface protocol.

Table 5.6.1-1: Nnef_PFDmanagement specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ApplicationForPfdRequest | 5.6.2.8 | Contains the application identifier(s) for the PFDs request. | PartialPull |
| NotificationPush | 5.6.2.7 | Provides the information for the NF service consumer to retrieve the PFDs and/or remove the PFDs of the applicable application identifier(s). | NotificationPush |
| PfdChangeNotification | 5.6.2.4 | Represents PFD change information. |  |
| PfdChangeReport | 5.6.2.6 | Represents error of PFD change. |  |
| PfdContent | 5.6.2.5 | Represents the content of a PFD for an application identifier. |  |
| PfdDataForApp | 5.6.2.2 | Represents the PFDs for an application identifier. |  |
| PfdOperation | 5.6.3.3 | Indicates the operation of the PFDs. | NotificationPush |
| PfdSubscription | 5.6.2.3 | Represents a PFD subscription. |  |

Table 5.6.1-2 specifies data types re-used by the Nnef_PFDmanagement service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnef_PFDmanagement service based interface.

Table 5.6.1-2: Nnef_PFDmanagement re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ApplicationId | 3GPP TS 29.571 [10] |  |  |
| DateTime | 3GPP TS 29.571 [10] |  |  |
| DomainNameProtocol | 3GPP TS 29.122 [11] | Indicates the additional protocol and protocol field for domain names to be matched. | DomainNameProtocol |
| DurationSec | 3GPP TS 29.571 [10] | Identifies a period of time in units of seconds. | NotificationPush |
| RedirectResponse | 3GPP TS 29.571 [10] | Contains redirection related information. | ES3XX |
| SupportedFeatures | 3GPP TS 29.571 [10] |  |  |
| Uri | 3GPP TS 29.571 [10] |  |  |

# *** 7th Change ***

A.2	Nnef_PFDmanagement API

openapi: 3.0.0

info:

title: Nnef_PFDmanagement Service API

version: 1.2.0-alpha.4

description: |

Packet Flow Description Management Service.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

#

externalDocs:

description: 3GPP TS 29.551 v17.3.0, 5G System; Packet Flow Description Management Service

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.551/'

#

servers:

- url: '{apiRoot}/nnef-pfdmanagement/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nnef-pfdmanagement

paths:

/applications:

get:

summary: Retrieve PFDs for all applications or for one or multiple applications with query parameter.

tags:

- PFD of applications

operationId: Nnef_PFDmanagement_AllFetch

parameters:

- name: application-ids

description: The required application identifier(s) for the returned PFDs.

in: query

required: true

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

- name: supported-features

in: query

description: To filter irrelevant responses related to unsupported features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: The PFDs for one or more application identifier(s) in the request URI are returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PfdDataForApp'

minItems: 0

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/applications/partialpull:

post:

summary: retrieve the PFD(s) by partial update

operationId: Nnef_PFDmanagement_AppFetchPartialUpdate

tags:

- PFD of applications by partial update

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/ApplicationForPfdRequest'

minItems: 1

responses:

'200':

description: OK. Changed PFD(s) is returned

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PfdDataForApp'

minItems: 1

'204':

description: The PFD(s) is not changed

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/applications/{appId}:

get:

summary: Retrieve the PFD for an application.

tags:

- Individual application PFD

operationId: Nnef_PFDmanagement_IndAppFetch

parameters:

- name: appId

description: The required application identifier(s) for the returned PFDs.

in: path

required: true

schema:

type: string

- name: supported-features

in: query

description: To filter irrelevant responses related to unsupported features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: A representation of PFDs for an application in the request URI is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PfdDataForApp'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions:

post:

summary: Subscribe the notification of PFD changes.

tags:

- PFD subscriptions

operationId: Nnef_PFDmanagement_CreateSubscr

requestBody:

description: a PfdSubscription resource to be created.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PfdSubscription'

callbacks:

PfdChangeNotification:

'{request.body#/notifyUri}':

post:

summary: Notification of PFD change.

tags:

- PfdChangeNotification data

operationId: Nnef_PFDmanagement_Notify

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PfdChangeNotification'

minItems: 1

responses:

'200':

description: The PFD operation in the notification is performed and the PfdChangeReport indicates failure reason.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/PfdChangeReport'

minItems: 1

'204':

description: The PFD operation in the notification is performed successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

NotificationPush:

'{request.body#/notifyUri}/notifypush':

post:

summary: Notification Push.

tags:

- NotificationPush data

operationId: Nnef_PFDmanagement_PushNotify

requestBody:

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/NotificationPush'

minItems: 1

responses:

'204':

description: Notificaiton PUSH is accepted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

responses:

'201':

description: The creation of a PfdSubscription resource is confirmed and a representation of that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PfdSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nnef-pfdmanagement/v1/subscriptions/{subscriptionId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

put:

summary: Updates/replaces an existing subscription resource

tags:

- Individual PFD subscription

operationId: Nnef_PFDmanagement_ModifySubscr

parameters:

- name: subscriptionId

description: Identify the subscription.

in: path

required: true

schema:

type: string

requestBody:

description: Parameters to update/replace the existing subscription

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PfdSubscription'

responses:

'200':

description: OK (Successful update of the subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/PfdSubscription'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete a subscription of PFD change notification.

tags:

- Individual PFD subscription

operationId: Nnef_PFDmanagement_Unsubscribe

parameters:

- name: subscriptionId

description: Identify the subscription.

in: path

required: true

schema:

type: string

responses:

'204':

description: The PfdSubscription resource matching the subscriptionId was deleted successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nnef-pfdmanagement: Access to the Nnef_PFDmanagement API

schemas:

#

# STRUCTURED DATA TYPES

#

PfdContent:

description: Represents the content of a PFD for an application identifier.

type: object

properties:

pfdId:

type: string

description: Identifies a PDF of an application identifier.

flowDescriptions:

type: array

items:

type: string

minItems: 1

description: Represents a 3-tuple with protocol, server ip and server port for UL/DL application traffic.

urls:

type: array

items:

type: string

minItems: 1

description: Indicates a URL or a regular expression which is used to match the significant parts of the URL.

domainNames:

type: array

items:

type: string

minItems: 1

description: Indicates an FQDN or a regular expression as a domain name matching criteria.

dnProtocol:

$ref: 'TS29122_PfdManagement.yaml#/components/schemas/DomainNameProtocol'

PfdDataForApp:

description: Represents the PFDs for an application identifier.

type: object

properties:

applicationId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

pfds:

type: array

items:

$ref: '#/components/schemas/PfdContent'

minItems: 1

cachingTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

pfdTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

partialFlag:

type: boolean

required:

- applicationId

PfdSubscription:

description: Represents a PFD subscription.

type: object

properties:

applicationIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

notifyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notifyUri

- supportedFeatures

PfdChangeNotification:

description: Represents information related to a notification of PFD change.

type: object

properties:

applicationId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

removalFlag:

type: boolean

default: false

partialFlag:

type: boolean

default: false

pfds:

type: array

items:

$ref: '#/components/schemas/PfdContent'

minItems: 1

required:

- applicationId

NotificationPush:

description: Represents the information to be used by the NF service consumer to retrieve the PFDs and/or remove the PFDs of the applicable application identifier(s).

type: object

properties:

appIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

allowedDelay:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

pfdOp:

$ref: '#/components/schemas/PfdOperation'

required:

- appIds

PfdChangeReport:

description: Represents an error report on PFD change.

type: object

properties:

pfdError:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

applicationId:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

required:

- pfdError

- applicationId

PfdOperation:

description: Indicates the operation to be applied on PFD(s).

anyOf:

- type: string

enum:

- RETRIEVE

- FULLPULL

- PARTIALPULL

- REMOVE

- type: string

ApplicationForPfdRequest:

description: Contains the application identifier(s) for the PFD(s) request.

type: object

properties:

applicationId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

pfdTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- applicationId

*** End of Changes ***