3GPP TSG-CT Meeting #96	CP-221236

# Budapest, Hungary, 05th – 06th June 2022										Revision of C4-223110

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.532 | CR | 0009 | rev | 3 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS information in Context Status Event Report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-03-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | QoS information in Context Status Event Report is not specified yet. The above information shall describe the MBS QoS flows of the MBS session added, modified and/or removed by the MB-SMF. Stage 2 requirements for MBS QoS flows are specified in clause 6.6 of TS 23.247. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Context Status Event Report is extended with new properties enabling the MB-SMF to signal MBS QoS flows to add, modify or release. These properties are defined like QoS flows signaled from the anchor SMF to an I-SMF/V-SMF in the SMF PDUSession service, but for the following properties which are not applicable to MBS QoS flows: - QoS properties for UL traffic - Reflective QoS - Alternative QoS profile - QoS notification control - QoS monitoring |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification, QoS support is not defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.9.2, 5.3.2.11, 6.2.6.1, 6.2.6.2.17, 6.2.6.2.w , 6.2.6.2.x, 6.2.6.2.y, 6.2.6.2.z, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces new backward compatible features to the MBSSession API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1: - Huawei is added as co-source (merge with CR 29.232 #0003) - Data type GbrQosFlowInformation re-used from the related 29.571 CR has been renamed GbrFlowInformation. Rev.2: - the GbrQosFlowInformation data type is added to this CR (it was initially defined in the 29.571 CR, but it is no longer now). Re.3: Corrects an OpenAPI error found during the implementation of Rev. 2 agreed at CT4#110-e: in the QosFlowProfile data type, the reference to 29.571 shall be removed in the gbrQosFlowInfo attribute as follows: gbrQosFlowInfo: $ref: 'TS29571_CommonData.yaml#/components/schemas/GbrQosFlowInformation' |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 5.3.2.9	ContextStatusSubscribe

##### 5.3.2.9.1	General

The ContextStatusSubscribe service operation enables to create and modify a subscription to notifications of events about a multicast MBS session context.

NOTE:	For a location dependent MBS service, one ContextStatusSubscribe service operation is performed per MBS session.

##### 5.3.2.9.2	Creation of a subscription

The ContextStatusSubscribe service operation shall be used to request information (e.g. QoS information) about a multicast MBS session and subscribe to notifications of events about the multicast MBS session context.

It is used in the following procedures:

-	Multicast session join and session establishment procedure (see clause 7.2.1.3 of 3GPP TS 23.247 [14]).

The NF Service Consumer may subscribe to multiple events in a subscription. A subscription shall be specific to a multicast MBS session context.

The NF Service Consumer (e.g. SMF) shall request information (e.g. QoS information) about a multicast MBS session and create a subscription to notification of events about the multicast MBS session context by using the HTTP POST method with the URI of the Subscriptions collection for MBS contexts resource as shown in Figure 5.3.2.9.2-1.

Figure 5.3.2.9.2-1: Creation of a subscription for a multicast MBS session context

1.	The NF Service Consumer shall send a POST request. The payload body of the POST request (ContextStatusSubscribeReqData structure) shall contain the description of the subscription requested to be created:

-	NF Instance ID of the NF Service Consumer creating the subscription;

-	MBS Session ID (i.e. TMGI or source specific multicast address) being the target of the subscription;

-	Event ID(s) of the events to which the NF service consumer requests to subscribe;

-	Notification URI, indicating the address where to send the events notifications generated by the subscription;

The payload body of the POST request may further contain the following parameters:

-	Notification Correlation ID, indicating the correlation identitity to be carried in event notifications generated by the subscription.

-	For each subscribed event:

-	Immediate Report Indication, to request to receive an immediate report in the response with the current event status;

-	Reporting Mode, to indicate how event shall be reported (One-time Reporting or Continuous);

-	Expiry time, indicating the time up to which the subscription is desired to be kept active and after which the subscribed events shall stop generating notifications.

In this release of the specification, the SMF shall subscribe to the "QOS_INFO", "STATUS_INFO", "SERVICE_AREA_INFO" and "SESSION_RELEASE" events, with the Reporting Mode set to "Continuous event reporting".

2a.	On success, the MB-SMF shall return a "201 Created" response, with an HTTP Location header providing the URI of the newly created resource. 

The payload body of the POST response (ContextStatusSubscribeRspData structure) shall include a representation of the created subscription. If the NF Service Consumer has included more than one event in the subscription creation request and some of the events cannot be subscribed, the MB-SMF shall accept the request and indicate the successfully subscribed event(s) in the response.

If the NF Service Consumer has requested an Immediate Report, the MB-SMF shall include the current status of the events subscribed in the response, if available:

-	list of MBS QoS flows to set up (in the qosFlowsAddModRequestList IE)for the multicast MBS session;

-	multicast MBS session's status (activated/deactivated);

-	multicast MBS session service area for local multicast service.

If the NF Service Consumer has requested One-time Reporting and if the MB-SMF has included the current status of the events subscribed in the response, then the MB-SMF shall not do any subsequent event notification for the subscribed events.

The payload body of the POST response shall also contain the following parameters:

-	the MBS Service Areas and their respective Area Session IDs, for a location dependent MBS session;

-	the MBS Service Area, for a local MBS session.

The payload body of the POST response may also contain the following parameters:

-	start time of the multicast MBS session;

-	the GTP-U Common TEID (C-TEID, see 3GPP TS 29.281 [17]) and the related IP multicast source address of the MB-UPF, for data reception over N19mb using multicast transport, if IP multicast transport may apply over N19mb;

-	MBS session authorization information (i.e. indication that the multicast MBS session allows any UE to join);

-	Expiry time after which the subscription becomes invalid, determined based on operator policies and taking into account the expiry time included in the request if any. If an expiry time was included in the request, then the expiry time returned in the response should be less than or equal to that value. The NF Service Consumer may update the subscription before the Expiry time to extend the subscription lifetime. Once the subscription expires, if the NF Service Consumer wants to keep receiving notifications, it shall create a new subscription in the MB-SMF. If the expiry time is not included in the response, the NF Service Consumer shall consider the subscription to be valid without an expiry time.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.2.3.6.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.2.3.6.3.1-3.

* * * Next Change * * * *

#### 5.3.2.11	ContextStatusNotify

##### 5.3.2.11.1	General

The ContextStatusNotify service operation shall be invoked by the MB-SMF to send a notification about event(s), when events about the multicast MBS session context included in the subscription occur.

It is used in the following procedures:

-	MBS session activation procedure (see clause 7.2.5.2 of 3GPP TS 23.247 [14]);

-	MBS session deactivation procedure (see clause 7.2.5.3 of 3GPP TS 23.247 [14]); and

-	Multicast session update procedure (see clause 7.2.6 of 3GPP TS 23.247 [14]).

The MB-SMF shall notify event(s) about the multicast MBS session context by using the HTTP POST method targeting the notification URI received in the subscription as shown in Figure 5.3.2.11.1-1.

Figure 5.3.2.11.1-1: Notification of a multicast MBS session context event

1.	The MB-SMF shall send a POST request targeting the notification URI. The notification, i.e. the payload body of the POST request (ContextStatusNotifyReqData structure) shall contain the following information:

-	Notification Correlation ID, if available in the subscription;

-	List of event(s) which occurred;

-	When reporting a QOS_INFO event:

-	list of MBS QoS flows to add, modify and/or release (in the qosFlowsAddModRequestList IE and/or qosFlowsRelRequestList) for the multicast MBS session.

2a.	On success, a "204 No Content" response shall be returned.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.2.5.3.3.1-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.2.5.3.3.1-2.

* * * Next Change * * * *

#### 6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the NMB-SMF service based interface protocol.

Table 6.2.6.1-1: NMB-SMF specific Data Types

| Data type | Clause defined |  | Description | Applicability |  |
| --- | --- | --- | --- | --- | --- |
| CreateReqData | 6.2.6.2.2 |  | Data within the Create Request |  |  |
| CreateRspData | 6.2.6.2.3 |  | Data within the Create Response |  |  |
| MbsSessionExtension | 6.2.6.2.4 |  | MB-SMF API specific MbsSession data type extensions |  |  |
| ExtMbsSession | 6.2.6.4.5 |  | MbsSession common data type extended with MB-SMF API specific extensions |  |  |
| ContextUpdateReqData | 6.2.6.2.5 |  | Data within the ContexUpdate Request |  |  |
| ContextUpdateRspData | 6.2.6.2.6 |  | Data within the ContextUpdate Response |  |  |
| StatusSubscribeReqData | 6.2.6.2.7 |  | Data within the Create Subscription Request for the collection of MBS Session subscriptions (StatusSubscribe service operation). |  |  |
| StatusSubscribeRspData | 6.2.6.2.8 |  | Data within the Create Subscription Response (StatusSubscribe service operation). |  |  |
| N2MbsSmInfo | 6.2.6.2.9 |  | N2 MBS Session Management Information |  |  |
| ContextStatusNotifyReqData | 6.2.6.2.10 |  | Data within ContextStatusNotify Request |  |  |
| StatusNotifyReqData | 6.2.6.2.11 |  | Data within StatusNotify Request |  |  |
| ContextStatusSubscribeReqData | 6.2.6.2.12 |  | Data within ContextStatusSubscribe Request |  |  |
| ContextStatusSubscription | 6.2.6.2.13 |  | Context Status Subscription |  |  |
| ContextStatusEvent | 6.2.6.2.14 |  | Context Status Event |  |  |
| ContextStatusSubscribeRspData | 6.2.6.2.15 |  | Data within ContextStatusSubscribe Response |  |  |
| MbsContextInfo | 6.2.6.2.16 |  | MBS Context Information |  |  |
| ContextStatusEventReport | 6.2.6.2.17 |  | Context Status Event Report |  |  |
| QosInfo | 6.2.6.2.w | QoS Information |  |
| QosFlowAddModifyRequestItem | 6.2.6.2.x |  | Individual MBS QoS flow requested to be created or modified |  |  |
| QosFlowProfile | 6.2.6.2.y |  | MBS QoS flow profile |  |  |
| GbrQosFlowInformation | 6.2.6.2.z | GBR MBS QoS flow information |  |
| ContextUpdateAction | 6.2.6.3.3 |  | The requested action for the MBS session context |  |  |
| ContextStatusEventType | 6.2.6.3.4 |  | Context Status Event Type |  |  |
| ReportingMode | 6.2.6.3.5 |  | Reporting Mode |  |  |
| NgapIeType | 6.2.6.3.6 |  | NGAP Information Element Type |  |  |

Table 6.2.6.1-2 specifies data types re-used by the NMB-SMF service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the NMB-SMF service based interface.

Table 6.2.6.1-2: NMB-SMF re-used Data Types

| Data type | Reference | Comments | Applicability |  |
| --- | --- | --- | --- | --- |
| MbsSession | 3GPP TS 29.571 [18] | MBS session |  |  |
| Tmgi | 3GPP TS 29.571 [18] | TMGI |  |  |
| TunnelAddress | 3GPP TS 29.571 [18] | Tunnel Address (UDP/IP) |  |  |
| MbsSessionId | 3GPP TS 29.571 [18] | MBS Session Identifier |  |  |
| AreaSessionId | 3GPP TS 29.571 [18] | Area Session Identifier used for MBS session with location dependent content |  |  |
| Ssm | 3GPP TS 29.571 [18] | Source specific IP multicast address |  |  |
| Uint32 | 3GPP TS 29.571 [18] | Unsigned 32-bit integer |  |  |
| NfInstanceId | 3GPP TS 29.571 [18] | NF Instance Identifier |  |  |
| Bytes | 3GPP TS 29.571 [18] | Base64-encoded characters |  |  |
| RefToBinaryData | 3GPP TS 29.571 [18] | Cross-Reference to binary data encoded within a binary body part in an HTTP multipart message |  |  |
| GlobalRanNodeId | 3GPP TS 29.571 [18] | Global RAN Node Identity |  |  |
| DateTime | 3GPP TS 29.571 [18] | Date and time |  |  |
| Uri | 3GPP TS 29.571 [18] | URI |  |  |
| DateTime | 3GPP TS 29.571 [18] | Date and Time |  |  |
| MbsSessionActivityStatus | 3GPP TS 29.571 [18] | MBS Session Activity Status |  |  |
| MbsServiceArea | 3GPP TS 29.571 [18] | MBS Service Area |  |  |
| MbsAreaSession | 3GPP TS 29.510 [10] | MBS Session in a specific MBS Service Area |  |  |
| MbsSessionSubscription | 3GPP TS 29.571 [18] | MBS Session Subscription |  |  |
| MbsSessionEventReportList | 3GPP TS 29.571 [18] | MBS Session Event Report List |  |  |
| Qfi | 3GPP TS 29.571 [13] | QoS Flow Identifier |  |  |
| Arp | 3GPP TS 29.571 [13] | Allocation and Retention Priority |  |  |
| Dynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics for a 5QI that is neither standardized nor pre-configured. |  |  |
| NonDynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics that replace the default QoS characteristics for a standardized or pre-configured 5QI. |  |  |
| 5Qi | 3GPP TS 29.571 [13] | 5G QoS Identifier |  |
| BitRate | 3GPP TS 29.571 [13] | Bit rate |  |
| PacketLossRate | 3GPP TS 29.571 [13] | Packet loss rate |  |

* * * Next Change * * * *

##### 6.2.6.2.17	Type: ContextStatusEventReport

Table 6.2.6.2.17-1: Definition of type ContextStatusEventReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventType | ContextStatusEventType | M | 1 | MBS session context status event type |  |
| timeStamp | DateTime | M | 1 | This IE shall contain the time at which the event is generated. |  |
| qosInfo | QosInfo | C | 0..1 | This IE shall be present if the eventType IE indicates "QOS_INFO". |  |
| statusInfo | MbsSessionActivityStatus | C | 0..1 | This IE shall be present if the eventType IE indicates "STATUS_INFO". |  |
| mbsServiceArea | MbsServiceArea | C | 0..1 | This IE shall be present for a Local MBS session if the eventType IE indicates "SERVICE_AREA_INFO". The receiver of this IE shall overwrite any previously stored value with the value received in this IE. |  |
| mbsAreaSessions | map(MbsAreaSession) | C | 1..N | This IE shall be present for a location dependent MBS service if the eventType IE indicates "SERVICE_AREA_INFO". When present, one map entry shall be provided for each MBS Service Area served by the MBS session. The key of the map shall be the areaSessionId. The receiver of this IE shall overwrite any previously stored value with the value received in this IE. |  |

* * * Next Change * * * *

##### 6.2.6.2.w	Type: QosInfo

Table 6.2.6.2.w-1: Definition of type QosInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| qosFlowsAddModRequestList | array(QosFlowAddModifyRequestItem) | C | 1..N | This IE shall be present if the eventType IE indicates "QOS_INFO" and MBS QoS flows are requested to be established or modified. |  |
| qosFlowsRelRequestList | array(Qfi) | C | 1..N | This IE shall be present if the eventType IE indicates "QOS_INFO" and MBS QoS flows are requested to be released. When present, it shall indicate the MBS QoS flows to be released. |  |

* * * Next Change * * * *

##### 6.2.6.2.x	Type: QosFlowAddModifyRequestItem

Table 6.2.6.2.x-1: Definition of type QosFlowAddModifyRequestItem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| qfi | Qfi | M | 1 | This IE shall contain the MBS QoS Flow Identifier. |  |
| qosFlowProfile | QosFlowProfile | O | 0..1 | When present, this IE shall contain the description of the MBS QoS Flow level QoS parameters. When modifying an MBS QoS flow, the IE shall only contain the MBS QoS Flow profile's attributes which are modified. |  |

* * * Next Change * * * *

##### 6.2.6.2.y	Type: QosFlowProfile

Table 6.2.6.2.y-1: Definition of type QosFlowProfile

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| 5qi | 5Qi | M | 1 | This IE shall contain the 5G QoS Identifier (5QI) of the MBS QoS flow. |
| nonDynamic5Qi | NonDynamic5Qi | C | 0..1 | When present, this IE shall indicate the QoS Characteristics for a standardized or pre-configured 5QI for downlink. (NOTE) |
| dynamic5Qi | Dynamic5Qi | C | 0..1 | When present, this IE shall indicate the QoS Characteristics for a Non-standardised or not pre-configured 5QI for downlink. (NOTE) |
| arp | Arp | C | 0..1 | This IE shall be present when establishing an MBS QoS flow; it may be present when modifying an MBS QoS flow. When present, this IE shall contain the Allocation and Retention Priority (ARP) assigned to the MBS QoS flow. |
| gbrQosFlowInfo | GbrQosFlowInformation | C | 0..1 | This IE shall be present when establishing a GBR MBS QoS flow or if the GBR MBS QoS flow information is modified. |
| NOTE : Either the nonDynamic5Qi IE or the dynamic5Qi IE may be present when establishing a QoS flow. Either the nonDynamic5Qi IE or the dynamic5Qi IE may be present when modifying a QoS flow; when present, the received nonDynamic5Qi IE or dynamic5Qi IE shall replace any value received previously for this IE. |  |  |  |  |

* * * Next Change * * * *

##### 6.2.6.2.z	Type: GbrQosFlowInformation

Table 6.2.6.2.z-1: Definition of type GbrQosFlowInformation

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| maxFbrDl | BitRate | M | 1 | Maximum Bit Rate in Downlink. See 3GPP TS 23.501 [2]. |
| guaFbrDl | BitRate | M | 1 | Guaranteed Bit Rate in Downlink. See 3GPP TS 23.501 [2]. |
| maxPacketLossRateDl | PacketLossRate | O | 0..1 | Maximum rate for lost packets that can be tolerated in the downlink direction. See 3GPP TS 23.501 [2]. |

* * * Next Change * * * *

## A.3	Nmbsmf_MBSSession API

openapi: 3.0.0

info:

title: Nmbsmf-MBSSession

version: 1.0.0-alpha.5

description: |

<API Name> Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

ContextStatusEventReport:

description: Context Status Event Report

type: object

properties:

eventType:

$ref: '#/components/schemas/ContextStatusEventType'

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

qosInfo:

$ref: '#/components/schemas/QosInfo'

statusInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionActivityStatus'

mbsServiceArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsServiceArea'

mbsAreaSessions:

description: >

A map (list of key-value pairs) where the key identifies an areaSessionId

additionalProperties:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/MbsAreaSession'

minProperties: 1

required:

- eventType

- timeStamp

[…]

N2MbsSmInfo:

description: N2 MBS Session Management information

type: object

properties:

ngapIeType:

$ref: '#/components/schemas/NgapIeType'

ngapData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

QosInfo:

description: QoS Information

type: object

properties:

qosFlowsAddModRequestList:

type: array

items:

$ref: '#/components/schemas/QosFlowAddModifyRequestItem'

minItems: 1

qosFlowsRelRequestList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

minItems: 1

QosFlowAddModifyRequestItem:

description: Individual MBS QoS flow requested to be created or modified

type: object

properties:

qfi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

qosFlowProfile:

$ref: '#/components/schemas/QosFlowProfile'

required:

- qfi

QosFlowProfile:

description: MBS QoS flow profile

type: object

properties:

5qi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5Qi'

nonDynamic5Qi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NonDynamic5Qi'

dynamic5Qi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dynamic5Qi'

arp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Arp'

gbrQosFlowInfo:

$ref: '#/components/schemas/GbrQosFlowInformation'

required:

- 5qi

GbrQosFlowInformation:

description: GBR MBS QoS flow information

type: object

properties:

maxFbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

guaFbrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

maxPacketLossRateDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

required:

- maxFbrDl

- guaFbrDl

[…]

* * * End of Changes * * * *