3GPP TSG-CT Meeting #96	CP-221291

# Budapest, Hungary, 05th – 06th June 2022

3GPP TSG-CT WG4 Meeting #110-e	C4-223309

# E-Meeting, 12th – 20th May 2022

Title:	LS on clarification on per access NSAC requirement

Response to:

Release:	Rel-17

Work Item:	eNS_Ph2

Source:	CT4

To:	SA2

Cc:	CT

Contact Person:

Name:	Zhijun Li

E-mail Address:	li.zhijun3@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	C4-223310 (agreed)

1. Overall Description:

CT4 is currently working on release-17 eNS_Ph2 to finish the work, and now finds ambiguity in stage 2 specification which has impacts to CT4 specification of TS29.536. See below:

<<TS23.501, clause 5.15.11.1>> Since the UE may register or deregister for an S-NSSAI via 3GPP access and/or non-3GPP access as described in clause 5.15.5.2.1. The Allowed NSSAI for the access type may change while the UE is registering to a network. The AMF provides the Access Type to the NSACF when triggering a request to increase or decrease the current number of UEs registered with a S-NSSAI. The NSACF may take the Access Type into account for increasing and decreasing the number of UEs per network slice by storing the UE ID with the associated one or more Access Type(s), i.e. the NSACF is able to add or remove a registration for the UE ID for each Access Type and trigger the increase or decrease of the current number of UEs registered with a S-NSSAI based on a policy that takes the access type into account. If the Access Type provided by the AMF is not configured for NSAC in the NSACF, the NSACF always accepts the request from the AMF without increasing or decreasing the number of UEs. If the Access Type provided by the AMF is configured for NSAC in the NSACF and the maximum number is reached, the NSACF sends a reject response to the AMF including the access type. NOTE 2: For example, if the NSACF is configured to apply NSAC for 3GPP Access Type only, the NSACF counts registration via 3GPP access type only. If the NSACF is configured to apply NSAC for both Access Types, and the UE newly registers via 3GPP access while the UE is already registered via non-3GPP access (or vice versa), the NSACF updates the UE ID entry with both 3GPP Access Type and non-3GPP Access Type and the NSACF may count the UE once or twice based on its policy.

CT4 had agreed in previous meetings that per-access quota may be configured, however after recent discussions, most companies in CT4 think that there is only one total quota configured for the entire PLMN, regardless the NSAC may be configured for one specific access type or for both access types. However, there is other opinion from one company that different quotas may additionally be configured for each access type in a PLMN.

Thus, CT4 would like to ask SA2 to answer the question below:

Q1: Is there any requirement to possibly configure different quotas for each access type in a PLMN, or there is only one total quota configured for the PLMN, if NSAC is configured in the network to control the number of UEs / PDU sessions?

Q2: If the answer to Q1 is NO, what is the need of sending Access-Type in the response message of NSAC request?

CT4 has agreed C4-223310 (CR#0011) in CT4#110E meeting and expects it to be approved in the CT#96 plenary meeting in June unless CT4/CT receives any counter-indication from SA2.

2. Actions:

To SA2 group.

ACTION: 	CT4 kindly asks SA2 group to answer the above question. And, update their specifications accordingly, if needed.

3. Date of Next CT4 Meetings:

3GPP TSG CT4#111e			08/2022	E-Meeting

3GPP TSG CT4#112e			10/2022	E-Meeting