3GPP TSG-CT WG1 Meeting #111bis	C1-184048

# Sophia-Antipolis (France), 9-13 July 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 0015 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for abnormal cases in the UE of service request procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, NTT DOCOMO, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  |  | Date: |  | 2018-07-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Issue-1: If MO MMTEL voice, MO MMTEL video or MO SMSoIP is barred by UAC, the UE in 5GMM-IDLE mode is operating in the single-registration mode and the UE's usage setting is "voice centric", and the UE decides to attempt the access in EPC: - if the N26 is supported, the UE needs to perform EMM attach procedure or EMM tracking area update procedure (rather than EMM service request procedure) as described in subclause 4.8.2.2. - if the N26 is not supported, the UE needs to perform EMM attach procedure with ESM UE requested PDN connectivity with request type ="handover" and APN="IMS" and PDN type derived PDU session type of the IMS PDU session (rather than of EMM service request procedure) as described in subclause 4.8.2.3. Issue-2: If MO MMTEL voice, MO MMTEL video or MO SMSoIP is barred by UAC and the UE operates in dual-registration mode, and the UE decides to attempt the access in EPC: - if the UE is already attached in EPC, the UE needs to perform ESM UE requested PDN connectivity with request type ="handover" and APN="IMS" and PDN type derived PDU session type of the IMS PDU session (rather than of EMM service request procedure); and - if the UE is not attached in EPC yet, the UE needs to perform EMM attach procedure with ESM UE requested PDN connectivity with request type ="handover" and APN="IMS" and PDN type derived PDU session type of the IMS PDU session (rather than of EMM service request procedure); as described in subclause 4.8.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Abnormal case in the UE due to a barred access attempt updated to refer to subclause 4.8.2 and subclause 4.8.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to move to EPS after MO MMTEL voice, MO MMTEL video or MO SMSoIP is barred by UAC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** change *****

#### 5.6.1.7	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	T3517 expired.

The UE shall enter the state 5GMM-REGISTERED.

If the UE triggered the service request procedure in 5GMM-IDLE mode, then the 5GMM sublayer shall abort the procedure and release locally any resources allocated for the service request procedure.

If the UE triggered the service request procedure in 5MM-CONNECTED mode, the 5GMM sublayer shall abort the procedure, and stay in 5GMM-CONNECTED mode.

b)	The lower layers indicate that the access attempt is barred.

The UE shall not start the service request procedure. The UE stays in the current serving cell and applies the normal cell reselection process.

If the event which triggered the access attempt was an MO-MMTEL-voice-call-started indication, an MO-MMTEL-video-call-started indication or an MO-SMSoIP-attempt-started indication and:

1)	if the UE in 5GMM-IDLE mode is operating in the single-registration mode and the UE's usage setting is "voice centric", the UE may attempt to select an E-UTRA cell connected to EPC. If the UE finds a suitable E-UTRA cell connected to EPC, it then proceeds with the appropriate EMM specific procedures and, if necessary, ESM procedures to make a PDN connection providing access to IMS available; see subclause 4.8.2 and 3GPP TS 24.301 [15];

Editor's note:	It is FFS whether the UE is recommended to attempt to select E-UTRA cell connected to EPC in this case.

2)	if the UE is operating in the dual-registration mode, the UE may proceed in S1 mode with the appropriate EMM specific procedures and ESM procedures to make a PDN connection providing access to IMS available; see subclause 4.8.3 and 3GPP TS 24.301 [15]; or

3)	otherwise, the UE NAS layer shall notify the upper layers that the access attempt is barred. In this case, upon receiving an indication from the lower layers that the barring is alleviated for the access category with which the access attempt was associated, the NAS shall notify the upper layers that the barring is alleviated for the access category. Otherwise, the service request procedure is started, if still needed, when the lower layers indicate that the barring is alleviated for the access category with which the access attempt was associated.

Editor's note:	Abnormal case handling for N1 NAS signalling connection establishment rejected by the network with "Extended wait time" received from lower layers is FFS.

c)	Timer T3346 is running.

The UE shall not start the service request procedure unless:

1)	the UE receives a paging;

2)	the UE receives a NOTIFICATION message over non-3GPP access when the UE is in 5GMM-CONNECTED mode over non-3GPP access and in 5GMM-IDLE mode over 3GPP access;

3)	the UE receives a NOTIFICATION message over 3GPP access when the UE is in 5GMM-CONNECTED mode over 3GPP access and in 5GMM-IDLE mode over non-3GPP access;

4)	the UE is a UE configured for high priority access in selected PLMN; or

5)	the UE has a PDU session for emergency services established or is establishing a PDU session for emergency services;

If the UE is in 5GMM-IDLE mode, the UE stays in the current serving cell and applies normal cell reselection process. The service request procedure is started, if still necessary, when timer T3346 expires or is stopped.

d)	Registration procedure for mobility and periodic registration update is triggered.

The UE shall abort the service request procedure, stop timer T3517, if running and perform the registration procedure for mobility and periodic registration update. The "follow-on request pending" indication shall be set in the REGISTRATION REQUEST message.

e)	Switch off.

If the UE is in state 5GMM-SERVICE-REQUEST-INITIATED at switch off, the de-registration procedure shall be performed.

f)	De-registration procedure collision.

If the UE receives a DEREGISTRATION REQUEST message from the network in state 5GMM-SERVICE-REQUEST-INITIATED, the UE shall progress the DEREGISTRATION REQUEST message and the service request procedure shall be aborted.