3GPP TSG-CT WG1 Meeting #111bis	C1-184938

# Sophia-Antipolis (France), 9-13 July 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 0028 | rev | 2 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for operator-defined access categories |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Qualcomm Incorporated, T-Mobile USA, NTT Docomo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  |  | Date: |  | 2018-07-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Issue-1 Subclause 4.5.3 refers to "Operator-defined access categories" and "Operator-defined access category definitions". These terms seem to have the same meaning and thus it is proposed to use "Operator-defined access category definitions" whenever referring to the information provided by the network. Issue-2 24.501 contains the following editor's notes: Editor’s note: The encoding of the operator-defined access categories is FFS. Editor’s note: Whether the operator-defined access categories are sent to the UE in a CONFIGURATION UPDATE COMMAND message, a DL NAS TRANSPORT message or another NAS message is FFS. Issue-3 It is not clear how the UE acts when several operator-defined access category definitions have the same precedence value. It is proposed to state that each operator-defined access category definition has a different precedence value. Issue-4 It is not clear how to configure the UE with information that a particular operator-defined access category number is to be used: - for access attemps related to DNN X (regardless of application which sent them); or - for access attemps related to app ID Y (regardless of DNN to which they relate). It is proposed to enable several operator-defined access category definitions to contain the same operator-defined access category number and then the UE would receive two operator-defined access category definitions with the same operator-defined access category number but different criteria. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Coding and transport method for operator-defined access category definitions is defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No standardized means to configure the UE with operator-defined access category definitions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.5.2, 4.5.3, 5.4.4.1, 5.4.4.2, 5.4.4.3, 8.2.19.1, 8.2.19.x (new), 9.10.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** change *****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.101: "Service aspects; Service principles".

[3]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[4]	3GPP TS 23.003: "Numbering, addressing and identification".

[5]	3GPP TS 23.122: "Non-Access-Stratum functions related to Mobile Station (MS) in idle mode".

[6]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[7]	3GPP TS 23.401: "GPRS enhancements for E-UTRAN access".

[8]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[9]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[10]	3GPP TS 23.503: " Policy and Charging Control Framework for the 5G System; Stage 2".

[11]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[12]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[13]	3GPP TS 24.011: "Point-to-Point Short Message Service (SMS) support on mobile radio interface".

[14]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[15]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[16]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3"

[17]	3GPP TS 24.368: "Non-Access Stratum (NAS) configuration Management Object (MO)".

[18]	3GPP TS 24.502: "Access to the 3GPP 5G System (5GS) via non-3GPP access networks; Stage 3".

[19]	3GPP TS 24.5xx: "UE policies for 5G System (5GS); Stage 3".

[20]	3GPP TS 24.623: "Extensive Markup Language (XML) Configuration Access Protocol (XCAP) over the Ut interface for Manipulating Supplementary Services".

[21]	3GPP TS 29.507: "5G System; Access and Mobility Policy Control Service; Stage 3".

[22]	3GPP TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

[23]	3GPP TS 33.102: "3G security; Security architecture".

[24]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[25]	3GPP TS 36.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[26]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[27]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[28]	3GPP TS 38.304: "New Generation Radio Access Network; User Equipment (UE) procedures in Idle mode".

[29]	3GPP TS 38.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[30]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[31]	3GPP TS 38.413: "NG Radio Access Network (NG-RAN); NG Application Protocol (NGAP)".

[32]	IETF RFC 768: "User Datagram Protocol".

[33]	IETF RFC 793: "Transmission Control Protocol."

[34]	IETF RFC 3748: "Extensible Authentication Protocol (EAP)".

[35]	IETF RFC 3736 (April 2004): "Stateless Dynamic Host Configuration Protocol (DHCP) Service for IPv6"

[36]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[37]	IETF RFC 4282: "The Network Access Identifier".

[38]	IETF RFC 4303: "IP Encapsulating Security Payload (ESP)".

[39]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[40]	IETF RFC 5448: "Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

[41]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)".

[42]	ITU-T Recommendation E.212: "The international identification plan for mobile terminals and mobile users".

[rfc4122]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

***** change *****

### 4.5.2	Determination of the access identities and access category associated with a request for access

When the UE needs to initiate an access attempt in one of the events listed in subclause 4.5.1, the UE shall determine one or more access identities from the set of standardized access identities, and one access category from the set of standardized access categories and operator-defined access categories, to be associated with that access attempt.

The set of the access identities applicable for the request is determined by the UE in the following way:

a)	for each of the access identities 1, 2, 11, 12, 13, 14 and 15 in table 4.5.2.1, the UE shall check whether the access identity is applicable in the selected PLMN, if a new PLMN is selected, or otherwise if it is applicable in the RPLMN or equivalent PLMN; and

b)	if none of the above access identities is applicable, then access identity 0 is applicable.

Table 4.5.2.1: Access identities

| Access Identity number | UE configuration |
| --- | --- |
| 0 | UE is not configured with any parameters from this table |
| 1 (NOTE 1) | UE is configured for multimedia priority service (MPS). |
| 2 (NOTE 2) | UE is configured for mission critical service (MCS). |
| 3-10 | Reserved for future use |
| 11 (NOTE 3) | Access Class 11 is configured in the UE. |
| 12 (NOTE 3) | Access Class 12 is configured in the UE. |
| 13 (NOTE 3) | Access Class 13 is configured in the UE. |
| 14 (NOTE 3) | Access Class 14 is configured in the UE. |
| 15 (NOTE 3) | Access Class 15 is configured in the UE. |
| NOTE 1: Access identity 1 is used by UEs configured for MPS, in the PLMNs where the configuration is valid. The PLMNs where the configuration is valid are HPLMN (if the EHPLMN list is not present or is empty) or EHPLMN (if the EHPLMN list is present), visited PLMNs of the home country, and configured visited PLMNs outside the home country. NOTE 2: Access identity 2 is used by UEs configured for MCS, in the PLMNs where the configuration is valid. The PLMNs where the configuration is valid are HPLMN (if the EHPLMN list is not present or is empty) or EHPLMN (if the EHPLMN list is present). NOTE 3: Access identities 11 and 15 are valid in HPLMN (if the EHPLMN list is not present or is empty) or EHPLMN (if the EHPLMN list is present). Access Identities 12, 13 and 14 are valid in HPLMN and visited PLMNs of home country only. For this purpose the home country is defined as the country of the MCC part of the IMSI. |  |

Editor's note:	The definition and configuration of Access Classes 11 to 15 for 5GS is FFS.

Editor's note:	How the UE is configured for MPS and MCS is FFS.

The UE uses the MPS indicator bit of the 5GS network feature support IE to determine if access identity 1 is valid when the UE is not in the country of its HPLMN (see 3GPP TS 23.122 [5]). Processing of the MPS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message is described in subclause 5.5.1.2.4 and subclause 5.5.1.3.4. The UE shall not consider access identity 1 to be valid when the UE is not in the country of its HPLMN prior to receiving the MPS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message being set to "Access identity 1 valid in RPLMN or equivalent PLMN".

When the UE is in the country of its HPLMN, the contents of the USIM files EFUAC_AIC and EFACC as specified in 3GPP TS 31.102 [22] and the rules specified in table 4.5.2.1 are used to determine the applicability of access identity 1 and access classes 11 - 15. When the UE is not in the country of its HPLMN, the contents of the USIM files EFUAC_AIC and EFACC are not applicable.

In order to determine the access category applicable for the access attempt, the NAS shall check the rules in table 4.5.2.2, and use the access category for which there is a match for barring check. If the access attempt matches more than one rule, the access category of the lowest rule number shall be selected.

Table 4.5.2.2: Mapping table for access categories

| Rule # | Type of access attempt | Requirements to be met | Access Category |
| --- | --- | --- | --- |
| 1 | Response to paging | Access attempt is for MT access | 0 (= MT_acc) |
| 2 | Emergency | UE is attempting access for an emergency session (NOTE 1, NOTE 2) | 2 (= emergency) |
| 3 | Access attempt for operator-defined access category | UE was provided with operator-defined access category definitions for the current PLMN as specified in subclause 4.5.3, and access attempt is matching criteria of an operator-defined access category definition | 32-63 (= based on operator classification) |
| 4 | Access attempt for delay tolerant service | UE is configured for NAS signalling low priority, the PLMN is broadcasting one of the categories a, b or c, and the UE is a member of the broadcasted category in the selected PLMN or RPLMN/equivalent PLMN (NOTE 3, NOTE 5) | 1 (= delay tolerant) |
| 5 | MO MMTel voice call | Access attempt is for MO MMTel voice call or for NAS signalling connection recovery during ongoing MO MMTel voice call (NOTE 2) | 4 (= MO MMTel voice) |
| 6 | MO MMTel video call | Access attempt is for MO MMTel video call or for NAS signalling connection recovery during ongoing MO MMTel video call (NOTE 2) | 5 (= MO MMTel video) |
| 7 | MO SMS over NAS or MO SMSoIP | Access attempt is for MO SMS over NAS (NOTE 4) or MO SMS over SMSoIP transfer or for NAS signalling connection recovery during ongoing MO SMS or SMSoIP transfer (NOTE 2) | 6 (= MO SMS and SMSoIP) |
| 8 | UE NAS initiated 5GMM specific procedures | Access attempt is for MO signalling | 3 (= MO_sig) |
| 9 | UE NAS initiated 5GMM connection management procedures or 5GMM NAS transport procedure | Access attempt is for MO data | 7 (= MO_data) |
| NOTE 1: This includes 5GMM specific procedures while the service is ongoing and 5GMM connection management procedures required to establish a PDU session with request type = "initial emergency request" or "existing emergency PDU session", or to re-establish user-plane resources for such a PDU session. This further includes the service request procedure initiated with a SERVICE REQUEST message with the Service type IE set to "emergency services fallback". NOTE 2: Access for the purpose of NAS signalling connection recovery during an ongoing service is mapped to the access category of the ongoing service in order to derive an RRC establishment cause, but barring checks will be skipped for this access attempt. NOTE 3: If the UE selects a new PLMN, then the selected PLMN is used to check the membership; otherwise the UE uses the RLPMN or a PLMN equivalent to the RPLMN. NOTE 4: This includes the 5GMM connection management procedures triggered by the UE-initiated NAS transport procedure for transporting the MO SMS. NOTE 5: The UE configured for NAS signalling low priority is not supported in this release of specification. |  |  |  |

***** change *****

### 4.5.3	Operator-defined access categories

Operator-defined access category definitions can be signalled to the UE using NAS signalling. Each operator-defined access category definition consists of the following parameters:

a)	a precedence value which indicates in which order the UE shall evaluate the operator-defined category definition for a match;

b)	an operator-defined access category number, i.e. access category number in the 32-63 range that uniquely identifies the access category in the PLMN in which the access categories are being sent to the UE; and

c)	one or more access category criteria type and associated access category criteria type values. The access category criteria type can be set to one of the following:

1)	DNN name;

2)	5QI;

Editor's note:	Whether the 5QI is a suitable access category criteria type is FFS.

3)	OS Id + OS App Id of application triggering the access attempt; or

4)	S-NSSAI; and

d)	optionally, a standardized access category, that is used in combination with the access identities to determine the establishment cause.

Editor's note:	Other access category criteria types, in particular whether QFI is a suitable parameter, are FFS.

Editor's note:	When the standardized access category, that is used in combination with the access identities to determine the establishment cause, is not provided, then it is FFS how the UE derives RRC establishment cause.

NOTE 1:	An access category criteria type can be associated with more than one access category criteria values. In this case, the access attempt matches the access category if the access criteria for the access attempt match any of the associated access criteria type values.

Each operator-defined access category definition has a different precedence value.

Several operator-defined access category definitions can have the same operator-defined access category number.

If the UE is configured with operator-defined access category definitions for a PLMN, then access control in 5GMM-IDLE mode will only be performed for the event a) defined in subclause 4.5.1.

If the UE is configured with operator-defined access category definitions for a PLMN, then access control in 5GMM-CONNECTED mode and in 5GMM-CONNECTED mode with RRC inactive indication will only be performed for the events 1) to 5) defined in subclause 4.5.1.

Upon receiving a NAS signalling message with one or more operator-defined access category definitions, the UE shall store the operator-defined access category definitions for the registered PLMN.

Upon receiving a NAS signalling message with zero operator-defined access category definitions, the UE shall delete the operator-defined access category definitions stored for the registered PLMN.

When the UE is switched off, the UE shall keep the operator-defined access category definitions so that the operator-defined access category definitions can be used after switch on.

When the UE selects a new PLMN which is not equivalent to the previously selected PLMN, the UE shall stop using the operator-defined access category definitions configured for the previously selected PLMN and should keep the operator-defined access category definitions configured for the previously selected PLMN.

NOTE 2:	When the UE selects a new PLMN which is not equivalent to the previously selected PLMN, the UE can discard the operator-defined access category definitions configured for the previously selected PLMN e.g. if there is no storage space in the UE.

***** change *****

#### 5.4.4.1	General

The purpose of this procedure is to:

a)	allow the AMF to update the UE configuration for access and mobility management-related parameters decided and provided by the AMF by providing new parameter information within the command; or

b)	request the UE to perform a registration procedure for mobility and periodic registration update towards the network to update access and mobility management-related parameters decided and provided by the AMF (see subclause 5.5.1.3).

The procedure may be initiated by the network and can only be used when the UE has an established 5GMM context, and the UE is in 5GMM-CONNECTED mode. The AMF may require a confirmation response in order to ensure that the parameter has been updated by the UE.

The following parameters are supported by the generic UE configuration update procedure without the need for triggering the UE to perform the registration procedure for mobility and periodic registration update:

a)	5G-GUTI;

b)	TAI list;

c)	Service area list;

d)	Network identity and time zone information (Full name for network, short name for network, local time zone, universal time and local time zone, network daylight saving time);

e)	LADN information;

f)	Rejected NSSAI; and

g)	operator-defined access category definitions.

The following parameters may trigger the UE to perform the registration procedure for mobility and periodic registration update:

a)	Allowed NSSAI; or

b)	Configured NSSAI.

The following parameter requires triggering the UE to perform the registration procedure for mobility and periodic registration update:

a)	MICO.

Editor's note:	Other parameters requiring negotiation are FFS.

Figure 5.4.4.1.1: Generic UE configuration update procedure

***** change *****

#### 5.4.4.2	Generic UE configuration update procedure initiated by the network

The AMF shall initiate the generic UE configuration procedure by sending the CONFIGURATION UPDATE COMMAND message to the UE.

The AMF shall in the CONFIGURATION UPDATE COMMAND message either:

a)	include one or more of 5G-GUTI, TAI list, allowed NSSAI that may include the mapping to the configured NSSAI for the HPLMN, LADN information, service area list, MICO indication NITZ information, configured NSSAI that may include the mapping to the configured NSSAI for the HPLMN, rejected NSSAI, or operator-defined access category definitions;

b)	indicate registration requested; or

c)	a combination of both.

If an acknowledgement from the UE is requested, the AMF shall indicate acknowledgement requested in the Configuration update indication IE in the CONFIGURATION UPDATE COMMAND message and shall start timer T3555. Acknowledgement shall be requested for all parameters except when only NITZ is included.

To initiate parameter re-negotiation between the UE and network, the AMF shall indicate "registration requested" in the Configuration update indication IE in the CONFIGURATION UPDATE COMMAND message. In this case, the acknowledgement shall be requested.

If a new allowed NSSAI information or AMF re-configuration of supported S-NSSAIs requires an AMF relocation, the AMF shall indicate "registration requested" in the Configuration update indication IE and include the Allowed NSSAI IE in the CONFIGURATION UPDATE COMMAND message. In this case, the acknowledgement shall be requested.

If the AMF includes a new configured NSSAI in the CONFIGURATION UPDATE COMMAND message and the new configured NSSAI requires an AMF relocation as specified in 3GPP TS 23.501 [8], the AMF shall indicate "registration requested" in the Configuration update indication IE in the message.

During an established 5GMM context, the network may send none, one, or more CONFIGURATION UPDATE COMMAND messages to the UE. If more than one CONFIGURATION UPDATE COMMAND message is sent, the messages need not have the same content.

***** change *****

#### 5.4.4.3	Generic UE configuration update accepted by the UE

Upon receiving the CONFIGURATION UPDATE COMMAND message, the UE shall use the contents to update appropriate information stored within the UE.

If acknowledgement requested is indicated in the Configuration update indication IE in the CONFIGURATION UPDATE COMMAND message and:

a)	if all information elements included are successfully accepted by the UE; or

b)	if "registration requested" in the Configuration update indication IE is indicated;

the UE shall send a CONFIGURATION UPDATE COMPLETE message.

If the UE receives a new 5G-GUTI in the CONFIGURATION UPDATE COMMAND message, the UE shall consider the new 5G-GUTI as valid, the old 5G-GUTI as invalid, stop timer T3519 if running, and delete any stored SUCI; otherwise, the UE shall consider the old 5G-GUTI as valid.

If the UE receives a new TAI list in the CONFIGURATION UPDATE COMMAND message, the UE shall consider the new TAI list as valid and the old TAI list as invalid; otherwise, the UE shall consider the old TAI list as valid.

If the UE receives a new service area list in the CONFIGURATION UPDATE COMMAND message, the UE shall consider the new service area list as valid and the old service area list as invalid; otherwise, the UE shall consider the old service area list, if any, as valid.

If the UE receives new NITZ information in the CONFIGURATION UPDATE COMMAND message, the UE considers the new NITZ information as valid and the old NITZ information as invalid; otherwise, the UE shall consider the old NITZ information as valid.

If the UE receives a LADN information IE in the CONFIGURATION UPDATE COMMAND message, the UE shall consider the old LADN information as invalid and the new LADN information as valid, if any; otherwise, the UE shall consider the old LADN information as valid.

If the UE receives a new allowed NSSAI for the associated access type in the CONFIGURATION UPDATE COMMAND message, the UE shall consider the new allowed NSSAI as valid for the associated access type, store the allowed NSSAI for the associated access type as specified in subclause 4.6.2.2 and consider the old allowed NSSAI for the associated access type as invalid; otherwise, the UE shall consider the old Allowed NSSAI as valid for the associated access type.

If the UE receives an allowed NSSAI in the CONFIGURATION UPDATE COMMAND message and the UE has one or more PDU sessions associated with S-NSSAI(s) not included in the received allowed NSSAI, the UE shall locally release all such PDU session(s).

If the UE receives a new configured NSSAI in the CONFIGURATION UPDATE COMMAND message, the UE shall consider the new configured NSSAI for the registered PLMN as valid and the old configured NSSAI for the registered PLMN as invalid; otherwise, the UE shall consider the old configured NSSAI for the registered PLMN as valid The UE shall store the new configured NSSAI as specified in subclause 4.6.2.2.

If the UE receives a operator-defined access category definitions IE in the CONFIGURATION UPDATE COMMAND message, the UE shall consider the old operator-defined access category definitions as invalid and the new operator-defined access category definitions as valid, if any; otherwise, the UE shall consider the old new operator-defined access category definitions as valid.

If the CONFIGURATION UPDATE COMMAND message indicates "registration requested" in the Configuration update indication IE and:

-	either a new allowed NSSAI or a new configured NSSAI or both are included, the UE shall, after the completion of the generic UE configuration update procedure, and the release of the existing N1 NAS signalling connection, start a registration procedure for mobility and periodic registration update as specified in subclause 5.5.1.3; or

-	an MICO indication is included, the UE shall, after the completion of the generic UE configuration update procedure, start a mobility registration update procedure as specified in subclause 5.5.1.3 to re-negotiate MICO mode with the network.

The UE receiving the rejected NSSAI in the CONFIGURATION UPDATE COMMAND message takes the following actions based on the rejection cause in the rejected NSSAI:

"S-NSSAI not available in the current PLMN"

The UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the current PLMN as specified in subclause 4.6.2.2 and not attempt to use this S-NSSAI in the current PLMN until switching off the UE or the UICC containing the USIM is removed.

"S-NSSAI not available in the current registration area"

The UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the current PLMN and registration area combination as specified in subclause 4.6.2.2 and not attempt to use this S-NSSAI in the current registration area until switching off the UE, the UE moving out of the current registration area or the UICC containing the USIM is removed.

***** change *****

#### 8.2.19.1	Message definition

The CONFIGURATION UPDATE COMMAND message is sent by the AMF to the UE. See table 8.2.19.1.1.

Message type:	CONFIGURATION UPDATE COMMAND

Significance:		dual

Direction:			network to UE

Table 8.2.19.1.1: CONFIGURATION UPDATE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | Security header type | Security header type 9.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 9.5 | M | V | 1/2 |
|  | Configuration update command message identity | Message type 9.7 | M | V | 1 |
| D- | Configuration update indication | Configuration update indication 9.10.3.16 | O | TV | 1 |
| 2C | 5G-GUTI | 5GS mobile identity 9.10.3.4 | O | TLV | 13 |
| 54 | TAI list | 5GS tracking area identity list 9.10.3.9 | O | TLV | 9-114 |
| 70 | Allowed NSSAI | NSSAI 9.10.3.34 | O | TLV | 4-74 |
| 27 | Service area list | Service area list 9.10.3.45 | O | TLV | 6-114 |
| 43 | Full name for network | Network name 9.10.3.33 | O | TLV | 3-n |
| 45 | Short name for network | Network name 9.10.3.33 | O | TLV | 3-n |
| 46 | Local time zone | Time zone 9.10.3.47 | O | TV | 2 |
| 47 | Universal time and local time zone | Time zone and time 9.10.3.48 | O | TV | 8 |
| 49 | Network daylight saving time | Daylight saving time 9.10.3.17 | O | TLV | 3 |
| 79 | LADN information | LADN information 9.10.3.27 | O | TLV-E | 3-1707 |
| B- | MICO indication | MICO indication 9.10.3.28 | O | TV | 1 |
| 31 | Configured NSSAI | NSSAI 9.10.3.34 | O | TLV | 4-146 |
| 11 | Rejected NSSAI | Rejected NSSAI 9.10.3.42 | O | TLV | 4-42 |
| x | Operator-defined access category definitions | Operator-defined access category definitions 9.10.3.x | O | TLV-E | 3-TBD |

***** change *****

#### 8.2.19.x	Operator-defined access category definitions

This IE may be included to assign new operator-defined access category definitions to the UE or delete the operator-defined access category definitions at the UE side.

***** change *****

#### 9.10.3.x	Operator-defined access category definitions

The purpose of the Operator-defined access category definitions information element is to provide the UE with the operator-defined access category definitions or to delete the operator-defined access category definitions at the UE.

The Operator-defined access category definitions information element is coded as shown in figure 9.10.3.x.1, figure 9.10.3.x.2 and table 9.10.3.x.1.

The Operator-defined access category definitions is a type 6 information element with a minimum length of 3 octets and a maximum length of TBD octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operator-defined access category definitions IEI |  |  |  |  |  |  |  | octet 1 |
| Length of operator-defined access category definitions contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Operator-defined access category definition 1 |  |  |  |  |  |  |  | octet 4 octet a |
| Operator-defined access category definition 2 |  |  |  |  |  |  |  | octet a+1* octet b* |
| … |  |  |  |  |  |  |  | octet b+1* octet g* |
| Operator-defined access category definition 1 n |  |  |  |  |  |  |  | octet g+1* octet h* |

Figure 9.10.3.x.1: Operator-defined access category definitions information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of operator-defined access category definition contents |  |  |  |  |  |  |  | octet 4 |
| Precedence value |  |  |  |  |  |  |  | octet 5 |
| PSAC | 0 Spare | 0 Spare | Operator-defined access category number |  |  |  |  | octet 6 |
| Length of criteria |  |  |  |  |  |  |  | octet 7 |
| Criteria |  |  |  |  |  |  |  | octet 8 octet a-1 |
| 0 Spare | 0 Spare | 0 Spare | Standardized access category |  |  |  |  | octet a* |

Figure 9.10.3.x.2: Operator-defined access category definition

Table 9.10.3.x.1: Operator-defined access category definitions information element

Value part of the Operator-defined access category definitions information element (octet 3 to h) The value part of the Operator-defined access category definitions information element consists of one or several operator-defined access category definition fields. Each operator-defined access category definition field is coded as described in figure 9.10.3.x.2. The length of each operator-defined access category definition field is determined by the length of operator-defined access category definition contents field.

Operator-defined access category definition (octet 4 to octet a): Precedence value (octet 5) Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 Precedence value 0 to 1 1 1 1 1 1 1 1 Precedence value 255 Operator-defined access category number (bits 5 to 1 of octet 6) Bits 5 4 3 2 1 0 0 0 0 0 Access category number 32 to 1 1 1 1 1 Access category number 63 Presence of standardized access category (PSAC) (bit 8 of octet 6) PSAC field indicates whether the standardized access category field is present or absent. Bit 8 0 Standardized access category field is not included 1 Standardized access category field is included Length of criteria (octet 7) Length of criteria field indicates binary coded length of the criteria field. Criteria (octets 8 to octet a-1) The criteria field contains zero or more criteria components fields. Each criteria component field shall be encoded as a sequence of a one octet criteria type field and zero or more octets criteria value field. The criteria type field shall be transmitted first. Criteria type Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 DNN type 0 0 0 0 0 0 1 0 OS Id + OS App Id type 0 0 0 0 0 0 1 1 S-NSSAI type All other values are reserved. For "DNN type", the criteria value field shall be encoded as a sequence of one octet DNN length-value pair count field and one or more DNN length-value pair fields. The DNN length-value pair count field indicates the number of included DNN length-value pair fields. Each DNN length-value pair field is coded as a sequence of one octet DNN value length field and a DNN value field. The DNN value length field indicates the length in octets of the DNN value field. The DNN value field contains an APN as specified in 3GPP TS 23.003 [4]. For "OS Id + OS App Id type", the criteria value field shall be encoded as a sequence of one octet app id value count field and one or more app id value fields. The app id value count field indicates the number of included app id value fields. Each app id value field is coded as a sequence of a sixteen octet OS id value field, one octet OS app id value length field and an OS app id value field. The OS app id value length field indicates the length in octets of the OS app id value field. The OS id value field contains a Universally Unique IDentifier (UUID) as specified in IETF RFC 4122 [rfc4122]. The OS app id value field contains an OS specific application identifier. Coding of the OS app id value field is out of scope of the present document. For "S-NSSAI type", the criteria value field shall be encoded as a sequence of one octet S-NSSAI length-value pair count field and one or more S-NSSAI length-value value fields. The S-NSSAI length-value pair count field indicates the number of included S-NSSAI length-value pair fields. Each S-NSSAI length-value pair field is coded as a sequence of one octet S-NSSAI value length field and an S-NSSAI value field. The S-NSSAI value length field indicates the length in octets of the S-NSSAI value field. The S-NSSAI value field contains one octet SST field optionally followed by three octets SD field. The SST field contains a SST. The SD field contains an SD. SST and SD are specified in 3GPP TS 23.003 [4]. Standardized access category (bits 5 to 1 of octet a) Standardized access category field indicates the access category number of the standardized access category, that is used in combination with the access identities to determine the establishment cause. Bits 5 4 3 2 1 0 0 0 0 0 Access category number 0 to 0 0 1 1 1 Access category number 7 All other values are reserved.

Editor's note:	Whether the 5QI is a suitable access category criteria type is FFS.