3GPP TSG-CT WG1 Meeting #116	C1-192739

# Xi'an, P.R. of China, 8-12 April 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 1062 | rev | 1 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on coding of "all other values are spare" |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtocol16 |  |  |  |  |  | Date: |  |  | 2019-03-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For the coding description of Integrity protection maximum data rate IE, it states “All other values are spare”. However, from binary coding perspective, it is impossible to encode “all other values are spare” as spare value normally was set to ‘0’ but in fact, all other values are not full ‘0’. Hence, here the “spare” value actually is a treatment not a coding value. It proposes to clarify the handling of “spare” values as one of defined value if received. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It proposes to clarify the handling of “spare” values as one of defined value if received. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Handling of “spare” values are not clear at the receiver. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.11.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.11.4.7	Integrity protection maximum data rate

The purpose of the integrity protection maximum data rate information element is for the UE to indicate to the network the maximum data rate per UE for user-plane integrity protection for uplink and the maximum data rate per UE for user-plane integrity protection for downlink that are supported by the UE.

The integrity protection maximum data rate is coded as shown in figure 9.11.4.7.1 and table 9.11.4.7.2.

The integrity protection maximum data rate is a type 3 information element with a length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Integrity protection maximum data rate IEI |  |  |  |  |  |  |  | octet 1 |
| Maximum data rate per UE for user-plane integrity protection for uplink |  |  |  |  |  |  |  | octet 2 |
| Maximum data rate per UE for user-plane integrity protection for downlink |  |  |  |  |  |  |  | octet 3 |

Figure 9.11.4.7.1: Integrity protection maximum data rate information element

Table 9.11.4.7.2: Integrity protection maximum data rate information element

| Maximum data rate per UE for user-plane integrity protection for uplink (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 64 kbps |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | Full data rate |
| All other values are spare. If received they shall be interpreted as "64 kbps". |  |  |  |  |  |  |  |  |  |
| Maximum data rate per UE for user-plane integrity protection for downlink (octet 3) |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 64 kbps |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | Full data rate |
| All other values are spare. If received they shall be interpreted as "64 kbps". |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *