3GPP TSG-CT WG1 Meeting #128-e	C1-210566

Electronic meeting, 25 February – 5 March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.371 | CR | 0115 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reference update: RFC 8864 and RFC 8873 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eWebRTCi-CT |  |  |  |  |  | Date: |  |  | 2021-02-02 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | IETF draft-ietf-mmusic-data-channel-sdpneg has now been published as RFC 8864, and IETF draft-ietf-mmusic-msrp-usage-data-channel has now been published as RFC 8873. Therefore the specification requires updating to the published versions. A number of editorial changes were implemented in draft-ietf-mmusic-data-channel-sdpneg-28 before it was published as RFC 8864 (e.g. title was changed), but there are no technical changes between the referenced draft version -28 and RFC 8864. A number of editorial changes were implemented in draft-ietf-mmusic-msrp-usage-data-channel-24 before it was published as RFC 8873 (e.g. title was changed), but there are no technical changes between the referenced draft version -24 and RFC 8873. |  |  |  |  |  |  |  |  |
| Summary of change: |  | IETF draft-ietf-mmusic-data-channel-sdpneg is replaced with RFC 8864 and IETF draft-ietf-mmusic-msrp-usage-data-channel is replaced with RFC 8873. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 3GPP specification is not aligned with IETF and will continue to refer to draft versions that are formally not available, leading to potential interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.1, 8.2.2, 8.2.3, 8.4.2, 8.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	IETF RFC 7118: "The WebSocket Protocol as a Transport for the Session Initiation Protocol (SIP)".

[3]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[4]	3GPP TS 23.228: " IP Multimedia Subsystem (IMS); Stage 2".

[5]	IETF RFC 5763: "Framework for Establishing a Secure Real-time Transport Protocol (SRTP) Security Context Using Datagram Transport Layer Security (DTLS)".

[6]	IETF RFC 5764: "Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP)".

[7]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[8]	3GPP TS 24.173: "IMS multimedia telephony communication service and supplementary services; Stage 3".

[9]	3GPP TS 33.203: "Access security for IP based services".

[10]	RFC 6750 (October 2012): "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[11]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) Centralized Services; Stage 2".

[12]	RFC 5009 (September 2007): "Private Header (P-Header) Extension to the Session Initiation Protocol (SIP) for Authorization of Early Media".

[13]	3GPP TS 23.334: "IMS Application Level Gateway (IMS-ALG) – IMS Access Gateway (IMS-AGW) interface".

[14]	RFC 4145 (September 2005): "TCP-Based Media Transport in the Session Description Protocol (SDP)".

[15]	RFC 8122 (March 2017): "Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP)".

[16]	draft-ietf-rtcweb-data-channel-13 (January 2015): "WebRTC Data Channels".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[17]	draft-ietf-rtcweb-data-protocol-09 (January 2015): "WebRTC Data Channel Establishment Protocol".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[18]	draft-ietf-mmusic-sctp-sdp-25 (March 2017): "Stream Control Transmission Protocol (SCTP)-Based Media Transport in the Session Description Protocol (SDP)".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[19]	RFC 3261 (June 2002): "SIP: Session Initiation Protocol".

[20]	RFC 3264 (June 2002): "An Offer/Answer Model with the Session Description Protocol (SDP)".

[21]	RFC 7675 (October 2015): "STUN Usage for Consent Freshness".

[22]	RFC 5245 (April 2010): "Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer/Answer Protocols".

[23]	RFC 8261 (November 2017): "Datagram Transport Layer Security (DTLS) Encapsulation of SCTP Packets".

[24]	RFC 6455 (December 2011): "The WebSocket Protocol".

[25]	draft-ietf-mmusic-sdp-bundle-negotiation-29 (April 2016): "Negotiating Media Multiplexing Using the Session Description Protocol (SDP)".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[26]	RFC 3581 (August 2003): "An Extension to the Session Initiation Protocol (SIP) for Symmetric Response Routing".

[27]	RFC 8898 (September 2020): "Third-Party Token-Based Authentication and Authorization for Session Initiation Protocol (SIP)".

[28]	RFC 6544 (March 2012): "TCP Candidates with Interactive Connectivity Establishment (ICE)".

[29]	Void.

[30]	draft-ietf-rtcweb-overview-18 (March 2017): "Overview: Real Time Protocols for Brower-based Applications".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[31]	Void.

[32]	RFC 3310 (September 2002): "Hypertext Transfer Protocol (HTTP) Digest Authentication Using Authentication and Key Agreement (AKA)".

[33]	RFC 4169 (November 2005): "Hypertext Transfer Protocol (HTTP) Digest Authentication Using Authentication and Key Agreement (AKA) Version-2".

[34]	3GPP TS 26.114: "IP multimedia subsystem (IMS); Multimedia telephony, Media handling and interaction".

[35]	RFC 7519 (May 2015): "JSON Web Token (JWT)".

[36]	RFC 8864 (January 2021): "Negotiation Data Channels Using the Session Description Protocol (SDP)".

[37]	RFC 8873 (January 2021): "Message Session Relay Protocol (MSRP) over Data Channels".

[38]	RFC 5761 (April 2010): "Multiplexing RTP Data and Control Packets on a Single Port".

[39]	draft-ietf-mmusic-mux-exclusive-12 (May 2017): "Indicating Exclusive Support of RTP/RTCP Multiplexing using SDP".

Editor's note [WI: TEI14, CR#0050]: The above document cannot be formally referenced until it is published as an RFC.

[40]	draft-ietf-mmusic-t140-usage-data-channel-14 (April 2020): "T.140 Real-time Text Conversation over WebRTC Data Channels".

Editor's note [WI: TEI14, CR#0051]: The above document cannot be formally referenced until it is published as an RFC.

[41]	Void.

[42]	RFC 8035 (November 2016): "Session Description Protocol (SDP) Offer/Answer Clarifications for RTP/RTCP Multiplexing".

[43]	draft-ietf-ice-trickle-09 (April 2017): "Trickle ICE: Incremental Provisioning of Candidates for the Interactive Connectivity Establishment (ICE) Protocol".

[44]	RFC 5766 (April 2010): "Traversal Using Relays around NAT (TURN): Relay Extensions to Session Traversal Utilities for NAT (STUN)".

# *** Next Change ***

## 8.1	General

This clause specifies the procedures for negotiating usage of, and opening and closing of, a WebRTC data channel between the WIC and the eP-CSCF.

WebRTC data channels are realized using an SCTP association running on top of DTLS, as described in RFC 8261 [23] and draft-ietf-rtcweb-data-channel [16].

If WebRTC data channels are supported, UDP transport of DTLS shall be supported and TCP transport of DTLS may be supported to enable traversal of UDP-blocking NATs/firewalls. ICE procedures as defined in 3GPP TS 24.229 [3] shall be used to negotiate the transport protocol. UDP shall be offered as ICE default candidate and TCP may be offered as additional ICE candidate.

Once the SCTP association has been negotiated (using draft-ietf-mmusic-sctp-sdp [18]) and established, the WIC and eIMS-AGW (controlled by the eP-CSCF) either use the Data Channel Establishment Protocol (DCEP) for opening data channels, as described in draft-ietf-rtcweb-data-protocol [17], or they use the SDP negotiation based on RFC 8864 [36].

Closing of a data channel is signalled via SCTP and, if SDP negotiation was used for the data channel establishment, also using RFC 8864 [36].

# *** Next Change ***

### 8.2.2	WIC originating call

Upon generating an SDP offer, the WIC shall insert an "m=" line with the proto value set to "UDP/DTLS/SCTP", and the fmt value set to "webrtc-datachannel" according to draft-ietf-mmusic-sctp-sdp [18], in the SDP offer. The WIC may additionally, within the same "m=" line, offer TCP transport protocol with appropriate ICE candidates according to RFC 6544 [28]. In addition, the WIC shall insert an SDP sctp-port attribute according to draft-ietf-mmusic-sctp-sdp [18].

The procedures of RFC 8864 [36] should be used as follows:

a)	If MSRP is transported over the data channel, the WIC shall follow the procedures of RFC 8873 [37].

b)	If T.140 is transported over the data channel, the WIC shall follow the procedures of draft-ietf-mmusic-t140-usage-data-channel [40].

NOTE:	Alternatively, if a single data channel is to be used, then knowledge of the usage of the data channel could be based on configuration, or could be implicitly determined based on proprietary information carried in the signalling protocol. Such mechanisms are outside the scope of this specification.

# *** Next Change ***

### 8.2.3	WIC terminating call

Upon receiving an SDP offer, with an "m=" line with the proto value set to "UDP/DTLS/SCTP", and the "m=" line fmt value set to "webrtc-datachannel", the WIC shall follow the procedures in draft-ietf-mmusic-sctp-sdp [18] for generating the associated SDP answer. In addition, the WIC shall insert an SDP sctp-port attribute according to draft-ietf-mmusic-sctp-sdp [18].

NOTE 1:	As specified in subclause 7.2.3, the WIC will perform the ICE procedures as defined in RFC 5245 [22] and possibly RFC 6544 [28], and determines the appropriate transport protocol (UDP or TCP) for the data channel in that manner.

The procedures of RFC 8864 [36] should be used as follows:

a)	If MSRP is transported over the data channel, the WIC shall follow the procedures of RFC 8873 [37].

b)	If T.140 is transported over the data channel, the WIC shall follow the procedures of draft-schwarz-mmusic-t140-usage-data-channel [40].

NOTE 2:	Alternatively, if a single data channel is to be used, then knowledge of the usage of the data channel could be based on configuration, or could be implicitly determined based on proprietary information carried in the signalling protocol. Such mechanisms are outside the scope of this specification.

# *** Next Change ***

### 8.4.2	WIC originating call

Upon receiving an SDP offer from the served WIC, with an "m=" line with the proto value set to "UDP/DTLS/SCTP", and the fmt value set to "webrtc-datachannel", according to draft-ietf-mmusic-sctp-sdp [18], the eP-CSCF shall:

-	remove the "m=" line from the SDP offer; and

-	for each media transported between the WIC and the eIMS-AGW using a data channel, and for which the eIMS-AGW is able to perform transport protocol interworking between a data channel and another transport protocol, insert an "m=" line describing the media and the transport protocol used for the media in the SDP offer;

before forwarding the SDP offer towards the remote user.

NOTE 1:	As specified in subclause 7.4.2, the eP-CSCF will perform the ICE procedures as defined in 3GPP TS 24.229 [3], and determines the appropriate transport protocol (UDP or TCP) for the data channel in that manner.

Upon receiving an SDP answer to the SDP offer, the eP-CSCF shall:

-	remove each "m=" line associated with an "m=" line that the eP-CSCF inserted in the associated SDP offer from the SDP answer;

-	insert an "m=" line with the proto value set to "UDP/DTLS/SCTP", and the fmt value set to "webrtc-datachannel" according to draft-ietf-mmusic-sctp-sdp [18], in the SDP answer;

-	insert an SDP sctp-port attribute according to draft-ietf-mmusic-sctp-sdp [18], in the SDP answer; and

-	for each media accepted by the remote user, for which the eIMS-AGW can perform transport protocol interworking between a data channel and another transport protocol, instruct the eIMS-AGW to perform the transport protocol interworking;

before forwarding the SDP answer towards the served WIC.

The procedures of RFC 8864 [36] should be used as follows:

a)	If MSRP is transported over the data channel, the eP-CSCF shall follow the procedures of RFC 8873 [37].

b)	If T.140 is transported over the data channel, the eP-CSCF shall follow the procedures of draft-ietf-mmusic-t140-usage-data-channel [40].

NOTE 2:	Alternatively, if a single data channel is to be used, then knowledge of the usage of the data channel could be based on configuration, or could be implicitly determined based on proprietary information carried in the signalling protocol. Such mechanisms are outside the scope of this specification.

# *** Next Change ***

### 8.4.3	WIC terminating call

Upon receiving an SDP offer from the remote user, the eP-CSCF shall:

-	remove each "m=" line that describes media which is transported between the WIC and the eIMS-AGW using a data channel from the SDP offer;

-	insert an "m=" line with the proto value set to "UDP/DTLS/SCTP", and the fmt value set to "webrtc-datachannel" according to draft-ietf-mmusic-sctp-sdp [18], in the SDP offer; and

-	insert an SDP sctp-port attribute according to draft-ietf-mmusic-sctp-sdp [18], in the SDP offer;

before forwarding the SDP offer towards the served WIC.

NOTE 1:	As specified in subclause 7.4.2, the eP-CSCF will perform the ICE procedures as defined in 3GPP TS 24.229 [3], and determines the appropriate transport protocol (UDP or TCP) for the data channel in that manner.

Upon receiving an SDP answer to the SDP offer, with an "m=" line with the proto value set to "UDP/DTLS/SCTP", and the fmt value set to "webrtc-datachannel", according to draft-ietf-mmusic-sctp-sdp [18], the eP-CSCF shall:

-	remove the "m=" line from the SDP answer;

-	for each media which the eIMS-AGW is able to perform transport protocol interworking between a data channel and another transport protocol, and for which the SDP offer contained an "m=" line, insert an "m=" line describing the media and the transport protocol used for the media in the SDP answer; and

-	for each media accepted by the remote user, for which the eIMS-AGW can perform transport protocol interworking between a data channel and another transport protocol, instruct the eIMS-AGW to perform the transport protocol interworking;

before forwarding the SDP answer towards the remote user.

The procedures of RFC 8864 [36] should be used as follows:

a)	If MSRP is transported over the data channel, the eP-CSCF shall follow the procedures of RFC 8873 [37].

b)	If T.140 is transported over the data channel, the eP-CSCF shall follow the procedures of draft-ietf-mmusic-t140-usage-data-channel [40].

NOTE 2:	Alternatively, if a single data channel is to be used, then knowledge of the usage of the data channel could be based on configuration, or could be implicitly determined based on proprietary information carried in the signalling protocol. Such mechanisms are outside the scope of this specification.

# *** End of Changes ***