3GPP TSG-CT WG1 Meeting #129-e	C1-212157

- Electronic meeting; 19-23 April 2021

Source:	Samsung

Title:	Eees_EECRegistration_Request Service Operation

Spec:	3GPP TS 24.558 v0.1.0

Agenda item:	17.2.16

Document for:	Agreement

1. Introduction

This pCR specifies Eees_EECRegistration_Request Service Operation.

2. Reason for Change

The Eees_EECRegistration_Request operation is defined in clause 8.4.2.4.2 of 3GPP TS 23.558. It is required to provide service operations details in stage#3.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.558 v0.1.0.

* * * First Change * * * *

#### 5.x.2.2	Eees_EECRegistration_Request

##### 5.x.2.2.1	General

This service operation is used by EEC to register itself with a given EES.

Editor’s note: Details about EEC security credentials, verification and authorization of Eees_EECRegistration Request, by the EES, to be aligned with security aspects defined by SA3.

##### 5.x.2.2.2	EEC registering to EES using Eees_EECRegistration_Request operation

For an EEC to register at the EES, the EEC shall send an HTTP POST message to the EES on the "EEC Registrations" collection resource. The body of the HTTP POST message shall include the EEC ID, may include UE identifier, AC Profile(s), proposed expiration time for the registration, EEC context ID obtained from a previous registration, as specified in clause 6.x.2.2.3.1. If EEC context ID is included in the body of the HTTP POST message, it shall also include Source EES ID and Source EES Endpoint of the EES that provided EEC context ID.

Upon receiving the HTTP POST message from the EEC, the EES shall:

a)	Process the EEC registration request information;

b)	verify if the EEC is authorized to register itself at EES. The EES further determines whether the requirements that were indicated in the AC Profile(s) is fulfilled by matching AC Service KPIs with registered EAS Servicve KPIs; and

c)	if the EEC is authorized to register with EES, then the EES shall;

1)	if the received EEC registration request contains an EEC context ID, a source EESID and EES endpoint, the EES retrieves the EEC's context from the source EES;

Editor’s note: The reference of how EES retrieves the EEC’s context from source EES will be added once it is available.

2)  the EEC creates a new resource with the EEC registration information as specified in clause 6.x.2.1, and assigns and stores new EEC context ID; and

3)	return the EEC registration information, including the resource URI of the EEC registration resource, in the response message. The response message may include expiration time to indicate when the EEC registration will automatically expire.  The response message may include a newly assigned EEC context ID. The URI of the created resource shall be returned in the “Location” HTTP header.

The EEC stores the new EEC context ID and uses it when it registers with another EES.

If the expiration time is provided, then to maintain the registration, the EEC shall send a registration update request (as described in clause 5.x.2.2.3) prior to the expiration time. If the registration update request is not received prior to the expiration time, then the EES shall treat the EEC as implicitly deregistered and remove the corresponding EEC registration resource.

* * * End of Change * * * *