3GPP TSG-CT WG1 Meeting #129-e	C1-212107

Electronic meeting; 19-23 April 2021

Source:	Huawei, HiSilicon

Title:	EN resolution of arranging PLMN in an area for Solution #24 KI#5.

Spec:	3GPP TR 24.811

Agenda item:	17.2.9

Document for:	Agreement

1. Introduction

This pCR attempts to resolve the following editors note for the solution #24 for KI #5

Editor's note: It is FFS on how the AMF of the serving PLMN arranges the PLMN in an order based on an area for equal distribution.

AMF is aware of the UEs that are registering in a particular area. So when the UEs register, as an example, if there are 2 PLMNs PLMN1 and PLMN2 that supports disaster roaming in an area, then the first set of UEs are assigned the PLMN list as {PLMN1, PLMN 2} and for the next set of UEs the assigned PLMN list is {PLMN2, PLMN1}

2. Reason for Change

Editors note need to be resolved.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 24.811.

* * * First Change * * * *

### 6.24.1	General

The solution aims at solving the below study item in the key issue #5

How the UE selects a PLMN if it is determined that a "Disaster Condition" applies;

a)	If the UE determines that a "Disaster Condition" applies as described in Key Issue #1 "Notification of Disaster Condition to the UE", then how to update PLMN selection procedure so that the UE avoids selecting the PLMN with "Disaster Condition".

b)	How the UE handles the list of "forbidden PLMNs" when selecting a PLMN indicating accessibility for the UE in the bullet a).

Once the UE detects "Disaster Condition" based on the solution for key issue #1, UE shall perform a PLMN selection and shall consider the list of “List of PLMNs to be used while in Disaster condition” in the PLMN selection. In the PLMN selection procedure, if the UE finds PLMN that is in the forbidden PLMN list, UE shall consider the forbidden PLMN also for PLMN selection if they are present in the “List of PLMNs to be used while in Disaster condition”. Forbidden PLMNs shall be considered in the same order in which they are present in the “List of PLMNs to be used while in Disaster condition”.

This is because the serving PLMN can arrange the PLMNs in a particular order to distribute the UEs so that all the UEs does not try to attempt registration on to the same PLMN for disaster roaming in case of a disaster condition.and if these PLMNs are available. When the UEs are registering in a particular area (before the disaster has happened), AMF can assign the priority of the PLMNs in the order in “List of PLMNs to be used while in Disaster condition”.e.g If there are 2 PLMNs, PLMN_1 and PLMN_2 supporting disaster roaming in a particular area, then AMF can assign the list as {PLMN1, PLMN2} for UE_1 and then {PLMN2, PLMN1} for UE_2.

As long as the "Disaster Condition" is applicable, PLMNs in the forbidden list can be considered as available if they are present in the “List of PLMNs to be used while in Disaster condition” and if those PLMNs indicate that they support disaster roaming. Once the "Disaster Condition" is over, UE shall treat the PLMNs in the forbidden PLMN list as forbidden. Otherwise no need for any change for the forbidden PLMN list.

Editor's note: For a UE supporting CAG, it is FFS how the CAG cells that are not in the Allowed CAG list of the UE are considered for disaster roaming.

* * * End of Change * * * *