3GPP TSG-CT WG1 Meeting #129-e	C1-212xxx

- Electronic meeting; 19-23 April 2021

Source:	Huawei, HiSilicon

Title:	EN resolution for Solution #29 for KI#6

Spec:	3GPP TR 24.811

Agenda item:	17.2.9

Document for:	Agreement

1. Introduction

This pCR attempts to resolve an EN for Solution #29 for KI#6: Notification that Disaster Condition is no longer applicable to the UEs.

2. Reason for Change

For Solution #29 for KI#6, there is a following EN :

“Editor's Note:	How does the Disaster Roaming PLMN AMF know the previous serving PLMN of the Disaster Inbound Roamer is FFS.”

The AMF of PLMN A determines the previously selected PLMN of the Disaster Inbound roamers when the Disaster Inbound Roamers register on PLMN A, as specified in the solution(s) to Key Issue #4 (Registration to the roaming PLMN without Disaster Condition in case of Disaster Condition).

Hence, this EN for Solution #29 can be removed as it can be resolved by the solution for KI#4.

3. Conclusions

It proposes to resolve an EN for Solution #29 by referring the solution for KI#4.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 24.811 v1.0.0.

* * * First Change * * * *

### 6.29.2	Solution description

The Disaster Condition caused RAN unavailability can be treated as a network fault. As per 3GPP TS 32.101 [5] for defining the legacy PLMN management functional architecture, the fault management is a basic management function provided by the PLMN NMS.

Furthermore, as per 3GPP TS 28.533 [6] for the SBA based NMS defined for 5GS, the network fault supervision service is also a basic management service provided by the PLMN NMS for the management of the 3GPP network.

The PLMN NMS can directly interface with different network elements within 3GPP network, including core network functions (e.g. AMF, SMF) and RAN nodes (e.g. gNB) via O&M operations. The required O&M operations may or may not be standardized, i.e. to use the proprietary implementation.

The O&M-based solution for each question within Key Issue #6 is described as below:

When the Disaster Condition is no longer applicable to the serving PLMN, the serving PLMN NMS detects the fault recovery/correction via O&M operations, removes the stored disaster PLMN ID and disaster area information and then sends an indicaiton to other Disaster Roaming PLMN NMS to inform that the Disaster Condition is no longer applicable for it. The Disaster Roaming PLMN NMS removes the stored disaster PLMN ID and disaster area information and then sends an indicaiton to its AMFs which are providing the Disaster Roaming for the Disaster Inbound Roamers. The Disaster Roaming PLMN AMF removes the stored disaster PLMN ID and disaster area information if any, and informs the Disaster Inbound Roamers currently registered to it that the Disaster Condition is no longer applicable for its previous selected PLMN for which the Disaster Condition is no longer applicable over NAS.

The Disaster Roaming PLMN AMF determines the previous selected PLMN of the Disaster Inbound roamers when the Disaster Inbound Roamers register to the Disaster Roaming PLMN, as specified in the solution(s) to Key Issue #4 (Registration to the roaming PLMN without Disaster Condition in case of Disaster Condition).

NOTE 1:	The Disaster Roaming PLMN NMS can also send the indicaiton to its RAN nodes which are providing the Disaster Roaming for the Disaster Inbound Roamers and then the RAN node informs the Disaster Inbound Roamers that the Disaster Condition is no longer applicable for its previous selected PLMN for which the Disaster Condition is no longer applicable over radio interface. However this is not considered as an alternative in this solution.

Editor's Note:	The content and format of disaster area information is FFS.

"-	How to minimize interruption of the service receiving from Disaster Roaming PLMN (e.g. emergency service or high priority service) when the UE is notified that Disaster Condition is no longer applicable;"

When the Disaster Roaming PLMN AMF was notified that the Disaster Condition is no longer applicable for Disaster Inbound Roamers, to minimize interruption of the service receiving from Disaster Roaming PLMN (e.g. emergency service or high priority service), the AMF handles as following:

(1)	When the Disaster Inbound Roamer is current in the connected mode:

In this case, when the user-plane resources was established for the Disaster Inbound Roamer (i.e. the user data services are ongoing, including emergency service or high priority service), then after the completion of the ongoing user data services and before the release of the current N1 NAS signalling connection, the AMF initiates a network-initiated de-registration procedure by sending DEREGISTRATION REQUEST message with 5GMM cause #11 (PLMN not allowed) to the UE (i.e. Disaster Inbound Roamer). After completion of the network-initiated de-registration procedure, the AMF releases the current N1 NAS signalling connection to move the UE to the idle mode.

NOTE 2:	The AMF can based on the existing mechanism to know the completion of the ongoing user data services, e.g. based on the N2 UE Context Release Request from RAN.

(2)	When the Disaster Inbound Roamer is current in the idle mode:

The AMF defers the handling to the next time when the UE moves to the connected mode. When next time received a REGISTRATION REQUEST message or SERVICE REQUEST message from the UE, the AMF rejects it with 5GMM cause #11 (PLMN not allowed).

NOTE 3:	There is another alternative for the AMF actively pages the UE to move to the connected mode and then initiate a network-initiated de-registration procedure by sending DEREGISTRATION REQUEST message with 5GMM cause #11 (PLMN not allowed) to the UE. However, this is not considered as an alternative in this solution due to it will consume a lot of radio resources considering due to paging.

"-	How to remove the stored information on Disaster Condition from the UE’s storage;"

Upon receipt of registration reject or service reject or de-registration request with 5GMM cause #11 (PLMN not allowed), if the UE currently is registered for the Disaster Roaming, then the UE knows the Disaster Condition is no longer applicable to its previous selected PLMN and then removes the stored information on Disaster Condition from the UE’s storage.

"-	How Disaster Inbound Roamer UEs perform network selection when notified that Disaster Condition is no longer applicable."

Upon receipt of registration reject or service reject or de-registration request with 5GMM cause #11 (PLMN not allowed), if the UE currently is registered for the Disaster Roaming, then the UE knows the Disaster Condition is no longer applicable to its previous selected PLMN and then performs the PLMN selection as legacy with the difference that to treat the previous selected PLMN as higher priority.

Editor's Note:	Whether the UE shall deregister and perform PLMN reselection, or only perform PLMN reselection, when a disaster condition no longer applies is FFS.

The end-to-end flow of O&M-based solution for KI#6 can be shown in Figure 6.29.2-1 for the UE in the connected mode and in Figure 6.29.2-2 for the UE in the idle mode:

Figure 6.29.2-1: End-to-end flow of O&M-based solution for KI#6, connected mode

Figure 6.29.2-2: End-to-end flow of O&M-based solution for KI#6, idle mode

* * * End of Change * * * *