3GPP TSG-CT WG1 Meeting #129-e	C1-212268

- Electronic meeting; 19-23 April 2021

Source:	vivo

Title:	TS 24.554: Overview of 5G ProSe direct communications

Spec:	TS 24.554

Agenda item:	17.2.18

Document for:	Agreement

1. Reason for Change

This paper proposes to add the overview of 5G ProSe direct communications based on the TS23.304.

2. Proposal

It is proposed to capture the following changes into TS 24.554.

* * * First Change * * * *

# 7	5G ProSe direct communications

## 7.1	Overview

- This clause describes the procedures at the UE, and between UEs, for 5G ProSe direct communication over PC5.

The UE shall support requirements for securing 5G ProSe direct communication over PC5.

For unicast mode 5G ProSe direct communication, the following data unit types are supported: IPv4, IPv6, Ethernet, and Unstructured.

For broadcast and groupcast mode 5G ProSe communication, the following data unit types are supported: IPv4, IPv6, Ethernet, Unstructured, and Address Resolution Protocol (see RFC 826 [xx]).

Editor's note:	Whether the data unit type of Address Resolution Protocol is supported needs to be coordinated with RAN WG.

5G ProSe direct communication over NR-PC5 supports broadcast mode, groupcast mode, and unicast mode. If the upper layer of the UE indicates the mode of communication, the UE shall set the mode of communication based on the request of the upper layer. Otherwise, the UE shall set the mode of communication based on the mapping rules between the 5G ProSe service identifiers and the default mode of communication defined in clause 5.2.b.

NOTE:	Further details about whether broadcast, unicast or groupcast can be used over PC5 are described in 3GPP TS 23.304 [3] clause 5.3.

* * * End of Changes * * * *