3GPP TSG-CT WG1 Meeting #129-e	C1-21xxxx was C1-212129

- Electronic meeting, 19-23 April 2021

Source:	CATT

Title:	Precedence of ProSe configuration parameters

Spec:	3GPP TS 24.554 v0.1.0

Agenda item:	17.2.18

Document for:	Agreement

1. Introduction

In SA2#144e meeting, the precedence of 5G ProSe configuration parameters is proposed. So far, the configuration parameters for 5G ProSe Direct Discovery, 5G ProSe Direct Communication ProSe usage reporting configuration and rules and ProSe service path selection are included.

2. Reason for Change

The procedence of 5G ProSe configuration paramaters needs to be specified if there are multiple source of 5G ProSe configuration parameters.

3. Conclusions

It is needed to specify the procedence of 5G ProSe configuration parameters in CT1 to align with SA2’s conclusion

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.554 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS); Stage 2".

[x]	3GPP TS 24.501: "Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

* * * Next Change * * * *

### 5.2.z	General

UE’s usage of 5G ProSe service is controlled by 5G ProSe configration parameters.

The 5G ProSe configuration parameters consist of the configuration parameters for ProSe direct discovery, ProSe direct communication, ProSe usage reporting configuration and rules and ProSe service path selection.

* * * Next Change * * * *

### 5.2.y	Precedence of 5G ProSe configuration parameters

The 5G ProSe configuration parameters for ProSe direct discovery, ProSe direct communication, ProSe usage reporting configuration and rules and ProSe service path selection can be:

a)	pre-configured in the ME;

b)	configured in the UICC;

c)	provided as a ProSeP using the UE policy delivery service as specified in 3GPP TS 24.501 [xx] annex D;

d)	provided by a ProSe application server via PC1 reference point; or

e)	a combination of case a), b), c) or d) above.

Editor’s note:	It is FFS and subject to SA2 to decide that whether or not the 5G ProSe configuration parameters for ProSe usage reporting configuration and rules and ProSe service path selection can be provided by a ProSe application server via PC1 reference point.

For ProSe group member discovery and ProSe groupcast mode direct communication, the 5G ProSe configuration parameters provided/updated by ProSe application server shall take a higher precedence than the same parameters that are provided/updated by the PCF, provisioned in the ME or configured in the UICC.

Apart from that, the UE shall use the 5G ProSe configuration parameters in the following order of decreasing precedence:

a)	the 5G ProSe configuration parameters provided as a PSP using the UE policy delivery service as specified in annex D of 3GPP TS 24.501 [x];

b)	the 5G ProSe configuration parameters provided by a ProSe application server via PC1 reference point;

c)	the 5G ProSe configuration parameters configured in the UICC; and

d)	the 5G ProSe configuration parameters pre-configured in the ME.