3GPP TSG-CT WG1 Meeting #129-e	C1-21xxxx was C1-212249

- Electronic meeting; 19-23 April 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	5G ProSe IE definitions for Direct Establishment and Direct Release procedures

Spec:	3GPP TS 24.554 V0.1.0

Agenda item:	17.2.18

Document for:	Agreement

1. Introduction

Stage-2 spec TS 23.304 has defined the Proximity based Services (ProSe) features in 5GS, such as ProSe direct discovery and ProSe direct communication.

2. Reason for Change

The definitions of the IEs used for 5G ProSe Direct Establishment and Direct Release procedures need to be specified in stage-3 spec TS 24.554.

It is worth to note that, the principles for the IE definitions for V2X communication described in TS 24.587 are reused with the following differences:

-	V2X service is replaced by ProSe Application.

-	V2X service identifier is replaced by ProSe Application identifier.

-	New additional PQI values for PC5 QoS are defined for ProSe.

-	New IP address configuration values are defined and reflected in the procedure, to support IPv4 and IPv6.

-	Some IEs are not defined yet, where they are waiting for SA2/SA3 decisions such as some security related IEs.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.554 V0.1.0.

* * * First Change * * * *

- 12.3	PC5 signalling message formats

- 12.3.1	ProSe PC5 signalling message type

The purpose of the ProSe PC5 signalling message type information element is to indicate the type of messages used in ProSe PC5 signalling protocol.

The value part of the ProSe PC5 signalling message type information element used in the ProSe PC5 signalling messages is coded as shown in table 12.3.1.1.

The ProSe PC5 signalling message type is a type 3 information element, with the length of 1 octet.

Table 12.3.1.1: ProSe PC5 signalling message type

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | PROSE DIRECT LINK ESTABLISHMENT REQUEST |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | PROSE DIRECT LINK ESTABLISHMENT ACCEPT |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  | PROSE DIRECT LINK ESTABLISHMENT REJECT |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |  | PROSE DIRECT LINK MODIFICATION REQUEST |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |  | PROSE DIRECT LINK MODIFICATION ACCEPT |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |  | PROSE DIRECT LINK MODIFICATION REJECT |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |  | PROSE DIRECT LINK RELEASE REQUEST |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |  | PROSE DIRECT LINK RELEASE ACCEPT |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |  | PROSE DIRECT LINK KEEPALIVE REQUEST |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |  | PROSE DIRECT LINK KEEPALIVE RESPONSE |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 |  | PROSE DIRECT LINK AUTHENTICATION REQUEST |
| 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |  | PROSE DIRECT LINK AUTHENTICATION RESPONSE |
| 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 |  | PROSE DIRECT LINK AUTHENTICATION REJECT |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 |  | PROSE DIRECT LINK SECURITY MODE COMMAND |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |  | PROSE DIRECT LINK SECURITY MODE COMPLETE |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |  | PROSE DIRECT LINK SECURITY MODE REJECT |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |  | PROSE DIRECT LINK REKEYING REQUEST |
| 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |  | PROSE DIRECT LINK REKEYING RESPONSE |
| 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 |  | PROSE DIRECT LINK IDENTIFIER UPDATE REQUEST |
| 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |  | PROSE DIRECT LINK IDENTIFIER UPDATE ACCEPT |
| 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |  | PROSE DIRECT LINK IDENTIFIER UPDATE ACK |
| 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 |  | PROSE DIRECT LINK IDENTIFIER UPDATE REJECT |
| 0 | 0 | 0 | 1 | 0 | 1 | 1 | 1 |  | PROSE DIRECT LINK AUTHENTICATION FAILURE |

- 12.3.2	Sequence number

The purpose of the Sequence number information element is to uniquely identify a ProSe PC5 signalling message being sent or received. The sending UE will increment the sequence number for each outgoing new ProSe PC5 signalling message.

The Sequence number information element is an integer in the 0-255 range.

The Sequence number is a type 3 information element, with a length of 1 octet.

- 12.3.3	ProSe application identifier

The purpose of the ProSe application identifier parameter is to carry the identifier of a ProSe application.

The ProSe application identifier information element is coded as shown in figure 12.3.3.1 and table 12.3.3.1.

The ProSe application identifier is a type 4 information element with a minimum length of 6 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ProSe application identifier IEI |  |  |  |  |  |  |  |  | octet 1 |
| Length of ProSe application identifier contents |  |  |  |  |  |  |  |  | octet 2 |
| ProSe application identifier 1 |  |  |  |  |  |  |  |  | octet 3 octet 6 |
| ProSe application identifier 2 |  |  |  |  |  |  |  |  | octet 7* octet 10* |
| … |  |  |  |  |  |  |  |  | octet 11* octet 4n-2* |
| ProSe application identifier n |  |  |  |  |  |  |  |  | octet 4n-1* |
|  |  |  |  |  |  |  |  |  | octet 4n+2* |

Figure 12.3.3.1: ProSe application identifier information element

Table 12.3.3.1: ProSe application identifier information element

ProSe application identifier: The ProSe application identifier field contains a binary coded ProSe application identifier. The format of ProSe application identifier is out of scope of this specification.

- 12.3.4	Application layer ID

The purpose of the Application layer ID parameter information element carries an application layer ID as specified in 3GPP TS 23.304 [2].

The Application layer ID information element is coded as shown in figure 12.3.4.1 and table 12.3.4.1.

The Application layer ID is a type 4 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Application layer ID IEI |  |  |  |  |  |  |  |  | octet 1 |
| Length of Application layer ID contents |  |  |  |  |  |  |  |  | octet 2 |
| Application layer ID contents |  |  |  |  |  |  |  |  | octet 3 |
|  |  |  |  |  |  |  |  |  | octet m |

Figure 12.3.4.1: Application layer ID information element

Table 12.3.4.1: Application layer ID information element

The length of Application layer ID contents field contains the binary coded representation of the length of the Application layer ID contents field. The Application layer ID contents field contains the octets indicating the Application layer ID. The format of the Application layer ID parameter is out of scope of this specification.

- 12.3.5	PC5 QoS flow descriptions

The purpose of the PC5 QoS flow descriptions information element is to indicate a set of PC5 QoS flow descriptions to be used by the UE over the direct link, where each PC5 QoS flow description is a set of parameters as described in clause 5.7 of 3GPP TS 23.304 [2].

The PC5 QoS flow descriptions is a type 6 information element with a minimum length of 6 octets. The maximum length for the information element is 65538 octets.

The PC5 QoS flow descriptions information element is coded as shown in figure 12.3.5.1, figure 12.3.5.2, figure 12.3.5.3, figure 12.3.5.4, and table 12.3.5.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC5 QoS flow descriptions IEI |  |  |  |  |  |  |  | octet 1 |
| Length of PC5 QoS flow descriptions contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| PC5 QoS flow description 1 |  |  |  |  |  |  |  | octet 4 octet u |
| PC5 QoS flow description 2 |  |  |  |  |  |  |  | octet u+1 octet v |
| ... |  |  |  |  |  |  |  | octet v+1 octet w |
| PC5 QoS flow description n |  |  |  |  |  |  |  | octet w+1 octet x |

Figure 12.3.5.1: PC5 QoS flow descriptions information element

| 8 | 7 |  | 6 |  | 5 |  | 4 |  | 3 | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare |  | 0 Spare | PQFI |  |  |  |  |  |  |  |  |  | octet 4 |
| Operation code |  |  |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare | 0 Spare |  | octet 5 |
| 0 Spare |  | E | Number of parameters |  |  |  |  |  |  |  |  |  | octet 6 |
| Associated ProSe application identifiers |  |  |  |  |  |  |  |  |  |  |  |  | octet 7* octet k* |
| Parameters list |  |  |  |  |  |  |  |  |  |  |  |  | octet k+1* octet u* |

Figure 12.3.5.2: PC5 QoS flow description

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter 1 |  |  |  |  |  |  |  | octet k+1 octet m |
| Parameter 2 |  |  |  |  |  |  |  | octet m+1 octet n |
| ... |  |  |  |  |  |  |  | octet n+1 octet o |
| Parameter n |  |  |  |  |  |  |  | octet o+1 octet u |

Figure 12.3.5.3: Parameters list

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter identifier |  |  |  |  |  |  |  | octet k+1 |
| Length of parameter contents |  |  |  |  |  |  |  | octet k+2 |
| Parameter contents |  |  |  |  |  |  |  | octet k+3 octet m |

Figure 12.3.5.4: Parameter

Table 12.3.4.1: PC5 QoS flow descriptions information element

PC5 QoS flow identifier (PQFI) (bits 6 to 1 of octet 4) PQFI field contains the PC5 QoS flow identifier. Bits 6 5 4 3 2 1 0 0 0 0 0 1 PQFI 1 to 1 1 1 1 1 1 PQFI 63 The UE shall not set the PQFI value to 0.

Operation code (bits 8 to 6 of octet 5) Bits 8 7 6 0 0 1 Create new PC5 QoS flow description 0 1 0 Delete existing PC5 QoS flow description 0 1 1 Modify existing PC5 QoS flow description All other values are reserved.

E bit (bit 7 of octet 6) For the "create new PC5 QoS flow description" operation, the E bit is encoded as follows: Bit 7 0 reserved 1 parameters list is included For the "Delete existing PC5 QoS flow description" operation, the E bit is encoded as follows: Bit 7 0 parameters list is not included 1 reserved For the "modify existing PC5 QoS flow description" operation, the E bit is encoded as follows: Bit 7 0 extension of previously provided parameters 1 replacement of all previously provided parameters If the E bit is set to "parameters list is not included", the number of parameters field has zero value. If the E bit is set to "parameters list is included", the number of parameters field has non-zero value. If the E bit is set to "extension of previously provided parameters" or "replacement of all previously provided parameters", the number of parameters field has non-zero value. If the E bit is set to "extension of previously provided parameters" and one of the parameters in the new parameters list already exists in the previously provided parameters, the parameter shall be set to the new value. Number of parameters (bits 6 to 1 of octet 6) The number of parameters field contains the binary coding for the number of parameters in the parameters list field. The number of parameters field is encoded in bits 6 through 1 of octet 6 where bit 6 is the most significant and bit 1 is the least significant bit. Associated ProSe application identifiers (octet 7 to k) The associated ProSe application identifiers field contains a variable number of ProSe application identifiers associated with the PC5 QoS flow. Associated ProSe application identifiers field is coded as the length and value part of ProSe application identifier information element as specified in clause 12.3.3 starting with the second octet. Parameters list (octets k+1 to u) The parameters list contains a variable number of parameters. Each parameter included in the parameters list is of variable length and consists of: - a parameter identifier (1 octet); - the length of the parameter contents (1 octet); and - the parameter contents itself (variable amount of octets). The parameter identifier field is used to identify each parameter included in the parameters list and it contains the hexadecimal coding of the parameter identifier. Bit 8 of the parameter identifier field contains the most significant bit and bit 1 contains the least significant bit. In this version of the protocol, the following parameter identifiers are specified: - 01H (PQI); - 02H (GFBR); (see NOTE) - 03H (MFBR); (see NOTE) - 04H (Averaging window) ; - 05H (Resource type); - 06H (Default priority level); - 07H (Packet delay budget); - 08H (Packet error rate); - 09H (Default maximum data burst volume). If the parameters list contains a parameter identifier that is not supported by the receiving entity the corresponding parameter shall be discarded. The length of parameter contents field contains the binary coded representation of the length of the parameter contents field. The first bit in transmission order is the most significant bit. When the parameter identifier indicates PQI, the parameter contents field contains the binary representation of PQI that is one octet in length. PQI: Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 Reserved 0 0 0 0 0 0 0 1 to Spare 0 0 0 1 0 1 0 0 0 0 0 1 0 1 0 1 PQI 21 0 0 0 1 0 1 1 0 PQI 22 0 0 0 1 0 1 1 1 PQI 23 0 0 0 1 1 0 0 0 PQI 24 0 0 0 1 1 0 0 1 PQI 25 0 0 0 1 1 0 1 0 PQI 26 0 0 0 1 1 0 1 1 to Spare 0 0 1 1 0 1 1 0 0 0 1 1 0 1 1 1 PQI 55 0 0 1 1 1 0 0 0 PQI 56 0 0 1 1 1 0 0 1 PQI 57 0 0 1 1 1 0 1 0 PQI 58 0 0 1 1 1 0 1 1 PQI 59 0 0 1 1 1 1 0 0 PQI 60 0 0 1 1 1 1 0 1 PQI 61 0 0 1 1 1 1 1 0 to Spare 0 1 0 1 1 0 0 1 0 1 0 1 1 0 1 0 PQI 90 0 1 0 1 1 0 1 1 PQI 91 0 1 0 1 1 1 0 0 PQI 92 0 1 0 1 1 1 0 1 PQI 93 0 1 0 1 1 1 1 0 to Spare 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 to Operator-specific PQIs 1 1 1 1 1 1 1 0 1 1 1 1 1 1 1 1 Reserved The UE shall consider all other values not explicitly defined in this version of the protocol as unsupported. When the parameter identifier indicates "GFBR", the parameter contents field contains one octet indicating the unit of the guaranteed flow bit rate followed by two octets containing the value of the guaranteed flow bit rate. Unit of the guaranteed flow bit rate (octet 1) Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 value is not used 0 0 0 0 0 0 0 1 value is incremented in multiples of 1 Kbps 0 0 0 0 0 0 1 0 value is incremented in multiples of 4 Kbps 0 0 0 0 0 0 1 1 value is incremented in multiples of 16 Kbps 0 0 0 0 0 1 0 0 value is incremented in multiples of 64 Kbps 0 0 0 0 0 1 0 1 value is incremented in multiples of 256 Kbps 0 0 0 0 0 1 1 0 value is incremented in multiples of 1 Mbps 0 0 0 0 0 1 1 1 value is incremented in multiples of 4 Mbps 0 0 0 0 1 0 0 0 value is incremented in multiples of 16 Mbps 0 0 0 0 1 0 0 1 value is incremented in multiples of 64 Mbps 0 0 0 0 1 0 1 0 value is incremented in multiples of 256 Mbps 0 0 0 0 1 0 1 1 value is incremented in multiples of 1 Gbps 0 0 0 0 1 1 0 0 value is incremented in multiples of 4 Gbps 0 0 0 0 1 1 0 1 value is incremented in multiples of 16 Gbps 0 0 0 0 1 1 1 0 value is incremented in multiples of 64 Gbps 0 0 0 0 1 1 1 1 value is incremented in multiples of 256 Gbps 0 0 0 1 0 0 0 0 value is incremented in multiples of 1 Tbps 0 0 0 1 0 0 0 1 value is incremented in multiples of 4 Tbps 0 0 0 1 0 0 1 0 value is incremented in multiples of 16 Tbps 0 0 0 1 0 0 1 1 value is incremented in multiples of 64 Tbps 0 0 0 1 0 1 0 0 value is incremented in multiples of 256 Tbps 0 0 0 1 0 1 0 1 value is incremented in multiples of 1 Pbps 0 0 0 1 0 1 1 0 value is incremented in multiples of 4 Pbps 0 0 0 1 0 1 1 1 value is incremented in multiples of 16 Pbps 0 0 0 1 1 0 0 0 value is incremented in multiples of 64 Pbps 0 0 0 1 1 0 0 1 value is incremented in multiples of 256 Pbps Other values shall be interpreted as multiples of 256 Pbps in this version of the protocol. Value of the guaranteed flow bit rate (octets 2 and 3) Octets 2 and 3 represent the binary coded value of the guaranteed flow bit rate in units defined by the unit of the guaranteed flow bit rate. When the parameter identifier indicates "GFBR downlink", the parameter contents field contains one octet indicating the unit of the guaranteed flow bit rate for downlink followed by two octets containing the value of the guaranteed flow bit rate for downlink. When the parameter identifier indicates "MFBR ", the parameter contents field contains the one octet indicating the unit of the maximum flow bit rate followed by two octets containing the value of maximum flow bit rate. Unit of the maximum flow bit rate (octet 1) The coding is identical to that of the unit of the guaranteed flow bit rate. Value of the maximum flow bit rate (octets 2 and 3) Octets 2 and 3 represent the binary coded value of the maximum flow bit rate in units defined by the unit of the maximum flow bit rate. When the parameter identifier indicates "averaging window", the parameter contents field contains the binary representation of the averaging window for both uplink and downlink in milliseconds and the parameter contents field is two octets in length.

When the parameter identifier indicates "resource type", the parameter contents field contains the binary representation of the resource type that is one octet in length. Resource type: Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 Reserved 0 0 0 0 0 0 0 1 Non-GBR 0 0 0 0 0 0 1 0 GBR 0 0 0 0 0 0 1 1 Delay critical GBR 0 0 0 0 0 1 0 0 to Spare 1 1 1 1 1 1 1 1 When the parameter identifier indicates "default priority level", the parameter contents field contains the binary representation of the default priority level that is one octet in length. Default priority level: Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 Reserved 0 0 0 0 0 0 0 1 1 0 0 0 0 0 0 1 0 2 0 0 0 0 0 0 1 1 3 0 0 0 0 0 1 0 0 4 0 0 0 0 0 1 0 1 5 0 0 0 0 0 1 1 0 6 0 0 0 0 0 1 1 1 7 0 0 0 0 1 0 0 0 8 0 0 0 0 1 0 0 1 to Spare 1 1 1 1 1 1 1 1 When the parameter identifier indicates "packet delay budget", the parameter contents field contains the binary representation of the packet delay budget for both uplink and downlink in milliseconds and the parameter contents field is two octets in length. When the parameter identifier indicates "packet error rate", the parameter contents field contains the binary representation of the power of 10-1 for both uplink and downlink and the parameter contents field is one octet in length. When the parameter identifier indicates "default maximum data burst volume", the parameter contents field contains the binary representation of the default maximum data burst volume for both uplink and downlink in bytes and the parameter contents field is two octets in length.

NOTE: The GFBR and MFBR apply to both directions of the PC5 unicast link.

- 12.3.6	IP address configuration

The purpose of the IP address configuration information element is to indicate the configuration options for IP address used by the UE over this direct link.

The IP address configuration is a type 3 information element with the length of 2 octets.

The IP address configuration information element is coded as shown in figure 12.3.6.1 and table 12.3.6.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IP address configuration IEI |  |  |  |  |  |  |  | octet 1 |
| IP address configuration content |  |  |  |  |  |  |  | octet 2 |

Figure 12.3.6.1: IP address configuration information element

Table 12.3.6.1: IP address configuration information element

| IP address configuration value (octet 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |
| 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 1 |  | IPv6 Router |
| 0 | 0 | 1 | 0 |  | address allocation not supported |
| 0 | 0 | 1 | 1 |  | DHCPv4 server |
| 0 | 1 | 0 | 0 |  | DHCPv4 server & IPv6 Router |
| All other values are reserved. |  |  |  |  |  |
| Bit 5 to 8 of octet 2 are spare and shall be coded as zero. |  |  |  |  |  |

- 12.3.7	Link local IPv6 address

The purpose of the Link local IPv6 address information element is to indicate the link local IPv6 address.

The Link local IPv6 address is a type 3 information element with the length of 17 octets.

The Link local IPv6 address information element is coded as shown in figure 12.3.7.1 and table 12.3.7.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Link local IPv6 address IEI |  |  |  |  |  |  |  | octet 1 |
| Link local IPv6 address content |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 17 |

Figure 12.3.7.1: Link local IPv6 address information element

Table 12.3.7.1: Link local IPv6 address information element

Link local IPv6 address value (octet 2 to 17) This contains the 128-bit IPv6 address. This IPv6 address is encoded as a 128-bit address according to IETF RFC 4291 [15].

- 12.3.9	PC5 signalling protocol cause

The purpose of the PC5 signalling protocol cause information element is to indicate the cause used in the PC5 signalling protocol procedures.

The PC5 signalling protocol cause is a type 3 information element with a length of 2 octets.

The PC5 signalling protocol cause information element is coded as shown in figure 12.3.9.1 and table 12.3.9.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC5 signalling protocol cause IEI |  |  |  |  |  |  |  | octet 1 |
| PC5 signalling cause value |  |  |  |  |  |  |  | octet 2 |

Figure 12.3.9.1: PC5 signalling protocol cause information element

Table 12.3.9.1: PC5 signalling protocol cause information element

| PC5 signalling cause value (octet 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 |  | 2 |  | 1 |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 0 |  | 0 |  | 0 |  | 1 |  |  |  | Direct communication to the target UE not allowed |
| 0 | 0 | 0 | 0 | 0 | 0 |  | 1 |  | 0 |  |  |  | Direct communication to the target UE no longer needed |  |
| 0 | 0 | 0 | 0 | 0 |  | 0 |  | 1 |  | 1 |  |  |  | Conflict of layer-2 ID for unicast communication is detected |
| 0 | 0 | 0 | 0 | 0 | 1 |  | 0 |  | 0 |  |  |  | Direct connection is not available anymore |  |
| 0 | 0 | 0 | 0 | 0 |  | 1 |  | 0 |  | 1 |  |  |  | Lack of resources for PC5 unicast link |
| 0 | 0 | 0 | 0 | 0 |  | 1 |  | 1 |  | 0 |  |  |  | Authentication failure |
| 0 | 0 | 0 | 0 | 0 |  | 1 |  | 1 |  | 1 |  |  |  | Integrity failure |
| 0 | 0 | 0 | 0 | 1 |  | 0 |  | 0 |  | 0 |  |  |  | UE security capabilities mismatch |
| 0 | 0 | 0 | 0 | 1 |  | 0 |  | 0 |  | 1 |  |  |  | LSB of KNRP-sess ID conflict |
| 0 | 0 | 0 | 0 | 1 |  | 0 |  | 1 |  | 0 |  |  |  | UE PC5 unicast signalling security policy mismatch |
| 0 | 0 | 0 | 0 | 1 |  | 0 |  | 1 |  | 1 |  |  |  | Required service not allowed |
| 0 | 0 | 0 | 0 | 1 |  | 1 |  | 0 |  | 0 |  |  |  | Security policy not aligned |
| 0 | 1 | 1 | 0 | 1 |  | 1 |  | 1 |  | 1 |  |  |  | Protocol error, unspecified |
| Any other value received by the UE shall be treated as 0110 1111, "protocol error, unspecified". |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- 12.3.12	Key establishment information container

The Key establishment information container information element contains information for PC5 unicast link key establishment.

The Key establishment information container is a type 6 information element with a minimum length of 4 octets.

The Key establishment information container information element is coded as shown in figure 12.3.12.1 and table 12.3.12.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Key establishment information container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of key establishment information container contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Key establishment information container contents |  |  |  |  |  |  |  | octet 4 |
|  |  |  |  |  |  |  |  | octet n |

Figure 12.3.a.1: Key establishment information container information element

Table 12.3.a.1: Key establishment information container information element

Key establishment information container contents (octet 4 to n) This field contains the key establishment information container.

- 12.3.13	Nonce

The Nonce information element contains a 128-bit nonce used during PC5 unicast link security establishment.

The Nonce information element is a type 3 information element, with a length of 17 octets.

The Nonce information element is coded as shown in figure 12.3.13.1 and table 12.3.13.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nonce IEI |  |  |  |  |  |  |  | octet 1 |
| Nonce contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 17 |

Figure 12.3.13.1: Nonce information element

Table 12.3.13.1: Nonce information element

Nonce contents (octet 2 to 17) This field contains the 128-bit nonce value.

- 12.3.14	UE security capabilities

The UE security capabilities information element is used to indicate which security algorithms are supported by the UE.

The UE security capabilities is a type 4 information element with a minimum length of 4 octets and a maximum length of 10 octets.

The UE security capabilities information element is coded as shown in figure 12.3.14.1 and table 12.3.14.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE security capabilities IEI |  |  |  |  |  |  |  | octet 1 |
| Length of UE security capabilities contents |  |  |  |  |  |  |  | octet 2 |
| 5G-EA0 | 128- 5G-EA1 | 128- 5G-EA2 | 128- 5G-EA3 | 5G-EA4 | 5G-EA5 | 5G-EA6 | 5G-EA7 | octet 3 |
| 5G-IA0 | 128- 5G-IA1 | 128- 5G-IA2 | 128- 5G-IA3 | 5G-IA4 | 5G-IA5 | 5G-IA6 | 5G-IA7 | octet 4 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| Spare |  |  |  |  |  |  |  | octet 5* -10* |

Figure 12.3.14.1: UE security capabilities information element

Table 12.3.14.1: UE security capabilities information element

| 5GS encryption algorithms supported (octet 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 5GS encryption algorithm 5G-EA0 supported (octet 3, bit 8) |  |  |  |  |
| 0 |  |  |  | 5GS encryption algorithm 5G-EA0 not supported |
| 1 |  |  |  | 5GS encryption algorithm 5G-EA0 supported |
| 5GS encryption algorithm 128-5G-EA1 supported (octet 3, bit 7) |  |  |  |  |
| 0 |  |  |  | 5GS encryption algorithm 128-5G-EA1 not supported |
| 1 |  |  |  | 5GS encryption algorithm 128-5G-EA1 supported |
| 5GS encryption algorithm 128-5G-EA2 supported (octet 3, bit 6) |  |  |  |  |
| 0 |  |  |  | 5GS encryption algorithm 128-5G-EA2 not supported |
| 1 |  |  |  | 5GS encryption algorithm 128-5G-EA2 supported |
| 5GS encryption algorithm 128-5G-EA3 supported (octet 3, bit 5) |  |  |  |  |
| 0 |  |  |  | 5GS encryption algorithm 128-5G-EA3 not supported |
| 1 |  |  |  | 5GS encryption algorithm 128-5G-EA3 supported |
| 5GS encryption algorithm 5G-EA4 supported (octet 3, bit 4) |  |  |  |  |
| 0 |  |  |  | 5GS encryption algorithm 5G-EA4 not supported |
| 1 |  |  |  | 5GS encryption algorithm 5G-EA4 supported |
| 5GS encryption algorithm 5G-EA5 supported (octet 3, bit 3) |  |  |  |  |
| 0 |  |  |  | 5GS encryption algorithm 5G-EA5 not supported |
| 1 |  |  |  | 5GS encryption algorithm 5G-EA5 supported |
| 5GS encryption algorithm 5G-EA6 supported (octet 3, bit 2) |  |  |  |  |
| 0 |  |  |  | 5GS encryption algorithm 5G-EA6 not supported |
| 1 |  |  |  | 5GS encryption algorithm 5G-EA6 supported |
| 5GS encryption algorithm 5G-EA7 supported (octet 3, bit 1) |  |  |  |  |
| 0 |  |  |  | 5GS encryption algorithm 5G-EA7 not supported |
| 1 |  |  |  | 5GS encryption algorithm 5G-EA7 supported |
| 5GS integrity algorithms supported (octet 4) |  |  |  |  |
| 5GS integrity algorithm 5G-IA0 supported (octet 4, bit 8) |  |  |  |  |
| 0 |  |  |  | 5GS integrity algorithm 5G-IA0 not supported |
| 1 |  |  |  | 5GS integrity algorithm 5G-IA0 supported |
| 5GS integrity algorithm 128-5G-IA1 supported (octet 4, bit 7) |  |  |  |  |
| 0 |  |  |  | 5GS integrity algorithm 128-5G-IA1 not supported |
| 1 |  |  |  | 5GS integrity algorithm 128-5G-IA1 supported |
| 5GS integrity algorithm 128-5G-IA2 supported (octet 4, bit 6) |  |  |  |  |
| 0 |  |  |  | 5GS integrity algorithm 128-5G-IA2 not supported |
| 1 |  |  |  | 5GS integrity algorithm 128-5G-IA2 supported |
| 5GS integrity algorithm 128-5G-IA3 supported (octet 4, bit 5) |  |  |  |  |
| 0 |  |  |  | 5GS integrity algorithm 128-5G-IA3 not supported |
| 1 |  |  |  | 5GS integrity algorithm 128-5G-IA3 supported |
| 5GS integrity algorithm 5G-IA4 supported (octet 4, bit 4) |  |  |  |  |
| 0 |  |  |  | 5GS integrity algorithm 5G-IA4 not supported |
| 1 |  |  |  | 5GS integrity algorithm 5G-IA4 supported |
| 5GS integrity algorithm 5G-IA5 supported (octet 4, bit 3) |  |  |  |  |
| 0 |  |  |  | 5GS integrity algorithm 5G-IA5 not supported |
| 1 |  |  |  | 5GS integrity algorithm 5G-IA5 supported |
| 5GS integrity algorithm 5G-IA6supported (octet 4, bit 2) |  |  |  |  |
| 0 |  |  |  | 5GS integrity algorithm 5G-IA6 not supported |
| 1 |  |  |  | 5GS integrity algorithm 5G-IA6 supported |
| 5GS integrity algorithm 5G-IA7 supported (octet 4, bit 1) |  |  |  |  |
| 0 |  |  |  | 5GS integrity algorithm 5G-IA7 not supported |
| 1 |  |  |  | 5GS integrity algorithm 5G-IA7 supported |

- 12.3.15	UE PC5 unicast signalling security policy

The purpose of the UE PC5 unicast signalling security policy information element is to indicate the UE’s configuration for integrity protection and ciphering of PC5 signalling messages.

The UE PC5 unicast signalling security policy is a type 3 information element with a length of 2 octets.

The UE PC5 unicast signalling security policy information element is coded as shown in figure 12.3.15.1.1 and table 12.3.15.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE PC5 unicast signalling security policy IEI |  |  |  |  |  |  |  | octet 1 |
| 0 spare | Signalling ciphering policy |  |  | 0 spare | Signalling integrity protection policy |  |  | octet 2 |

Figure 12.3.15.1: UE PC5 unicast signalling security policy information element

Table 12.3.15.1: UE PC5 unicast signalling security policy information element

| Signalling integrity protection policy (octet 2, bit 1 to 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 3 | 2 | 1 |  |  |
| 0 | 0 | 0 |  | Signalling integrity protection not needed |
| 0 | 0 | 1 |  | Signalling integrity protection preferred |
| 0 | 1 | 0 |  | Signalling integrity protection required |
| 0 | 1 | 1 |  |  |
| to Spare |  |  |  |  |
| 1 | 1 | 0 |  |  |
| 1 | 1 | 1 |  | Reserved |
| If the UE receives a signalling integrity protection policy value that the UE does not understand, the UE shall interpret the value as 010 "Signalling integrity protection required". Signaling ciphering policy (octet 2, bit 5 to 7) |  |  |  |  |
| Bits |  |  |  |  |
| 7 | 6 | 5 |  |  |
| 0 | 0 | 0 |  | Signalling ciphering not needed |
| 0 | 0 | 1 |  | Signalling ciphering preferred |
| 0 | 1 | 0 |  | Signalling ciphering required |
| 0 | 1 | 1 |  |  |
| to Spare |  |  |  |  |
| 1 | 1 | 0 |  |  |
| 1 | 1 | 1 |  | Reserved |
| If the UE receives a signalling ciphering policy value that the UE does not understand, the UE shall interpret the value as 010 "Signalling ciphering required". Bit 4 and 8 of octet 2 are spare and shall be coded as zero. |  |  |  |  |

- 12.3.16	MSB of KNRP-sess ID

The purpose of the MSB of KNRP-sess ID information element is to carry the 8 most significant bits of the KNRP-sess ID.

The MSB of KNRP-sess ID information element is a type 3 information element with a length of 2 octets.

The MSB of KNRP-sess ID information element is coded as shown in figure 12.3.16.1 and table 12.3.16.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MSB of KNRP-sess ID IEI |  |  |  |  |  |  |  | octet 1 |
| MSB of KNRP-sess ID contents |  |  |  |  |  |  |  | octet 2 |

Figure 12.3.16.1: MSB of KNRP-sess ID information element

Table 12.3.16.1: MSB of KNRP-sess ID information element

MSB of KNRP-sess ID contents (octet 2) This field contains the 8 most significant bits of KNRP-sess ID.

- 12.3.17	KNRP ID

The purpose of the KNRP ID information element is to carry the identity of the KNRP held by a UE.

The KNRP ID is a type 3 information element with a length of 5 octets.

The KNRP ID information element is coded as shown in figure 12.3.17.1 and table 12.3.17.1

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| KNRP ID IEI |  |  |  |  |  |  |  | octet 1 |
| KNRP ID contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 5 |

Figure 12.3.17.1: KNRP ID information element

Table 12.3.17.1: KNRP ID information element

KNRP ID contents (octet 2 to 5) This field contains the 32-bit identifier of a KNRP.

- 12.3.19	LSB of KNRP-sess ID

The purpose of the LSB of KNRP-sess ID information element is to carry the 8 least significant bits of the KNRP-sess ID.

The LSB of KNRP-sess ID is a type 3 information element with a length of 2 octets.

The LSB of KNRP-sess ID information element is coded as shown in figure 8.4.19.1 and table 8.4.19.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LSB of KNRP-sess ID |  |  |  |  |  |  |  | octet 1 |
| LSB of KNRP-sess ID contents |  |  |  |  |  |  |  | octet 2 |

Figure 8.4.19.1: LSB of KNRP-sess ID information element

Table 8.4.19.1: LSB of KNRP-sess ID information element

LSB of KNRP-sess ID contents (octet 2) This field contains the 8 least significant bits of KNRP-sess ID.

- 12.3.20	MSBs of KNRP ID

The purpose of the MSBs of KNRP ID information element is to carry the 16 most significant bits of the KNRP ID.

The MSBs of KNRP ID is a type 3 information element with a length of 3 octets.

The MSBs of KNRP ID information element is coded as shown in figure 8.4.20.1 and table 8.4.20.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MSBs of KNRP ID IEI |  |  |  |  |  |  |  | octet 1 |
| MSBs of KNRP ID contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |

Figure 8.4.20.1: MSBs of KNRP ID information element

Table 8.4.20.1: MSBs of KNRP ID information element

MSBs of KNRP ID contents (octet 2 to 3) This field contains the 16 most significant bits of KNRP ID.

- 12.3.21	LSBs of KNRP ID

The purpose of the LSBs of KNRP ID information element is to carry the 16 least significant bits of the KNRP ID.

The LSBs of KNRP ID is a type 3 information element with a length of 3 octets.

The LSBs of KNRP ID information element is coded as shown in figure 8.4.21.1 and table 8.4.21.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LSBs of KNRP ID IEI |  |  |  |  |  |  |  | octet 1 |
| LSBs of KNRP ID contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |

Figure 8.4.21.1: LSBs of KNRP ID information element

Table 8.4.21.1: LSBs of KNRP ID information element

LSBs of KNRP ID contents (octet 2 to 3) This field contains the 16 least significant bits of KNRP ID.

- 12.3.23	Configuration of UE PC5 unicast user plane security protection

The purpose of the configuration of UE PC5 unicast user plane security protection information element is to indicate the agreed configuration for security protection of PC5 user plane data between UEs over the PC5 unicast link.

The configuration of UE PC5 unicast user plane security protection is a type 3 information element with a length of 2 octets.

The configuration of UE PC5 unicast user plane security protection information element is coded as shown in figure 12.3.23.1.1 and table 12.3.23.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| configuration of UE PC5 unicast user plane security protection IEI |  |  |  |  |  |  |  | octet 1 |
| 0 spare | User plane ciphering configuration |  |  | 0 spare | User plane integrity protection configuration |  |  | octet 2 |

Figure 12.3.23.1: Configuration of UE PC5 unicast user plane security protection information element

Table 12.3.23.1: Configuration of UE PC5 unicast user plane security protection information element

| User plane integrity protection configuration (octet 2, bit 1 to 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 3 | 2 | 1 |  |  |
| 0 | 0 | 0 |  | Off |
| 0 | 0 | 1 |  | Off or On |
| 0 | 1 | 0 |  | On |
| 0 | 1 | 1 |  |  |
| to Spare |  |  |  |  |
| 1 | 1 | 0 |  |  |
| 1 | 1 | 1 |  | Reserved |
| User plane ciphering configuration (octet 2, bit 5 to 7) |  |  |  |  |
| Bits |  |  |  |  |
| 7 | 6 | 5 |  |  |
| 0 | 0 | 0 |  | Off |
| 0 | 0 | 1 |  | Off or On |
| 0 | 1 | 0 |  | On |
| 0 | 1 | 1 |  |  |
| to Spare |  |  |  |  |
| 1 | 1 | 0 |  |  |
| 1 | 1 | 1 |  | Reserved |
| Bit 4 and 8 of octet 2 are spare and shall be coded as zero. |  |  |  |  |