3GPP TSG-CT WG1 Meeting #129-e	C1-212104

Electronic meeting; 19-23 April 2021

Source:	Huawei, HiSilicon

Title:	EN resolution of misuse of registration type in Solution #19 KI #4

Spec:	3GPP TR 24.811

Agenda item:	17.2.9

Document for:	Agreement

1. Introduction

This pCR attempts to resolve the following editors note for the solution #19 for KI #4

Editor's note: It is FFS on how to make sure that an attacker UE does not misuse the new registration type request to get access to a PLMN where it is not allowed to access.

AMF of the VPLMN authenticates the UE by contacting the AUSF of the HPLMN. AUSF of the HPLMN can check the credentials to see if the UE is an attacker UE or not before accepting the authentication. So the editors note can be removed.

2. Reason for Change

Editors note need to be resolved.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 24.811.

* * * First Change * * * *

### 6.19.2	Solution description

To solve the problem the UE needs to differentiate the registration request due to disaster roaming and a normal registration request. So The UE needs to use a new registration request type in the registration request message. This will help the NW identify that the UE is performing a registration followed by disaster condition. The NW may accept the registration request from such a UE if the network supports disaster roaming in that area from the previously served PLMN of the UE. .

Editor's note: It is FFS on how AMF uses this information towards AUSF for the authentication of the UE.

- The registration type is used by the UE only if it has detected a disaster condition and needs to move away from the serving PLMN.

NOTE:	PLMN that supports disaster roaming can accept the registration request from the UE if the UE indicates that it is performing registration request due to a disaster condition and if the PLMN supports disaster roaming from the previously served PLMN of that UE in the disaster area.In order to accept the registration request, the visiting PLMN performs primary authentication towards the AUSF of the HPLMN with an indication that the registration is due to disaster roaming. AUSF of the HPLMN makes sure that the attacker UE is not authenticated with the information of the Disaster Roaming service stored in UDM/UDR, and the UE may also be pre-configured with the information of the Disaster Roaming service.

- Visiting NW shall assign the service area to UE and define the allowed area where the UE is allowed to have service due to disaster condition.

Editor's note: It is FFS on how the PLMN assign service area based on the disaster area.

- Visiting NW can also assign the tracking areal list (TAI list) based on the area where the disaster has happened. This also has the advantage that the UE will perform a mobility and registration update procedure as soon as UE moves out of the area.

* * * End of Change * * * *