3GPP TSG-CT WG1 Meeting #129-e	C1-212xyz

- Electronic meeting; 19-23 April 2021	(was C1-212324)

Source:	Huawei, HiSilicon, CATT, China Telecom, China Unicom

Title:	Service provisioning procedure based on request-response model

Spec:	3GPP TS 24.558

Agenda item:	17.2.16

Document for:	Agreement

1. Introduction

3GPP TS 24.558 contains under the Annex B a clause on Service provisioning procedure based on Request-Response model.

2. Reason for Change

The service provisioning procedure needs to be defined.

3GPP TS 23.558 clauses 8.3.2 and 8.3.3 provide the requirements for service provisioning between the UE (EEC) and the ECS. Based on 3GPP TS 23.558, the UE includes the service provisioning request (i.e., as defined by stage 2 3GPP TS 23.588 Table 8.3.3.1-1). A service provisioning response is received in the UE (i.e., as defined by stage 2 3GPP TS 23.588 Table 8.3.3.3.3.-1). Hence, two messages need to be defined for the service privisoning procedure.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.558 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[r24501]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Application Client

API	Application Programming Interface

APN	Access Point Name

EAS	Edge Application Server

ECS	Edge Configuration Server

EDN	Edge Data Network

EEC	Edge Enabler Client

EES	Edge Enabler Server

DNN	Data Network Name

NAS	Non Access Stratum

S-NSSAI	Single NSSAI

UE	User Equipment

* * * Next Change * * * *

#### B.2.2.2	Procedures

##### B.2.2.2.1	Service provisioning procedure based on request-response model

B.2.2.2.1.1	General

The purpose of request-response model of service provisioning procedure is to enable the EEC to obtain the list of EDN configuration information based on the hosting UE's location, service requirements, service preferences and connectivity.

###### B.2.2.2.1.2	Service provisioning request-response procedure

In order to initiate the service provisioning request, the EEC shall:

a)	create a SERVICE PROVISIONING REQUEST message as specified in clause B.2.4.1;

b)	send the SERVICE PROVISIONING REQUEST message to the ECS via the SMF and the PCF; and

c)	start timer Taaa (see example in figure B.2.2.2.1).

Figure B.2.2.2.1: Service provisioning request-response procedure

Upon receipt of the SERVICE PROVISIONING REQUEST message, the ECS shall:

a)	create a SERVICE PROVISIONING RESPONSE message as specified in clause B.2.4.2;

b)	send the SERVICE PROVISIONING RESPONSE message to the EEC via the PCF and the SMF.

Upon receipt of the SERVICE PROVISIONING RESPONSE message, the EEC shall stop timer Taaa.

* * * Next Change * * * *

#### B.2.4.1	Service provisioning request

B.2.4.1.1	Message definition

The SERVICE PROVISIONING REQUEST message is sent by the EEC to the ECS to request the EDN configuration information from the ECS, see table B.2.4.1.1.

Message type:	SERVICE PROVISIONING REQUEST

Significance:	dual

Direction:		EEC to ECS

Table B.2.4.1.1: SERVICE PROVISIONING REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | SERVICE PROVISIONING REQUEST message identity | Service provisioning service message type B.2.5.1 | M | V | 1 |
|  | EECID | TBD | M | LV-E |  |
|  | Security credentials | TBD | M | LV-E |  |
| TBD | AC Profile(s) | TBD | O | TLV-E |  |
| TBD | UE Identifier | TBD | O | V |  |
| TBD | Connectivity information | TBD | O | TLV-E |  |
| TBD | UE location | TBD | O | TLV-E |  |

Editor's note:	It is FFS to determine the format and length of the IEs above.

* * * Next Change * * * *

#### B.2.4.2	Service provisioning response

B.2.4.2.1	Message definition

The SERVICE PROVISIONING RESPONSE message is sent by the ECS to the EEC to provision the EDN configuration information to the EEC, see table B.2.4.2.1.

Message type:	SERVICE PROVISIONING RESPONSE

Significance:	dual

Direction:		ECS to EEC

Table B.2.4.2.1: SERVICE PROVISIONING RESPONSE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | SERVICE PROVISIONING RESPONSE message identity | Service provisioning service message type B.2.5.1 | M | V | 1 |
|  | DNN/APN | As specified in 3GPP TS 24.501 [r24501] clause 9.11.2.1 | M | LV-E | 2-102 |
| TBD | S-NSSAI | As specified in 3GPP TS 24.501 [r24501] clause 9.11.2.8 | O | TLV-E | 2-10 |
| TBD | EDN topological service area | TBD | O | TLV-E |  |
| TBD | EESID | TBD | M | LV-E |  |
| TBD | EES endpoint | TBD | M | LV-E |  |
| TBD | EASIDs | TBD | O | TLV-E |  |
| TBD | ECSP info | TBD | O | TLV-E |  |
| TBD | EES topological service area | TBD | O | TLV-E |  |
| TBD | EES geographical service area | TBD | O | TLV-E |  |
| TBD | List of EES DNAI(s) | TBD | O | TLV-E |  |
| TBD | Lifetime | TBD | O | TLV-E |  |

Editor's note:	It is FFS to determine the format and length of the IE above.

* * * Next Change * * * *

#### B.2.5.1	Service provisioning service message type

Table B.2.5.1: Service provisioning service message type

| Bits |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  | Reserved |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  |  | SERVICE PROVISIONING REQUEST message |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  |  | SERVICE PROVISIONING RESPONSE message |
| All other values are reserved |  |  |  |  |  |  |  |  |  |  |

* * * Next Change * * * *

### B.2.6	Timers

Timers of service provisioning service are shown in table B.2.6.1.

Table B.2.6.1: Timers of Service provisioning service – EEC side

| TIMER NUM. | TIMER VALUE | CAUSE OF START | NORMAL STOP | ON THE 1st, 2nd, 3rd, 4th EXPIRY |
| --- | --- | --- | --- | --- |
| Taaa | NOTE | Transmission of SERVICE PROVISIONING REQUEST message | SERVICE PROVISIONING RESPONSE message received | Retransmission of SERVICE PROVISIONING REQUEST message |
| NOTE: The value of this timer is EEC dependent. |  |  |  |  |