3GPP TSG-CT WG1 Meeting #129-e	C1-2123xyz

- Electronic meeting; 19-23 April 2021	(was C1-212377)

Source:	Huawei, HiSilicon

Title:	EEC registration procedure

Spec:	3GPP TS 24.558

Agenda item:	17.2.16

Document for:	Agreement

1. Introduction

3GPP TS 24.558 contains a clause on Edge Enabler Server API Definitions.

2. Reason for Change

3GPP TS 23.558 clause 8.4.2 provides the requirements for the EEC registration procedure. The ECC performs registration with an EES in order to use Edge services offered by the EES, e.g. discovering EASs in an area of interest. Clause 8.4.2.4 of 3GPP TS 23.558 provides the detail requirements for defining the necessary API.

A number of editor’s notes are proposed as further work is needed. For example, the PATCH HTTP method could also be considerred as partial update of the registration information.

The Appi version is needeed for Resource URI and it has to be set to v1 in this version of the specification.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.558 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[r29122]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[r29558]	3GPP TS 29.558: "Enabling Edge Applications; Application Programming Interface (API) specification; Stage 3".

[r29571]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[r29572]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Application Client

API	Application Programming Interface

EAS	Edge Application Server

ECS	Edge Configuration Server

EEC	Edge Enabler Client

EES	Edge Enabler Server

NAS	Non Access Stratum

UE	User Equipment

* * * Next Change * * * *

## 6.1	Information applicable to several EES APIs

The EES APIs as specified in clause 6.x allow secure access to the capabilities provided by the EES functional entity.

The stage-2 level requirements and signalling flows are defined in 3GPP TS 23.558 [r23558].

The usage of HTTP, content type, URI structure definition, notifications, error handling, feature negotiation, HTTP headers and Conventions for Open API specification files, as specified in subclauses 7.3, 7.4, 7.5, 7.6, 7.7, 7.8, 7.9 and 7.10 of 3GPP TS 29.558 [r29558] respectively, shall be applicable for the APIs in current specification.

* * * Next Change * * * *

## 6.x	Eees_EECRegistration API

### 6.x.1	API URI

The request URI used in each HTTP request from the EEC towards the EES shall have the structure as defined in clause 6.1.y with the following clarifications:

-	The <apiName> shall be "eees-eecregistration".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.x.2.

### 6.x.2	Resources

#### 6.x.2.1	Overview

Figure 6.x.2.1-1: Resource URI structure of the Eees_EECRegistration API

Table 6.x.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.x.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| EEC Registrations | /registrations | POST | Create a new EEC registration at the EES |
| Individual EEC registration | /registrations/{registrationId} | PUT | Update an existing EEC registration a the EES |
|  |  | DELETE | Remove an existing ECC registration at EES |

Editor’s note:	Whether the EEC Id should be included into the resource URI structure is FFS.

Editor’s note:	Whether PATCH method will be used to partially update the EEC registration information is FFS.

#### 6.x.2.2	Resource: EEC Registrations

##### 6.x.2.2.1	Description

This resource represents a collection of EEC registrations with an EES.

##### 6.x.2.2.2	Resource Definition

Resource URI: {apiRoot}/eees-eecregistration/<apiVersion>/registrations

This resource shall support the resource URI variables defined in table 6.x.2.2.2-1.

Table 6.x.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1 |
| apiVersion | string | See clause 6.1 |

##### 6.x.2.2.3	Resource Standard Methods

###### 6.x.2.2.3.1	POST

This method creates a new registration. This method shall support the URI query parameters specified in table 6.x.2.2.3.1-1.

Table 6.x.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.x.2.2.3.1-2 and the response data structures and response codes specified in table 6.x.2.2.3.1-3

Table 6.x.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EECRegistration | M | 1 | EEC registration request information |

Table 6.x.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EECRegistration | M | 1 | 201 Created | EEC information is registered successfully at EES. EEC information registered with EES is provided in the response body. The URI of the created resource shall be returned in the "Location" HTTP header |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [r29122] also apply. |  |  |  |  |

Table 6.x.2.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 6.x.2.2.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/eees-eecregistration/<apiVersion>/registrations/{registrationId} |

##### 6.x.2.2.4	Resource Custom Operations

None.

#### 6.x.2.3	Resource: Individual EEC registration

##### 6.x.2.3.1	Description

This resource represents an individual registration of an EEC.

##### 6.x.2.3.2	Resource Definition

Resource URI: {apiRoot}/eees-eecregistration//<apiVersion>/registrations/{registrationId}

This resource shall support the resource URI variables defined in table 6.x.2.3.2-1.

Table 6.x.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.y |
| apiVersion | string | See clause 6.1.z |
| registrationId | string | The Identifier of a specific EEC registration. |

##### 6.x.2.3.3	Resource Standard Methods

###### 6.x.2.3.3.1	PUT

This method updates the EEC registration data by completely replacing the existing registration data. This method shall support the URI query parameters specified in table 6.x.2.3.3.1-1.

Table 6.x.2.3.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.x.2.3.3.1-2 and the response data structures and response codes specified in table 6.x.2.3.3.1-3.

Table 6.x.2.3.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EecRegistration | M | 1 | An Individual registration resource to be updated. |

Table 6.x.2.3.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EecRegistration | M | 1 | 200 OK | An individual EEC registration resource updated successfully. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [r29122] also apply. |  |  |  |  |

###### 6.x.2.3.3.2	DELETE

This method deregisters (removes) an existing EEC registration. This method shall support the URI query parameters specified in table 6.x.2.3.3.2-1.

Table 6.x.2.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.x.2.3.3.2-2 and the response data structures and response codes specified in table 6.x.2.3.3.2-3.

Table 6.x.2.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.x.2.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | An individual EEC registration resource deleted successfully. |
| NOTE: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [r29122] also apply. |  |  |  |  |

Table 6.x.2.3.3.3-4: Headers supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 6.x.2.3.3.3-5: Headers supported by the 204 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 6.x.2.3.3.3-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 6.x.2.3.4	Resource Custom Operations

None.

### 6.x.3	Custom Operations without associated resources

None.

### 6.x.4	Notifications

None.

### 6.x.5	Data Model

#### 6.x.5.1	General

This clause specifies the application data model supported by the Eees_EECRegistration API.

Table 6.x.5.1-1 specifies the data types defined specifically for the Eees_EECRegistration API service.

Table 6.x.5.1-1: Eees_EECRegistration API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| EecRegistration | 6.x.5.2.2 | Describes the parameters to perform EEC Registration related operations |  |
| ACProfile | 6.x.5.2.3 | Describes information about AC used to determine services and service characteristics required |  |
| EasDetail | 6.x.5.2.4 | Describes EAS along with service KPIs that serves the AC |  |
| ACServiceKPIs | 6.x.5.2.5 | Describes the KPIs required by the AC in order to receive required services |  |

Table 6.x.5.1-2 specifies data types re-used by the Eees_EECRegistration API service.

Table 6.x.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| BitRate | 3GPP TS 29.571 [r29571] |  |  |
| DateTime | 3GPP TS 29.122 [r29122] |  |  |
| EndPoint | 3GPP TS 29.558 [r29558] |  |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [r29122] |  |  |
| LocationArea5G | 3GPP TS 29.122 [r29122] |  |  |
| Uinteger | 3GPP TS 29.571 [r29571] |  |  |
| DurationSec | 3GPP TS 29.122 [r29122] |  |  |
| SupportedFeatures | 3GPP TS 29.571 [r29571] |  |  |

#### 6.x.5.2	Structured data types

##### 6.x.5.2.1	Introduction

##### 6.x.5.2.2	Type: EecRegistration

Table 6.x.5.2.2-1: Definition of type EecRegistration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eecId | string | M | 0..1 | Represents a unique identifier of the EEC. |  |
| ueId | string | O | 0..1 | Represents the identifier of the UE. |  |
| acProfs | array(ACProfile) | O | 1..N | Profiles of ACs for which the EEC provides edge enabling services. |  |
| expTime | DateTime | O | 0..1 | Represents an expiration time for the registration. This attribute shall be present in the response of the HTTP POST message from EEC to create a new registration or in the response of the HTTP PUT message from EEC to update a specific registration. If abesent, then it indicates that the registration of EEC never expires. |  |
| eecCntxId | string | O | 0..1 | Identifier of the EEC context obtained from a previous registration. |  |
| srcEesId | string | O | 0..1 | Identifier of the EES that provided EEC context ID. |  |
| srcEesEndpoint | EndPoint | O | 0..1 | The endpoint address of the EES that provided EEC context ID. |  |

Editor’s note:	The security credentials of EEC depends on SA3 WG and is FFS.

##### 6.x.5.2.3	Type: ACProfile

Table 6.x.5.2.2-1: Definition of type ACProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| acId | string | M | 1 | Identity of the AC. |  |
| acType | string | O | 0..1 | The category or type of AC. |  |
| preferredEcsps | array(string) | O | 1..N | Indicates to the ECS which ECSPs are preferred for the AC. The ECS may use this information in the selection of EESs. |  |
| acSchedule | ScheduledCommunicationTime | O | 0..1 | Indicates the expected operation schedule of the AC (e.g. time windows) |  |
| expectedAcGeoServiceArea | LocationArea5G | O | 0..1 | Indicates the expected location(s) (e.g. route) of the hosting UE during the AC's operation schedule. |  |
| serviceContinuitySupport | boolean | O | 0..1 | Indicates if service continuity support is required or not for the application. |  |
| eass | Array(EasDetail) | O | 1..N | Provides the list of EAS that serve the AC along with the service KPIs required by the AC |  |

##### 6.x.5.2.4	Type: EasDetail

Table 6.x.5.2.2-1: Definition of type EasDetail

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easId | string | M | 1 | Identifier of the EAS |  |
| expectedSvcKPIs | ACServiceKPIs | O | 0..1 | Describes the KPIs expected in order for ACs to receive currently required services from the EAS |  |
| minimumReqSvcKPIs | ACServiceKPIs | O | 0..1 | Describes the minimum KPIs required in order for ACs to receive meaningful services from the EAS |  |

##### 6.x.5.2.5	Type: ACServiceKPIs

Table 6.x.5.2.2-1: Definition of type ACServiceKPIs

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| connectionBW | Bandwidth | O | 0..1 | The required connection bandwidth in Kbit/s for the application. |  |
| ReqRate | Uinteger | O | 0..1 | The request rate to be generated by the AC. |  |
| RespTime | DurationSec | O | 0..1 | Response time required for the server servicing the requests. |  |
| avail | Uinteger | O | 0..1 | Percentage of time the server is required to be available for the AC's use. |  |
| compute | string | O | 0..1 | The compute resources required by the AC. |  |
| grapCompute | string | O | 0..1 | The graphical compute resources required by the AC. |  |
| mery | string | O | 0..1 | The memory resources required by the AC. |  |
| strge | string | O | 0..1 | The storage resources required by the AC. |  |

Editor’s note:	The data type definitions for the attributes in ACServiceKPIs data type are FFS.

### 6.x.6	Error Handling

General error handling are described in clause 6.1.

### 6.x.7	Feature negotiation

General feature negotiation procedures are described in clause 6.1. Table 6.x.7-1 lists the supported features for Eees_EECRegistration API.

Table 6.x.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |