3GPP TSG-CT WG1 Meeting #129-e	C1-212xyz

- Electronic meeting; 19-23 April 2021	(was C1-212378)

Source:	Huawei, HiSilicon

Title:	EAS discovery procedure

Spec:	3GPP TS 24.558

Agenda item:	17.2.16

Document for:	Agreement

1. Introduction

3GPP TS 24.558 contains a clause on Edge Enabler Server API Definitions.

2. Reason for Change

3GPP TS 23.558 clause 8.5 provides the requirements for the EAS discovery procedure. The EAS discovery procedure enables the EEC to obtain information about available EASs of interest. The discovery of the EASs is based on matching EAS discovery filters provided in the request. Clause 8.5.4 of 3GPP TS 23.558 provides the detail requirements for defining the necessary API.

The API enables the EEC to obtain information about available EASs of interest. Similar as other RPC APIs defined by CT3 or CT4, e.g., the ECRControl API defined in 3GPP TS 29.122 which is applicable for 4G SCEF and 5G NEF, etc.

The request operation of the API should be RPCa custom API (RPC interaction) whereas the subscribe/unsubscribe operation should be RESTful as resources are needed.

A number of editor’s notes are proposed as further work is needed.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.558 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[r23558]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[r29122]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Application Client

API	Application Programming Interface

EAS	Edge Application Server

ECS	Edge Configuration Server

EEC	Edge Enabler Client

EES	Edge Enabler Server

NAS	Non Access Stratum

UE	User Equipment

* * * Next Change * * * *

## 6.x	Eees_EASDiscovery API

### 6.x.1	API URI

The request URI used in each HTTP request from the EEC towards the EES shall have the structure as defined in clause 6.1.y with the following clarifications:

-	The <apiName> shall be "eees-easdiscovery".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.x.2.

### 6.x.2	Resources

#### 6.x.2.1	Overview

Figure 6.x.2.1-1: Resource URI structure of the Eees_EASDiscovery API

Table 6.x.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.x.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| EAS Discovery Subscriptions | /subscriptions | POST | Creates a new individual EAS discovery subscription. |
| Individual EAS Discovery Subscription | /subscriptions/{subscriptionId} | PUT | Updates an existing individual EAS discovery subscription identified by the subscriptionId. |
|  |  | DELETE | Deletes an existing individual EAS discovery subscription identified by the subscriptionId. |
| Discovery | /discovery | discovery (POST) | Discovers the EAS information. |

Editor’s note:	Whether the EEC Id should be included in the resource URI structure is FFS.

Editor’s note:	Whether PATCH method will be used to partially update the individual subscription resource is FFS.

#### 6.x.2.2	Resource: EAS Discovery Subscriptions

##### 6.x.2.2.1	Description

This resource represents a collection of EAS Discovery Subscriptions with an EES.

##### 6.x.2.2.2	Resource Definition

Resource URI: {apiRoot}/eees-easdiscovery/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.x.2.2.2-1.

Table 6.x.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1. |
| apiVersion | string | See clause 6.1. |

##### 6.x.2.2.3	Resource Standard Methods

###### 6.x.2.2.3.1	POST

This method creates a new subscription. This method shall support the URI query parameters specified in table 6.x.2.2.3.1-1.

Table 6.x.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.x.2.2.3.1-2 and the response data structures and response codes specified in table 6.x.2.2.3.1-3.

Table 6.x.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EasDiscoverySubscription | M | 1 | Create an Individual EAS Discovery Subscription resource. |

Table 6.x.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EasDiscoverySubscription | M | 1 | 201 Created | Individual EAS Discovery Subscription resource created successfully. The URI of the created resource shall be returned in the "Location" HTTP header |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [r29122] also apply. |  |  |  |  |

Table 6.x.2.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 6.x.2.2.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/eees-easdiscovery/<apiVersion>/subscriptions/{subscriptionId} |

##### 6.x.2.2.4	Resource Custom Operations

None.

#### 6.x.2.3	Resource: Individual EAS Discovery Subscription

##### 6.x.2.3.1	Description

This resource represents modification or deletion of an Individual EAS Discovery Subscription resource.

##### 6.x.2.3.2	Resource Definition

Resource URI: {apiRoot}/eees-easdiscovery/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.x.2.3.2-1.

Table 6.x.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1. |
| apiVersion | string | See clause 6.1. |
| subscriptionId | string | The identifier of a specific individual EAS discovery subscription. |

##### 6.x.2.3.3	Resource Standard Methods

###### 6.x.2.3.3.1	PUT

This method updates the individual EAS discovery subscription resource by completely replacing the existing subscription data (except subscriptionId). This method shall support the URI query parameters specified in table 6.x.2.3.3.1-1.

Table 6.x.2.3.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.x.2.3.3.1-2 and the response data structures and response codes specified in table 6.x.2.3.3.1-3.

Table 6.x.2.3.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EasDiscoverySubscription | M | 1 | An individual EAS discovery subscription resource to be updated. |

Table 6.x.2.3.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EasDiscoverySubscription | M | 1 | 200 OK | An individual EAS discovery subscription resource updated successfully. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [r29122] also apply. |  |  |  |  |

Table 6.x.2.3.3.1-4: Headers supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 6.x.2.3.3.1-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 6.x.2.3.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

###### 6.x.2.3.3.2	DELETE

This method terminates an existing individual EAS discovery subscription. This method shall support the URI query parameters specified in table 6.x.2.3.3.2-1.

Table 6.x.2.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.x.2.3.3.2-2 and the response data structures and response codes specified in table 6.x.2.3.3.2-3.

Table 6.x.2.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.x.2.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | An individual individual EAS discovery subscription resource deleted successfully. |
| NOTE: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [r29122] also apply. |  |  |  |  |

##### 6.x.2.3.4	Resource Custom Operations

None.

### 6.x.3	Custom operations without associated resources

#### 6.x.3.1	Overview

Custom operations used for this API are summarized in table 6.x.3.1-1.

Table 6.x.3.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| discovery | /discovery | POST | Request |

#### 6.x.3.2	Operation: query

##### 6.x.3.2.1	Description

This custom operation allows an EEC to request for EAS discovery information as defined in 3GPP TS 23.558 [r23558].

##### 6.x.3.2.2	Operation Definition

This operation shall support the URI query parameters, request and response data structures, and response codes, as specified in the table 6.x.3.2.2-1 and table 6.x.3.2.2-2.

Table 6.x.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EasDiscoveryRequest | M | 1 | Parameters to request to retrieve EAS discovery information. |

Table 6.x.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EasDiscoveryData | M | 1 | 200 OK | The requested EAS discovery was returned successfully. |
| n/a |  |  | 204 No Content | If the requested data does not exist, the EES shall respond with "204 No Content". |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [r29122] also apply. |  |  |  |  |

### 6.x.4	Notifications

Editor’s note:	The definition of notifications is FFS.

### 6.x.5	Data Model

#### 6.x.5.1	General

This clause specifies the application data model supported by the Eees_EASDiscovery API.

Table 6.x.5.1-1 specifies the data types defined specifically for the Eees_EASDiscovery API service.

Table 6.x.5.1-1: Eees_EASDiscovery API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| EasDiscoverySubscription | 6.x.5.2.2 |  |  |
| EasDiscoveryRequest | 6.x.5.2.3 |  |  |
| EasDiscoveryData | 6.x.5.2.4 |  |  |

Table 6.x.5.1-2 specifies data types re-used by the Eees_EASDiscovery API service.

Table 6.x.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |

#### 6.x.5.2	Structured data types

##### 6.x.5.2.1	Introduction

##### 6.x.5.2.2	Type: EasDiscoverySubscription

Editor’s note:	The definition of the data type is FFS.

##### 6.x.5.2.3	Type: EasDiscoveryRequest

Editor’s note:	The definition of the data type is FFS.

##### 6.x.5.2.4	Type: EasDiscoveryData

Editor’s note:	The definition of the data type is FFS.

### 6.x.6	Error Handling

General error handling are described in clause 6.1.

### 6.x.7	Feature negotiation

General feature negotiation procedures are described in clause 6.1. Table 6.x.7-1 lists the supported features for Eees_EASDiscovery API.

Table 6.x.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |