3GPP TSG-CT WG1 Meeting #129-e	C1-212913

- Electronic meeting; 19th – 23rd April

Source:	Qualcomm Incorporated

Title:	Conclusion for Solution 4

Spec:	3GPP TR 24.821

Agenda item:	17.2.4

Document for:	Agreement

1. Introduction

TR 24.821 contains two solutions for key issue #4: Handling of shared/global PLMN IDs. No new solutions for this key issue is expected to be added to the TR.

2. Reason for Change

There is a need for the conclusion to key issue #4.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 24.821 v1.1.0.

* * * First Change * * * *

## 8.x	Conclusion for key issue 4

Key issue 4 was about Handling of shared/global PLMN IDs.

The following is the list of the issues studied in this key issue and the respective conclusions for the normative phase:

a)	what modifications are needed to the PLMN selection procedure to handle shared/global PLMN IDs

b)	should these changes also apply to the legacy PLMN selection procedure;

c)	what information could be available to aid in handling shared/global PLMN IDs:

▪	pre-configured in the UE; and

▪	provided by PLMN; and

d)	how to accommodate LI requirements when handling shared/global PLMN IDs.

For a), the following conclusion is adopted (solution 6):

When the UE is performing periodic search for higher priority PLMN, the UE should not eliminate any candidate PLMN due to the value of its MCC not representing the same country as the MCC of the serving PLMN if:

1)	The candidate PLMN has a PLMN ID with shared MCC; or

2)	The serving PLMN has a PLMN ID with shared MCC.

For b), the changes should apply to non-terrestrial access only.

For c), no information needs to be pre-configured in the UE or provided by the PLMN to aid in handling of shared/global PLMN ID within the scope of key issue 4..

NOTE:	Information may be provided to the UE as part of the solutions to other key issues.

For d), no solution is needed under key issue 4. This issue is handled under key issue 2.

* * * No more changes * * * *