- 3GPP TSG CT WG1 Meeting 130-e	TDoc C1-21xxxx

- Electronic meeting, 20-28 May 2021

Title:	Reply LS on introducing extended DRX for RedCap UEs

Response to:	LS R2-2104374 on introducing extended DRX for RedCap UEs from RAN2

Release:	Release 17

Work Item:	NR_redcap-Core

Source:	CT1

To:	RAN2, SA2

Cc:	RAN3

Contact person:	Lena Chaponniere

lguellec.chaponniere@gmail.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

CT1 thanks RAN2 for their LS on introducing extended DRX for RedCap UEs.

Extended DRX cycles of up to 10485.76 seconds in RRC_IDLE are already supported since Release 16 for NB-IoT connected to the 5GC. It is feasible from a NAS protocol point of view to extend this support to RedCap UEs in RRC_IDLE in Release 17 in CT1 specifications. However, CT1 would like to note that it does not make sense if the maximum supported eDRX value is larger or equal to the periodic update timer used for RedCap UEs and currently the default value of the periodic update timer is 54 minutes.

From a NAS protocol point of view, in order to support extended DRX cycles of up to 10485.76 seconds in RRC_INACTIVE, to avoid NAS message retransmission at the network, the NAS retransmission timer needs to be extended to about 3 hours and to avoid NAS procedure failure, the NAS retransmission timer needs to be extended to about 0.6 hr (= 36 minutes). Extending the NAS retransmission timer to about 3 hours can cause some problems related to NAS, e.g. bad user experience due to delayed DL NAS procedure, the AMF can be easily overloaded due to suspending all DL 5GSM messages, and NAS procedure collisions will happen more often.

The system level impacts of supporting extended DRX cycles of up to 10485.76 seconds in RRC_INACTIVE need to be assessed by SA2.

# 2	Actions

To RAN2

ACTION: 	3GPP TSG CT WG1 asks RAN2 group to take the above feedback into account.

To SA2

ACTION: 	3GPP TSG CT WG1 asks SA2 to comment on the system level impacts of supporting extended DRX cycles of up to 10485.76 seconds in RRC_INACTIVE.

# 3	Dates of next TSG CT WG1 meetings

TBD