- 3GPP TSG CT WG1 Meeting 130-e	TDoc C1-21xxxx

- Electronic meeting, 20-28 May 2021

Title:	Reply LS on introducing extended DRX for RedCap UEs

Response to:	LS R2-2104374 on introducing extended DRX for RedCap UEs from RAN2

Release:	Release 17

Work Item:	NR_redcap-Core

Source:	CT1

To:	RAN2, SA2

Cc:	RAN3

Contact person:	Lena Chaponniere

lguellec.chaponniere@gmail.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

CT1 thanks RAN2 for their LS on introducing extended DRX for RedCap UEs.

Extended DRX cycles of up to 10485.76 seconds in RRC_IDLE are already supported since Release 16 for NB-IoT connected to the 5GC. It is feasible to extend this support to RedCap UEs in RRC_IDLE in Release 17.

For extended DRX in RRC_INACTIVE,  as per existing specifications by which DL NAS procedures are initiated without consideration of Paging Occasions, in order to support extended DRX cycles of up to 10485.76 seconds, to avoid NAS message retransmission at the network, the NAS retransmission timer needs to be extended to about 3 hours and to avoid NAS procedure failure, the NAS retransmission timer needs to be extended to about 0.6 hr (= 36 minutes). Extending the NAS retransmission timer to about 3 hours can cause some problems related to NAS, e.g. bad user experience due to delayed DL NAS procedure, the AMF can be easily overloaded due to suspending all DL 5GSM messages, and NAS procedure collisions will happen more often. CT1 notes that SA2 has decided in Release 16 to only support extended DRX cycles of up to 10.24 seconds (see TS 23.501 subclause 5.31.7.2.1). Extending DRX cycles beyond 10.24 sec would require further work in SA2 and it is thus up to SA2 to indicate whether this is feasible in Release 17.

# 2	Actions

To RAN2

ACTION: 	3GPP TSG CT WG1 asks RAN2 group to take the above feedback into account.

To SA2

ACTION: 	3GPP TSG CT WG1 asks SA2 to comment on the feasibility of supporting extended DRX cycles greater than 10.24 sec in RRC_INACTIVE for Release 17.

# 3	Dates of next TSG CT WG1 meetings

TBD