3GPP TSG-CT WG1 Meeting #130-e	C1-213481

- Electronic meeting; 20-28 May 2021							  was C1-212461 was C1-212157

Source:	Samsung

Title:	Eees_EECRegistration_Request Service Operation

Spec:	3GPP TS 24.558 v0.2.0

Agenda item:	17.2.16

Document for:	Agreement

1. Introduction

This pCR specifies Eees_EECRegistration_Request Service Operation and adds EEC Service Continuity Support attribute in EEC registration request.

2. Reason for Change

The Eees_EECRegistration_Request operation is defined in clause 8.4.2.4.2 of 3GPP TS 23.558. It is required to provide service operations details in stage#3.

Further, in TS 23.558, there is an upate to EEC registration request (in S6-211114). It is required to make similar changes to stage#3 specification.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.558 v0.2.0.

* * * First Change * * * *

#### 5.x.2.2	Eees_EECRegistration_Request

##### 5.x.2.2.1	General

This service operation is used by EEC to register itself with a given EES.

Editor’s note: Details about EEC security credentials, verification and authorization of Eees_EECRegistration Request, by the EES, is FFS and depends on security requirements to be defined by SA3.

##### 5.x.2.2.2	EEC registering to EES using Eees_EECRegistration_Request operation

For an EEC to register at the EES, the EEC shall send an HTTP POST message to the EES on the "EEC Registrations" collection resource to create the resource associated to or representing the EEC. The body of the HTTP POST message shall include the EEC ID, may include UE identifier, AC Profile(s), proposed expiration time for the registration, EEC context ID obtained from a previous registration, as specified in clause 6.x.2.2.3.1. If EEC context ID is included in the body of the HTTP POST message, it shall also include Source EES ID and Source EES Endpoint of the EES that provided EEC context ID.

Upon receiving the HTTP POST message from the EEC, the EES shall:

a)	Process the EEC registration request information;

b)	verify if the EEC is authorized to register itself at EES; and

c)	if the EEC is authorized to register with EES, then;

1)	if the AC Profile(s) is included in the HTTP POST message, the EES further determines whether the registered EAS(s) fulfils the requirements that were indicated in the AC Profile(s) as follows:

i)	For the requirements not included in the List of EASs, the EAS has to be matched to at least one of the requirements if available.

ii)	For the requirements included in the List of EASs, the EAS has to be matched to all requirements included in the Minimum required Service KPIs if available and at least one of the requirements included in the Expected Service KPIs if available. In the case Minimum required Service KPIs and Expected Service KPI contains the same KPI attribute, the EES chooses the KPI value included in Minimum required Service KPIs for this determination.

When a matching EAS is identified, the EES determines that the corresponding requirements are fulfilled and are supported for the new resource.

Editor’s note: When the HTTP POST message includes multiple AC Profile(s), it is FFS how EES determines the requirements in the AC Profile(s) are fulfilled and how EES indicates to the EEC for which AC Profile the requirement is fulfilled.

2)	if the received EEC registration request contains an EEC context ID, a source EES endpoint, the EES retrieves the EEC's context from the source EES;

Editor’s note: The reference of how EES retrieves the EEC’s context from source EES will be added once it is available.

3)	the EES creates a new resource with the EEC registration information as specified in clause 6.x.2.1, and assigns and stores new EEC context ID;

i)	if the EES cannot reserve the necessary resources while meeting the capability requirements of the existing registered EECs, the EES shall determine the EEC Context information stale and send a failure response with a corresponding cause as specified in clause 6.2.2.2.3.1; and

ii)	Otherwise the EES shall return the EEC registration information in the response message. The response message may include expiration time to indicate when the EEC registration will automatically expire. The response message may include a newly assigned EEC context ID. The URI of the created resource shall be returned in the "Location" HTTP header.

The EEC stores the new EEC context ID and uses it when it registers with another EES.

If the expiration time is provided, then to maintain the registration, the EEC shall send a registration update request (as described in clause 5.x.2.2.3) prior to the expiration time. If the registration update request is not received prior to the expiration time, then the EES shall treat the EEC as implicitly deregistered and remove the corresponding EEC registration resource.

* * * Next Change * * * *

##### 6.2.5.2.2	Type: EecRegistration

Table 6.2.5.2.2-1: Definition of type EecRegistration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eecId | string | M | 0..1 | Represents a unique identifier of the EEC. |  |
| ueId | string | O | 0..1 | Represents the identifier of the UE. |  |
| acProfs | array(ACProfile) | O | 1..N | Profiles of ACs for which the EEC provides edge enabling services. This attribute shall be provided in the POST request and in the response of successful resource creation. |  |
| expTime | DateTime | O | 0..1 | Represents an expiration time for the registration. This attribute shall be present in the response of the HTTP POST message from EEC to create a new registration or in the response of the HTTP PUT message from EEC to update a specific registration. If abesent, then it indicates that the registration of EEC never expires. |  |
| eecCntxId | string | O | 0..1 | Identifier of the EEC context obtained from a previous registration. |  |
| srcEesId | string | O | 0..1 | Identifier of the EES that provided EEC context ID. |  |
| srcEesEndpoint | EndPoint | O | 0..1 | The endpoint address of the EES that provided EEC context ID. |  |
| eecSvcContSupp | boolean | O | 0..1 | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC. |  |

Editor’s note:	The security credentials of EEC depends on SA3 WG and is FFS.

* * * End of Change * * * *