3GPP TSG-CT WG1 Meeting #130-e	C1-213032

- Electronic meeting; 20-28 May 2021

Source:	vivo

Title:	TS24.554: Update UE-requested 5G ProSe policy provisioning procedure for adding new request type

Spec:	3GPP TS 24.554 v0.2.0

Agenda item:	17.2.18

Document for:	Agreement

1. Reason for Change

Based on clause 4.3.1 of TS 23.304, this contribution proposes to add Update UE-requested 5G ProSe policy provisioning procedure for adding new request type into TS 24.554.

2. Proposal

It is proposed to capture the following changes into TS 24.554 v0.2.0.

* * * First Change * * * *

- 5.3.2.1	General

The UE-requested PSP policy provisioning procedure enables the UE to request PSP from the PCF in the following cases:

a)	if the T5051 for UE policies for 5G ProSe direct discovery expires;

b)	if the T5052 for UE policies for 5G ProSe direct communications expires;

c)	if the T5053 for UE policies for 5G ProSe Layer-2 UE-to-network relay expires;

d)	if the T5054 for UE policies for 5G ProSe Layer-3 UE-to-network relay expires;

e)	if the T5055 for UE policies for 5G ProSe Layer-2 Remote UE expires;

f)	if the T5056 for UE policies for 5G ProSe Layer-3 Remote UE expires;

g)	if there are no valid configuration parameters, e.g., for the current area, or due to abnormal situation.

The UE shall follow the principles of PTI handling for UE policy delivery service procedures defined in 3GPP TS 24.501 [11] clause D.1.2.

Editor's note:	usage of UE-requested PSP provisioning procedure to request PSP for UE policies for 5G ProSe UE-to-UE relay is FFS.

* * * Next Change * * * *

- 5.3.2.3	UE-requested PSP policy provisioning procedure accepted by the network

Handling in 3GPP TS 24.587 [18] clause 5.3.2.3 shall apply.

If new UE policies for 5G ProSe direct discovery are included in the MANAGE UE POLICY COMMAND message, the UE shall stop timer T5051 if it is running and start timer T5051 with the value included in the UE policies for 5G ProSe direct discovery, and start using the new UE policies for 5G ProSe direct discovery included in the MANAGE UE POLICY COMMAND message.

If new UE policies for 5G ProSe direct communications are included in the MANAGE UE POLICY COMMAND message, the UE shall stop timer T5052 if it is running and start timer T5052 with the value included in the UE policies for 5G ProSe direct communications, and start using the new UE policies for 5G ProSe direct communications included in the MANAGE UE POLICY COMMAND message.

If new UE policies for 5G ProSe Layer-2 UE-to-network relay are included in the MANAGE UE POLICY COMMAND message, the UE shall stop timer T5053 if it is running and start timer T5053 with the value included in the UE policies for 5G ProSe Layer-2 UE-to-network relay, and start using the new UE policies for 5G ProSe Layer-2 UE-to-network relay included in the MANAGE UE POLICY COMMAND message.

If new UE policies for 5G ProSe Layer-3 UE-to-network relay are included in the MANAGE UE POLICY COMMAND message, the UE shall stop timer T5054 if it is running and start timer T5054 with the value included in the UE policies for 5G ProSe Layer-3 UE-to-network relay, and start using the new UE policies for 5G ProSe Layer-3 UE-to-network relay included in the MANAGE UE POLICY COMMAND message.

If new UE policies for 5G ProSe Layer-2 Remote UE are included in the MANAGE UE POLICY COMMAND message, the UE shall stop timer T5055 if it is running and start timer T5055 with the value included in the UE policies for 5G ProSe Layer-2 Remote UE, and start using the new UE policies for 5G ProSe Layer-2 Remote UE included in the MANAGE UE POLICY COMMAND message.

If new UE policies for 5G ProSe Layer-3 Remote UE are included in the MANAGE UE POLICY COMMAND message, the UE shall stop timer T5056 if it is running and start timer T5056 with the value included in the UE policies for 5G ProSe Layer-3 Remote UE, and start using the new UE policies for 5G ProSe Layer-3 Remote UE included in the MANAGE UE POLICY COMMAND message.

* * * Next Change * * * *

## 12.2	Timers of provisioning of parameters for 5G ProSe configuration procedures

Timers of provisioning of parameters for 5G ProSe configuration are shown in table 13.2.1.

NOTE:	Timer T5040 is defined in 3GPP TS 24.587 [18].

Table 12.2.1: Timers of provisioning of parameters for 5G ProSe configuration – UE side

| TIMER NUM. | TIMER VALUE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- |
| T5051 | TBD | Start using the new UE policies for 5G ProSe direct discovery received in MANAGE UE POLICY COMMAND message | Stop using the old UE policies for 5G ProSe direct discovery | Initiate the UE-requested PSP provisioning procedure (NOTE 3) |
| T5052 | TBD | Start using the new UE policies for 5G ProSe direct communications received in MANAGE UE POLICY COMMAND message | Stop using the old UE policies for 5G ProSe direct communications | Initiate the UE-requested PSP provisioning procedure (NOTE 3) |
| T5053 | TBD | Start using the new UE policies for 5G ProSe Layer-2 UE-to-network relay received in MANAGE UE POLICY COMMAND message | Stop using the old UE policies for 5G ProSe Layer-2 UE-to-network relay | Initiate the UE-requested PSP provisioning procedure (NOTE 3) |
| T5054 | TBD | Start using the new UE policies for 5G ProSe Layer-3 UE-to-network relay received in MANAGE UE POLICY COMMAND message | Stop using the old UE policies for 5G ProSe Layer-3 UE-to-network relay | Initiate the UE-requested PSP provisioning procedure (NOTE 3) |
| T5055 | TBD | Start using the new UE policies for 5G ProSe Layer-2 Remote UE received in MANAGE UE POLICY COMMAND message | Stop using the old UE policies for 5G ProSe Layer-2 Remote UE | Initiate the UE-requested PSP provisioning procedure (NOTE 3) |
| T5056 | TBD | Start using the new UE policies for 5G ProSe Layer-3 Remote UE received in MANAGE UE POLICY COMMAND message | Stop using the old UE policies for 5G ProSe Layer-3 Remote UE | Initiate the UE-requested PSP provisioning procedure (NOTE 3) |
| NOTE 3: The timers expire only once. |  |  |  |  |

Editor's note: timer values are FFS.

Editor's note:	timer for UE policies for 5G ProSe UE-to-UE relay is FFS.