3GPP TSG-CT WG1 Meeting #130-e	C1-21xxxx was C1-213204

- Electronic meeting; 20-28 May 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Defining the 5G ProSe signalling messages

Spec:	3GPP TS 24.554 V0.2.0

Agenda item:	17.2.18

Document for:	Agreement

1. Introduction

Multiple ProSe PC5 Direct Link procedures have been introduced in 3GPP TS 24.554 V0.2.0. The signalling messages for those procedures are not defined yet.

2. Reason for Change

The signalling messages for the ProSe PC5 Direct Link procedures need to be defined. Also Editor Notes are added in the caluses related to Security and Authentication Messages since they are subject to change as they are waiting for the SA3 work completion.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.554 V0.2.0.

* * * First Change * * * *

### 10.3.6	ProSe direct link modification request

#### 10.3.6.1	Message definition

This message is sent by the UE to another peer UE to initiate the direct link modification procedure. See table 10.3.4.6.1.

Message type:	PROSE DIRECT LINK MODIFICATION REQUEST

Significance:	dual

Direction:		UE to peer UE

Table 10.3.4.6.1: PROSE DIRECT LINK MODIFICATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK MODIFICATION REQUEST message identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | Link modification operation code | Link modification operation code 11.3.19 | M | V | 1 |
|  | QoS flow descriptions | PC5 QoS flow descriptions 11.3.5 | M | LV-E | 5-65537 |

### 10.3.7	ProSe direct link modification accept

#### 10.3.7.1	Message definition

This message is sent by the UE to another peer UE to indicate that the link modification request is accepted. See table 10.3.7.1.1.

Message type:	PROSE DIRECT LINK MODIFICATION ACCEPT

Significance:	dual

Direction:		UE to peer UE

Table 10.3.7.1.1: PROSE DIRECT LINK MODIFICATION ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK MODIFICATION ACCEPT message identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
| 79 | QoS flow descriptions | PC5 QoS flow descriptions 11.3.5 | O | TLV-E | 6-65538 |

#### 10.3.7.2	QoS flow descriptions

The UE shall include this IE if the PC5 unicast link modification procedure is to:

a)	add new PC5 QoS flow(s) to the existing 5G ProSe direct link;

b)	modify PC5 QoS parameters of the existing PC5 QoS flow(s);

c)	associate new ProSe Application(s) with existing PC5 QoS flow(s); or

d)	remove ProSe Application(s) from existing PC5 QoS flow(s).

### 10.3.8	ProSe direct link keepalive request

#### 10.3.8.1	Message definition

This message is sent by a UE to another peer UE when a PC5 unicast link keep-alive procedure is initiated. See table 10.3.8.1.1.

Message type:	PROSE DIRECT LINK KEEPALIVE REQUEST

Significance:	dual

Direction:		UE to peer UE

Table 10.3.8.1.1: PROSE DIRECT LINK KEEPALIVE REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK KEEPALIVE REQUEST message identity | ProSe PC5 signalling message type 11.3.1. | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | Keep-alive counter | Keep-alive counter 11.3.20 | M | V | 4 |
| 55 | Maximum inactivity period | Maximum inactivity period 11.3.21 | O | TV | 5 |

#### 10.3.8.2	Maximum inactivity period

The UE may include this IE to indicate its maximum inactivity period to the peer UE.

### 10.3.9	ProSe direct link keepalive response

#### 10.3.9.1	Message definition

This message is sent by a UE to another peer UE to respond to a PROSE DIRECT LINK KEEPALIVE REQUEST message. See table 10.3.9.1.1.

Message type:	PROSE DIRECT LINK KEEPALIVE RESPONSE

Significance:	dual

Direction:		UE to peer UE

Table 10.3.9.1.1: PROSE DIRECT LINK KEEPALIVE RESPONSE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK KEEPALIVE RESPONSE message identity | ProSe PC5 signalling message type 11.3.1. | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | Keep-alive counter | Keep-alive counter 11.3.20 | M | V | 4 |

### 10.3.10	ProSe direct link authentication request

Editor's note:	The security related aspects are FFS as they are waiting for the definitions in SA3 specification by SA3 working group.

#### 10.3.10.1	Message definition

This message is sent by a UE to another peer UE when a PC5 unicast link authentication procedure is initiated. See table 10.3.10.1.1.

Message type:	PROSE DIRECT LINK AUTHENTICATION REQUEST

Significance:	dual

Direction:		UE to peer UE

Table 10.3.10.1.1: PROSE DIRECT LINK AUTHENTICATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK AUTHENTICATION REQUEST message identity | ProSe PC5 signalling message type 11.3.1. | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | Key establishment information container | Key establishment information container 11.3.9 | M | LV-E | 3-n |

### 10.3.11	ProSe direct link authentication response

Editor's note:	The security related aspects are FFS as they are waiting for the definitions in SA3 specification by SA3 working group.

#### 10.3.11.1	Message definition

This message is sent by a UE to another peer UE to respond to a PROSE DIRECT LINK AUTHENTICATION REQUEST message. See table 10.3.11.1.1.

Message type:	PROSE DIRECT LINK AUTHENTICATION RESPONSE

Significance:	dual

Direction:		UE to peer UE

Table 10.3.11.1.1: PROSE DIRECT LINK AUTHENTICATION RESPONSE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK AUTHENTICATION RESPONSE message identity | ProSe PC5 signalling message type 11.3.1. | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | Key establishment information container | Key establishment information container 11.3.9 | M | LV-E | 3-n |

### 10.3.12	ProSe direct link authentication reject

Editor's note:	The security related aspects are FFS as they are waiting for the definitions in SA3 specification by SA3 working group.

#### 10.3.12.1	Message definition

This message is sent by a UE to another peer UE to reject a PROSE DIRECT LINK AUTHENTICATION REQUEST message. See table 10.3.12.1.1.

Message type:	PROSE DIRECT LINK AUTHENTICATION REJECT

Significance:	dual

Direction:		UE to peer UE

Table 10.3.12.1.1: PROSE DIRECT LINK AUTHENTICATION REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK AUTHENTICATION REJECT message identity | ProSe PC5 signalling message type 11.3.1. | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | PC5 signalling protocol cause | PC5 signalling protocol cause 11.3.8 | M | V | 1 |

### 10.3.13	ProSe direct link security mode command

Editor's note:	The security related aspects are FFS as they are waiting for the definitions in SA3 specification by SA3 working group.

#### 10.3.13.1	Message definition

This message is sent by a UE to another peer UE when a PC5 unicast link security mode control procedure is initiated. See table 10.3.13.1.1.

Message type:	PROSE DIRECT LINK SECURITY MODE COMMAND

Significance:	dual

Direction:		UE to peer UE

Table 10.3.13.1.1: PROSE DIRECT LINK SECURITY MODE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK SECURITY MODE COMMAND message identity | ProSe PC5 signalling message type 11.3.1. | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | Selected security algorithms | Selected security algorithms 11.3.22 | M | V | 1 |
|  | UE security capabilities | UE security capabilities 11.3.11 | M | LV | 3-9 |
| 59 | UE PC5 unicast signalling security policy | UE PC5 unicast signalling security policy 11.3.12 | O | TV | 2 |
| 55 | Nonce_2 | Nonce 11.3.10 | O | TV | 17 |
| 52 | LSB of KNRP-sess ID | LSB of KNRP-sess ID 11.3.15 | O | TV | 2 |
| 74 | Key establishment information container | Key establishment information container 11.3.9 | O | TLV-E | 4-n |
| 62 | MSBs of KNRP ID | MSBs of KNRP ID 11.3.16 | O | TV | 3 |

#### 10.3.13.2	Nonce_2

The UE shall include this IE if the selected integrity protection algorithms is not the null integrity protection algorithm.

#### 10.3.13.3	LSB of KNRP-sess ID

The UE shall include this IE if the selected integrity protection algorithms is not the null integrity protection algorithm.

#### 10.3.13.4	Key establishment information container

The UE shall include this IE if the UE has derived a new KNRP and the authentication method used to generate KNRP requires sending information to complete the authentication procedure.

#### 10.3.13.5	MSBs of KNRP ID

The UE shall include this IE if the UE has derived a new KNRP.

#### 10.3.13.6	UE PC5 unicast signalling security policy

The UE shall include this IE if the PROSE DIRECT LINK SECURITY MODE COMMAND message is triggered by the PROSE DIRECT LINK ESTABLISHMENT REQUEST message. The content of the IE is the same as the content of UE PC5 unicast signalling security policy IE in the received PROSE DIRECT LINK ESTABLISHMENT REQUEST message in order to provide protection against bidding down attacks.

### 10.3.14	ProSe direct link security mode complete

Editor's note:	The security related aspects are FFS as they are waiting for the definitions in SA3 specification by SA3 working group.

#### 10.3.14.1	Message definition

This message is sent by a UE to another peer UE to respond to a PROSE DIRECT LINK SECURITY MODE COMMAND message. See table 10.3.14.1.1.

Message type:	PROSE DIRECT LINK SECURITY MODE COMPLETE

Significance:	dual

Direction:		UE to peer UE

Table 10.3.14.1.1: PROSE DIRECT LINK SECURITY MODE COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK SECURITY MODE COMPLETE message identity | ProSe PC5 signalling message type 11.3.1. | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | QoS flow descriptions | PC5 QoS flow descriptions 11.3.5 | M | LV-E | 6-n |
|  | UE PC5 unicast user plane security policy | UE PC5 unicast user plane security policy 11.3.23 | M | V | 1 |
| 57 | IP address configuration | IP address configuration 11.3.6 | O | TV | 2 |
| 58 | Link local IPv6 address | Link local IPv6 address 11.3.7 | O | TV | 17 |
| 52 | LSBs of KNRP ID | LSBs of KNRP ID 11.3.17 | O | TV | 3 |

#### 10.3.14.2	IP address configuration

The UE shall include this IE if IP communication is used and the PC5 unicast link security mode control procedure was triggered during a PC5 unicast link establishment procedure.

#### 10.3.14.3	Link local IPv6 address

The UE shall include this IE if IP communication is used, the IP address configuration is set to "IPv6 address allocation not supported" and the PC5 unicast link security mode control procedure was triggered during a PC5 unicast link establishment procedure.

#### 10.3.14.4	LSBs of KNRP ID

The UE shall include this IE if a new KNRP was derived.

### 10.3.15	ProSe direct link security mode reject

Editor's note:	The security related aspects are FFS as they are waiting for the definitions in SA3 specification by SA3 working group.

#### 10.3.15.1	Message definition

This message is sent by a UE to another peer UE to reject a PROSE DIRECT LINK SECURITY MODE COMMAND message. See table 10.3.15.1.1.

Message type:	PROSE DIRECT LINK SECURITY MODE REJECT

Significance:	dual

Direction:		UE to peer UE

Table 10.3.15.1.1: PROSE DIRECT LINK SECURITY MODE REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK SECURITY MODE REJECT message identity | ProSe PC5 signalling message type 11.3.1. | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | PC5 signalling protocol cause | PC5 signalling protocol cause 11.3.8 | M | V | 1 |

### 10.3.16	ProSe direct link rekeying request

#### 10.3.16.1	Message definition

This message is sent by a UE to another peer UE when a PC5 unicast link re-keying procedure is initiated. See table 10.3.16.1.1.

Message type:	PROSE DIRECT LINK REKEYING REQUEST

Significance:	dual

Direction:		UE to peer UE

Table 10.3.16.1.1: PROSE DIRECT LINK REKEYING REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK REKEYING REQUEST message identity | ProSe PC5 signalling message type 11.3.1. | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | UE security capabilities | UE security capabilities 11.3.11 | M | LV | 3-9 |
| 74 | Key establishment information container | Key establishment information container 11.3.9 | O | TLV-E | 4-n |
| 53 | Nonce_1 | Nonce 11.3.10 | O | TV | 17 |
| 54 | MSB of KNRP-sess ID | MSB of KNRP-sess ID 11.3.13 | O | TV | 2 |
| 56 | Re-authentication indication | Re-authentication indication 11.3.24 | O | TV | 2 |

#### 10.3.16.2	Key establishment information container

The UE shall include this IE if the null integrity protection algorithm is not in use.

#### 10.3.16.3	Nonce_1

The UE shall include this IE if the null integrity protection algorithm is not in use.

#### 10.3.16.4	MSB of KNRP-sess ID

The UE shall include this IE if the null integrity protection algorithm is not in use.

#### 10.3.16.5	Re-authentication indication

The UE shall include this IE if the UE wants to derive a new KNRP.

### 10.3.17	ProSe direct link rekeying response

#### 10.3.17.1	Message definition

This message is sent by a UE to another peer UE to respond to a PROSE DIRECT LINK REKEYING REQUEST message. See table 10.3.17.1.1.

Message type:	PROSE DIRECT LINK REKEYING RESPONSE

Significance:	dual

Direction:		UE to peer UE

Table 10.3.17.1.1: PROSE DIRECT LINK REKEYING RESPONSE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK REKEYING RESPONSE message identity | ProSe PC5 signalling message type 11.3.1. | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |

### 10.3.18	ProSe direct link identifier update request

#### 10.3.18.1	Message definition

This message is sent by a UE to another peer UE to initiate the direct link identifier procedure. See table 10.3.18.1.1.

Message type:	PROSE DIRECT LINK IDENTIFIER UPDATE REQUEST

Significance:	dual

Direction:		UE to peer UE

Table 10.3.18.1.1: PROSE DIRECT LINK IDENTIFIER UPDATE REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK IDENTIFIER UPDATE REQUEST message identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | MSB of KNRP-sess ID | MSB of KNRP-sess ID 11.3.13 | M | V | 1 |
|  | Source layer-2 ID | Layer-2 ID 11.3.25 | M | V | 3 |
| 57 | Source user info | Application layer ID 11.3.4 | O | TLV | 4-254 |
| 58 | Source link local IPv6 address | Link local IPv6 address 11.3.7 | O | TV | 17 |

#### 10.3.18.2	Source user info

This IE is included when the initiating UE receives a new application layer ID.

#### 10.3.18.3	Source link local IPv6 address

This IE is included when the link local IPv6 address changes at the initiating UE.

### 10.3.19	ProSe direct link identifier update accept

#### 10.3.19.1	Message definition

This message is sent by the UE to another peer UE to indicate that the link identifier update request is accepted. See table 10.3.19.1.1.

Message type:	PROSE DIRECT LINK IDENTIFIER UPDATE ACCEPT

Significance:	dual

Direction:		UE to peer UE

Table 10.3.19.1.1: PROSE DIRECT LINK IDENTIFIER UPDATE ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK IDENTIFIER UPDATE ACCEPT message identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | LSB of KNRP-sess ID | LSB of KNRP-sess ID 11.3.15 | M | V | 1 |
|  | MSB of KNRP-sess ID | MSB of KNRP-sess ID 11.3.13 | M | V | 1 |
|  | Source layer-2 ID | Layer-2 ID 11.3.25 | M | V | 3 |
|  | Target layer-2 ID | Layer-2 ID 11.3.25 | M | V | 3 |
| 28 | Target user info | Application layer ID 11.3.4 | O | TLV | 4-254 |
| 59 | Target link local IPv6 address | Link local IPv6 address 11.3.7 | O | TV | 17 |
| 57 | Source user info | Application layer ID 11.3.4 | O | TLV | 4-254 |
| 58 | Source link local IPv6 address | Link local IPv6 address 11.3.7 | O | TV | 17 |

#### 10.3.19.2	Target user info

This IE is included if the target UE receives the Source user info IE in the PROSE DIRECT LINK IDENTIFIER UPDATE REQUEST message.

#### 10.3.19.3	Target link local IPv6 address

This IE is included if the target UE receives the Source link local IPv6 address IE in the PROSE DIRECT LINK IDENTIFIER UPDATE REQUEST message.

#### 10.3.19.4	Source user info

This IE is included when the application layer ID changes at the target UE and the target UE receives a new application layer ID from the upper layers.

#### 10.3.19.5	Source link local IPv6 address

This IE is included when the link local IPv6 address changes at the target UE and the target UE receives a new Link local IPv6 address from the upper layers.

### 10.3.20	ProSe direct link identifier update ack

#### 10.3.20.1	Message definition

This message is sent by the initiating UE to target UE to indicate that the initiating UE has received target UE’s accept message. See table 10.3.20.1.1.

Message type:	PROSE DIRECT LINK IDENTIFIER UPDATE ACK

Significance:	dual

Direction:		UE to peer UE

Table 10.3.20.1.1: PROSE DIRECT LINK IDENTIFIER UPDATE ACK message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK IDENTIFIER UPDATE ACK message identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | LSB of KNRP-sess ID | LSB of KNRP-sess ID 11.3.15 | M | V | 1 |
|  | Target layer-2 ID | Layer-2 ID 11.3.25 | M | V | 3 |
| 28 | Target user info | Application layer ID 11.3.4 | O | TLV | 4-254 |
| 59 | Target link local IPv6 address | Link local IPv6 address 11.3.7 | O | TV | 17 |

#### 10.3.20.2	Target user info

This IE is included when the initiating UE receives the Source user info IE in the PROSE DIRECT LINK IDENTIFIER UPDATE ACCEPT message.

#### 10.3.20.3	Target link local IPv6 address

This IE is included when the initiating UE receives the Source link local IPv6 address IE in the PROSE DIRECT LINK IDENTIFIER UPDATE ACCEPT message.

### 10.3.21	ProSe direct link identifier update reject

#### 10.3.21.1	Message definition

This message is sent by the target UE to initiating UE to indicate that the link identifier update request is not accepted. See table 10.3.21.1.1.

Message type:	PROSE DIRECT LINK IDENTIFIER UPDATE REJECT

Significance:	dual

Direction:		UE to peer UE

Table 10.3.21.1.1: PROSE DIRECT LINK IDENTIFIER UPDATE REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK IDENTIFIER UPDATE REJECT message identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | PC5 signalling protocol cause | PC5 signalling protocol cause 11.3.8 | M | V | 1 |

### 10.3.22	ProSe direct link modification reject

#### 10.3.22.1	Message definition

This message is sent by the UE to another peer UE to indicate that the link modification request is not accepted. See table 10.3.22.1.1.

Message type:	PROSE DIRECT LINK MODIFICATION REJECT

Significance:	dual

Direction:		UE to peer UE

Table 10.3.22.1.1: PROSE DIRECT LINK MODIFICATION REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK MODIFICATION REJECT message identity | ProSe PC5 signalling message type 11.3.1 | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
|  | PC5 signalling protocol cause | PC5 signalling protocol cause 11.3.8 | M | V | 1 |

### 10.3.23	ProSe direct link authentication failure

Editor's note:	The security related aspects are FFS as they are waiting for the definitions in SA3 specification by SA3 working group.

#### 10.3.23.1	Message definition

This message is sent by a UE to another peer UE to reject a PROSE DIRECT LINK AUTHENTICATION RESPONSE message. See table 10.3.23.1.1.

Message type:	PROSE DIRECT LINK AUTHENTICATION FAILURE

Significance:	dual

Direction:		UE to peer UE

Table 10.3.23.1.1: PROSE DIRECT LINK AUTHENTICATION FAILURE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | PROSE DIRECT LINK AUTHENTICATION FAILURE message identity | ProSe PC5 signalling message type 11.3.1. | M | V | 1 |
|  | Sequence number | Sequence number 11.3.2 | M | V | 1 |
| 74 | Key establishment information container | Key establishment information container 11.3.9 | O | TLV-E | 4-n |

#### 10.3.23.2	Key establishment information container

The UE shall include this IE if it is provided by upper layers.