3GPP TSG-CT WG1 Meeting #130-e	C1-21xxxx was C1-213209

- Electronic meeting; 20-28 May 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Introducing the UE-to-network relay selection procedure

Spec:	3GPP TS 24.554 V0.2.0

Agenda item:	17.2.18

Document for:	Agreement

1. Introduction

Stage-2 ProSe spec TS 23.304 has defined the procedures for UE-to-Network Relay Discovery. For the Relay Discovery to complete successfully, the procedure to enable a remote UE to select a suitable ProSe UE-to-network relay UE needs to be specified.

2. Reason for Change

Introducing the UE-to-network relay selection procedure.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.554 V0.2.0.

* * * First Change * * * *

## 8.2	Procedures for ProSe direct discovery for UE-to-network relay

### 8.2.Z	UE-to-network relay selection procedure

#### 8.2.Z.1	General

The purpose of the UE-to-network relay selection procedure is to enable a remote UE to select a suitable ProSe UE-to-network relay UE to obtain a connectivity service to 5GC.

#### 8.2.Z.2	UE-to-network relay selection procedure initiation

The remote UE shall trigger the UE-to-network relay selection procedure if the following conditions are met:

-	the UE is authorised to act as a remote UE towards a ProSe UE-to-network relay UE as specified in clause X;

-	the UE has obtained a list of ProSe UE-to-network relay UE candidate(s) fulfilling ProSe layer criteria with the monitoring procedure for UE-to-network relay discovery as specified in subclause XYZ or the discoverer procedure for UE-to-network relay discovery as specified in subclause ABC; and

-	the UE has obtained a list of ProSe UE-to-network relay UE candidate(s) fulfilling lower layers criteria as specified in 3GPP TS 38.331 [TBD].

#### 8.2.Z.3	UE-to-network relay selection procedure completion

If there exists only one ProSe UE-to-network relay candidate satisfying the conditions in subclause 8.2.Z.2, then that ProSe UE-to-network relay UE is selected. If there exist more than one ProSe UE-to-network relay candidate satisfying the conditions in subclause 8.2.Z.2, any relay candidates not satisfying the non-radio related ProSe layer criteria shall be discarded, and out of the remaining relay candidates, the relay candidate with the highest ranking of the lower layer criteria shall be selected. The UE may take the value of the Resource Status Indicator bit of the Status Indicator parameter of the PROSE PC5 DISCOVERY message for UE-to-Network Relay Discovery Announcement or PROSE PC5 DISCOVERY message for UE-to-Network Relay Discovery Response into account when deciding which ProSe UE-to-network relay to select. It is up to the UE implementation whether the ProSe layer or the lower layers takes the final selection on which ProSe UE-to-network relay UE to select.

* * * End of Changes * * * *