3GPP TSG-CT WG1 Meeting #130-e	C1-213279

- Electronic meeting; 20-28 May 2021

Source:	LG Electronics

Title:	Conclusion on the use of 3GPP access as a basis

Spec:	3GPP TR 24.811 v1.1.0

Agenda item:	17.2.9

Document for:	Agreement

1. Reason for Change

CT1 has had a discussion on whether a non-3GPP access can be utilized for delivering information on the disaster condition, which includes the information that Disaster Condition occurs in the PLMN D and the information that Disaster Condition no longer applies to the PLMN D. This issue is especially related to Key Issue #1 and Key Issue #6.

In the discussion during CT1#129e meeting, it was pointed out that the non-3gpp access based approach can be used under the condition that a non-3gpp access is deployed in that PLMN (PLMN D or PLMN A), and the UE is in connected mode over non-3gpp access. So there was an observation that non-3gpp access bases approach can be used as optional, not as mandatory way.

During the conference call held on 11 May 2021, the attendees agreed on the following conclusion for this aspect:

3GPP access based solution is the basis solution for the key issue #1. Whether or not to adopt non-3gpp access based solution as a complement will be determined in CT1#130e meeting.

So this pCR would like to add the conclusions on Key Issue #1 and #6, that 3GPP access based solution is the basis solution. Note that the added conclusion itself would not exclude whether the non-3gpp access based solutions will be considered in the normative phase or not, but it will be decided during CT1#130e meeting.

Updates in C1-21xxxx

In the conference call #2 (21 May 2021) for C1#130e, the following way forward was concluded:

way forward: solution for KI#1 uses 3G access provided info, n3G access provided info is not part of the solution in r17

2. Proposal

It is proposed to agree the following changes to 3GPP TTR 24.811 v1.1.0.

* * * First Change * * * *

## 8.1	Conclusions on Key Issue #1

Editor's Note:	The conclusions stated below for the Key Issue #1 are incomplete and preliminary.

The solution #3 is not progressed in the normative phase of FS_MINT-CT.

The solutions utilizing 3GPP access based approach will be the basis solutions for the Key Issue #1 on the notification of Disaster Condition to the UE.

The solutions utilizing non-3GPP access to provide information to the UE is not part of the solution in the Rel-17.

* * * Next Change * * * *

## 8.6	Conclusions on Key Issue #6

Editor's Note:	The conclusions stated below for the Key Issue #6 are incomplete and preliminary.

The solution #35 is not progressed in the normative phase of FS_MINT-CT.

The higher priority PLMN search can be modified under the Disaster Condition.

Solution #30 and #34 will be progressed to normative work to enable the UE to detect that Disaster Condition in PLMN D is no longer applicable without network notification and then to perform the PLMN selection, e.g. in order to return to PLMN D. This is treated as a pure UE based solution for Key Issue #6.

Editor's note:	Conclusions for network based solutions are FFS.

The solutions utilizing 3GPP access based approach will be the basis solutions for the Key Issue #6 on the notification that Disaster Condition is no longer applicable to the UEs.

* * * Next Change * * * *