- 3GPP TSG CT WG1 Meeting 130-e	TDoc C1-213000

- Electronic meeting, 20-28 May 2021

Title:	Reply LS on Small data transmission

Response to:	LS R2-2104644/C1-212849 on Small data transmission from RAN2

Release:	Rel-17

Work Item:	5GProtoc17

Source:	ZTE

To:	RAN2

Cc:	SA2

Contact person:	Shuang Liang

liang.shuang3@zte.com.cn

+86-15850539616

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

CT1 thanks TSG RAN for their LS on Small data transmission.

CT1 would like to confirm that the actual transport mechanism (e.g., SDT RB or non-SDT RB) is determined by the lower layers to transmit the pending data/NAS signalling.

[LS]  I prefer focusing on the pending data in this paragraph. I removes the NAS signalling or NAS procedure. Is it acceptable?

CT1 would like to confirm that the actual transport mechanism (e.g., SDT or UP-EDT etc) used by lower layers to transmit the pending data is transparent to NAS layer. The UE NAS layer will handle the pending uplink user data packe in the RRC_INACTIVE state as legacy.

CT1 would like to indicate that CT1 has not specified whether the pending NAS signalling can be sent to lower layer or not when the NAS layer requests the lower layers to transition to RRC_CONNECTED state. The NAS layer may have impact to ensure the pending NAS signaliing avaialble at the lower layer when the lower layer perfroms resume procedure.

[LS] It is acceptable for me.

Further for the question of RAN2:

RAN2 agreed that only radio bearers configured for SDT are resumed and additional UL and DL data can be exchanged between UE and network as part of a given SDT session while the UE is still in RRC_INACTIVE (i.e. without transition to RRC_CONNECTED). In this case, if new UL data or NAS message becomes available for non-SDT radio bearers (which are suspended), would it be possible that NAS triggers another request to transition into RRC_CONNECTED and provides access category, access identities and resume cause.

CT1 would like to provide the following feedback:

While the UE is in 5GMM-CONNECTED mode with RRC inactive indication and receives an indication from the lower layers that the RRC connection has been suspended, for pending NAS signalling   or uplink data packet (which is subject to the suspended PDU session), the NAS layer can request the lower layers to transition to RRC_CONNECTED state as per legacy behaviour.

[LS] I don’t know why the exception is removed.

In such case specified by CT1:

“Upon a trigger to send a REGISTRATION REQUEST message with the NG-RAN-RCU bit of the 5GS update type IE set to "UE radio capability update needed", the UE in 5GMM-CONNECTED mode with RRC inactive indication shall move to 5GMM-IDLE mode.”

Any ongoing SDT sessionwill be interrupted due to entering idle mode.

[LS] I think NAS layer can’t distinguish the SDT session or non-SDT session. Thus I prefer revision as:

Any data transfer(including for SDT session) will be interrupted due to entering idle mode.

# 2	Actions

To RAN2

ACTION: 	3GPP TSG CT WG1 asks RAN2 to take the above feedback into account.

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 131-e (electronic)	19th -27th August