- 3GPP TSG CT WG1 Meeting 130-e	TDoc C1-213823

- Electronic meeting, 20-28 May 2021

Title:	Reply LS on Small data transmission

Response to:	LS R2-2104644/C1-212849 on Small data transmission from RAN2

Release:	Rel-17

Work Item:	5GProtoc17

Source:	ZTE

To:	RAN2

Cc:	SA2

Contact person:	Shuang Liang

liang.shuang3@zte.com.cn

+86-15850539616

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

CT1 thanks TSG RAN2 for their LS on Small data transmission.

CT1 would like to ask RAN2 to confirm following questions:

-	Is the actual transport mechanism (i.e., SDT or non-SDT) to transmit the pending uplink user data determined by the lower layers without the NAS layer involvement?

-	Can the first pending uplink NAS message and subsequent multiple rounds of uplink and downlink NAS messages be transported via SDT?

CT1 would like to indicate that CT1 has not specified whether the pending uplink NAS message is sent to the lower layers or not when the NAS layer requests the lower layers to transition to RRC_CONNECTED state. Hence, it cannot be gauranteed that the uplink NAS message is avaialble at the lower layers when the lower layers sends the RRC resume request to the network.

For the question of RAN2:

RAN2 agreed that only radio bearers configured for SDT are resumed and additional UL and DL data can be exchanged between UE and network as part of a given SDT session while the UE is still in RRC_INACTIVE (i.e. without transition to RRC_CONNECTED). In this case, if new UL data or NAS message becomes available for non-SDT radio bearers (which are suspended), would it be possible that NAS triggers another request to transition into RRC_CONNECTED and provides access category, access identities and resume cause.

CT1 would like to provide the following feedback according to TS 24.501:

While the UE is in 5GMM-CONNECTED mode with RRC inactive indication, for pending uplink NAS message (except a REGISTRATION REQUEST message with the NG-RAN-RCU bit of the 5GS update type IE set to "UE radio capability update needed") or pending uplink data packets (which are subject to the suspended user plane resources), the NAS layer can request the lower layers to transition to RRC_CONNECTED state and provide access category, access identities and RRC establishment cause (i.e. resume cause used in RAN2) to the lower layers as per legacy behaviour.

If the pending uplink data packets associated with the PDU session with no suspended user plane resources, the Service Request procedure is initiated.

Furthermore, CT1 would like to note in below case, any ongoing SDT session will be interrupted due to entering idle mode:

“Upon a trigger to send a REGISTRATION REQUEST message with the NG-RAN-RCU bit of the 5GS update type IE set to "UE radio capability update needed", the UE in 5GMM-CONNECTED mode with RRC inactive indication shall move to 5GMM-IDLE mode.”

# 2	Actions

To RAN2

ACTION: 	3GPP TSG CT WG1 kindly asks RAN2 to confirm above questions and to take the above feedback into account.

# 3	Dates of next TSG CT WG1 meetings

TSG CT WG1 Meeting 131-e (electronic)	19th -27th August