3GPP TSG-CT WG1 Meeting #131-e	C1-214397

- E-meeting, 19-27 August 2021

Source:	Interdigital

Title:	Eees_AppContextRelocation API

Spec:	3GPP TS 24.558

Agenda item:	17.2.16

Document for:	Agreement

1. Introduction

3GPP TS 24.558 contains a clause on Edge Enabler Server API Definitions, which needs to be completed based on the requirements and procedures spcified in 3GPP TS 23.558

2. Reason for Change

Clause 8.5 of  3GPP TS 23.558 contains service continuity procedures that aim at ensuring succesful Application Context Relocation (ACR) between Edge Application Servers (EAS).

This pCR introduces the Eees_AppContextRelocation API used for ACR request determination and ACR request initiation.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.558.

* * * First Change * * * *

## 6.x	Eees_AppContextRelocation API

### 6.x.1	API URI

The request URI used in each HTTP request from the EEC towards the EES shall have the structure as defined in clause 6.1.y with the following clarifications:

-	The <apiName> shall be "eees-acrevents".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.4.2.

### 6.x.2	Resources

#### 6.x.2.1	Overview

Figure 6.x.2.1-1: Resource URI structure of the Eees_AppContextRelocation API

Table 6.x.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.x.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| n/a | /determine | determine (POST) | EES determines if ACR is needed and may initiate the procedure |
|  | /initiate | initiate (POST) | EES initiates the requested ACR procedure |

#### 6.x.2.2	Resources

None

### 6.x.3	Custom Operations without associated resources

#### 6.x.3.1	Overview

Table 6.x.3.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| /determine | POST | EES determines if ACR is needed and may initiate the procedure |
| /initiate | POST | EES initiates the requested ACR procedure |

#### 6.x.3.2	Operation: Determine

##### 6.x.3.2.1	Description

This custom operation allows the EEC to request that the EES evaluates if ACR is needed and subsequently initiate the ACR procedure if required.

##### 6.x.3.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in tables 6.x.3.2.2-1 and 6.x.3.2.2-2.

Table 6.x.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AcrDeterminationReq | M | 1 | Information about the requestor and requested ACR operation |

Table 6.x.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

#### 6.x.3.3	Operation: Initiate

##### 6.x.3.3.1	Description

This custom operation allows the EEC to request initiation of an ACR procedure.

##### 6.x.3.3.2	Operation Definition

This operation shall support the response data structures and response codes specified in tables 6.x.3.3.2-1 and 6.x.3.3.2-2.

Table 6.x.3.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AcrInitiationReq | M | 1 | Information about the requestor and requested ACR operation |

Table 6.x.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

### 6.x.4	Notifications

None

### 6.x.5	Data Model

#### 6.x.5.1	General

This clause specifies the application data model supported by the Eees_AppContextRelocation API.

Table 6.x.5.1-1 specifies the data types defined specifically for the Eees_AppContextRelocation API service.

Table 6.x.5.1-1: Eees_ AppContextRelocation API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AcrInitiationReq | 6.x.5.2.2 |  |  |
| AcrDeterminationReq | 6.x.5.2.3 |  |  |

Table 6.x.5.1-2 specifies data types re-used by the Eees_AppContextRelocation API service.

Table 6.x.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

#### 6.x.5.2	Structured data types

##### 6.x.5.2.1	Introduction

##### 6.x.5.2.2	Type: AcrInitiationReq

Editor’s note:	The definition of the data type is FFS.

##### 6.x.5.2.3	Type: AcrDeterminationReq

Editor’s note:	The definition of the data type is FFS.

### 6.x.6	Error Handling

General error handling are described in clause 6.1.

### 6.x.7	Feature negotiation

General feature negotiation procedures are defined in clause 6.1. Table 6.x.7-1 lists the supported features for Eees_AppContextRelocation API.

Table 6.x.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |