3GPP TSG-CT WG1 Meeting #131-e	C1-21xxxx

# E-meeting, 19-27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.486 | CR | 0111 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE initiated session-oriented service termination procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  |  | 2021-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The specification needs to define the stage 3 details of the UE initiated session-oriented service termination procedure defined in 3GPP TS 23.286 clause 9.19.3.4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add the UE initiated session-oriented service termination procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE initiated session-oriented service termination procedure is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.13.1.3(New), 6.13.2.3(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.13.1.3	UE initiated session-oriented service termination

In order to terminate a session-oriented service with one or more V2X UEs (remote vehicle), the VAE-C (acting as remote controller) shall generate an HTTP POST request according to procedures specified in IETF RFC 7231 [19]. In the HTTP POST request, the VAE-C:

a)	shall set the Request-URI to the URI included in the received HTTP response for the V2X service discovery procedure (see clause 6.6);

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info+xml";

c)	shall include an application/vnd.3gpp.vae-info+xml MIME body with a <session-oriented-termination-trigger-info> element included in the <VAE-info> root element which shall include:

1)	a <session-id> element set to the session identifier of the session-oriented service that is to be terminated; and

d)	shall send the HTTP POST request towards the VAE-S according to IETF RFC 7231 [19].

* * * NEXT Change * * * *

6.13.2.3	UE initiated session-oriented service termination

Upon receiving an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.vae-info+xml"; and

b)	an application/vnd.3gpp.vae-info+xml MIME body with a <session-oriented-service-termination-info> element,

the VAE-S shall generate an HTTP 200(OK) response message according to procedures specified in IETF RFC 7231 [19]. In the HTTP 200(OK) response, the VAE-S:

a)	shall set the Request-URI to the URI corresponding to the identity of the V2X UE;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info+xml";

c)	shall include an application/vnd.3gpp.vae-info+xml MIME body with a <session-oriented-service-termination-info> element in the <VAE-info> root element which shall include a <result> element indicating success or failure to terminate the session-oriented service; and

d)	shall send the HTTP POST request towards the VAE-C according to IETF RFC 7231 [19].

If the VAE-S terminates the requested session successfully, the VAE-S shall perform the procedure to terminate session oriented service with VAE client (e.g. remote vehicle).

Editor’s note: It is FFS to specify the procedure for VAE-S to terminate session oriented service with VAE client (e.g. remote vehicle).

* * * End of Change * * * *