3GPP TSG-CT WG1 Meeting #131-e	C1-21XXX

Electronic meeting, 19 – 27 Aug 2021	was C1-214625

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 3530 | rev | 1 | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE radio capability ID contains an odd number of hexadecimal digits |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc17, RACS |  |  |  |  |  |  | Date: |  |  | 2021-08-12 |
| Category: | F |  |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  |  | - As the text quoted from clause 29.2 of TS 23.003 specified, the length of UE radio capability ID is 20 hexadecimal digits (when TF=0), or 14 hexadecimal digits (when TF=1), or 12 hexadecimal digits (when TF=2~F). Figure 29.2-1: Structure of UE radio capability ID The UE radio capability ID is composed of the following elements (each element shall consist of hexadecimal digits only): 1) Type Field (TF): identifies the type of UE radio capability ID. The following values are defined: - 0: manufacturer-assigned UE radio capability ID; - 1: network-assigned UE radio capability ID; and - 2 to F: spare values for future use. 2) The Vendor ID is an identifier of UE manufacturer. This is defined by a value of Private Enterprise Number issued by Internet Assigned Numbers Authority (IANA) in its capacity as the private enterprise number administrator, as maintained at https://www.iana.org/assignments/enterprise-numbers/enterprise-numbers. Its length is 8 hexadecimal digits. This field is present only if the Type Field is set to 0; NOTE: The private enterprise number issued by IANA is a decimal number in the range between 0 and 4294967295 that needs to be converted to a fixed length 8 digit hexadecimal number when used within the UE Radio Capability ID. E.g. 32473 is converted to 00007ED9. 3) The Version ID is the current Version ID configured in the UCMF. This field is present only if the Type Field is set to 1. Its length is 2 hexadecimal digits. - Furthermore, according to the following text quoted from Table 9.11.3.68.1 of TS 24.501 specified, each hexadecimal digit coded over 4 bits. That is a UE radio capability ID occupy 80 bits (i.e., 10 octets, when TF=0), or 56 bits (i.e., 7 octets, when TF=1), or 48 bits (i.e., 6 octets, when TF=2~F). Table 9.11.3.68.1: UE radio capability ID information element UE radio capability ID (octets 3 to n). The UE radio capability ID contents contain the UE radio capability ID as specified in 3GPP TS 23.003 [4], with each hexadecimal digit coded over 4 bits, starting with the first hexadecimal digit coded in bits 4 to 1 of octet 3, the second hexadecimal digit coded in bits 8 to 5 of octet 3, and so on. If the UE radio capability ID contains an odd number of hexadecimal digits, bits 8 to 5 of the last octet (octet n) shall be coded as "1111".. Base on the above 2 points we know, the length of the UE radio capability ID IE only can be 12 octets (when TF=0), 9 octets (when TF=1), or 8 octets (when TF=2~F). |
| Summary of change: |  | Clarify that the length of the UE radio capability ID IE can only be 12, 9 or 8 octets |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Clarification for the value of n |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.11.3.68 |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

***** start of 1st change *****

9.11.3.68	UE radio capability ID

The purpose of the UE radio capability ID information element is to carry a UE radio capability ID.

The UE radio capability ID information element is coded as shown in figure 9.11.3.68.1 and table 9.11.3.68.1.

The UE radio capability ID is a type 4 information element with a length of n octets (NOTE X).

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE radio capability ID IEI |  |  |  |  |  |  |  | octet 1 |
| Length of UE radio capability ID contents |  |  |  |  |  |  |  | octet 2 |
| UE radio capability ID |  |  |  |  |  |  |  | octet 3 |
|  |  |  |  |  |  |  |  | octet n |

Figure 9.11.3.68.1: UE radio capability ID information element

Table 9.11.3.68.1: UE radio capability ID information element

UE radio capability ID (octets 3 to n)

The UE radio capability ID contents contain the UE radio capability ID as specified in 3GPP TS 23.003 [4], with each hexadecimal digit coded over 4 bits, starting with the first hexadecimal digit coded in bits 4 to 1 of octet 3, the second hexadecimal digit coded in bits 8 to 5 of octet 3, and so on. If the UE radio capability ID contains an odd number of hexadecimal digits, bits 8 to 5 of the last octet (octet n) shall be coded as "1111".

NOTE X: The value of n is equal to 12 (when the type field of the UE radio capability ID is "manufacturer-assigned UE radio capability ID"), 9 (when the type field of the UE radio capability ID is "network-assigned UE radio capability ID"), or 8 (when the type field of the UE radio capability ID is other values).

***** end of 1st change *****