3GPP TSG-CT WG1 Meeting #133e-bis	C1-220287_r1

- E-meeting, 17-21 January 2022

Source:	Huawei, HiSilicon

Title:	Message Topic unsubscription procedures at client

Spec:	3GPP TS 24.538 v0.2.1

Agenda item:	17.2.30

Document for:	Agreement

1. Introduction

This pCR is to propose the procedures on MSGin5G Client for message topic unsubscription.

2. Reason for Change

As specified in TS 23.554, the MSGin5G UE may unsubscribe one or more message topics.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.538 v0.2.1.

* * * First Change * * * *

### 6.6.x	Messaging Topic Unsubscription Procedure at MSGin5G Client

If the MSGin5G Client needs to unsubscribe a messsage topic, as specified in RFC 7461 [4], the MSGin5G Client shall send a CoAP GET request to MSGin5G Server, in the request, the MSGin5G Client:

a)	shall set the "T" field in the CoAP header to the value "0" to indicate this request is the type of Confirmable;

b)	shall include the MSGin5G Server host address in the Uri-Host Option;

c)	shall include the MSGin5G Server port number in the Uri-Port Option;

d)	shall include the message topic name in the Uri-Path Option (e.g. "\top")

e)	shall include the Observe Option with the value "1" which indicates the observer request to cancel the previous resource observation, i.e. the MSGin5G Client requests to unsubscribe the message topic;

f)	shall include the Content-Format Option with the value "50" which indicate the format of the CoAP payload is "application/json" as specified in RFC 7252 [5]; and

g)	shall include the CoAP Payload in JSON format, including the information elements:

1)	an "Originating UE Service ID" element indicating the MSin5G UE which requests the message topic unsubscription; and

2)	a "Messaging Topics" element to list one or more messaging topic(s) to be unsubscribed.

* * * Next Change * * * *

* * * Next Change * * * *

<Proposed change in revision marks>