3GPP TSG-CT WG1 Meeting #133e-bis	C1-220435_rev1draft

- E-meeting, 17-21 January 2022

Source:	Huawei, HiSilicon

Title:	Message Segmentation & Reassembly Procedures at server

Spec:	3GPP TS 24.538 v0.2.1

Agenda item:	17.2.30

Document for:	Agreement

1. Introduction

This pCR is to propose the procedures to support segmentation and reassembly at MSGin5G Server.

2. Reason for Change

To address the Segmentation & Reassembly specifiecation in TS 23.554

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.538 v0.2.1.

* * * First Change * * * *

### 6.5.2	Procedure at MSGin5G Server

To support the message segment and reassemble, the MSGin5G Server needs to follow the procedures specified in clause 6.4.1.1.6 additionally with the follow procedurs.

#### 6.5.2.1	Procedures on receiving message segments targeting to a MSGin5G UE

Upon receiving a message segment targeting to MSGin5G UE, the MSGin5G Server check if the segment size exceeds the configured maximum packet size of the targeted UE,

1)	if exceed, upon receiving all segments,

i)	reassembles them into a single MSGin5G message,

ii)	splits the re-assembled message to segments such that each segment is smaller than the maximum allowed packet size of the targeted UE.

iii)	sends each new segment to the target MSGin5G UE as specified in 6.4.1.2.6.

2)	if not exceed, upon receiving all segments, sends each segment to the target MSGin5G UE as specified in 6.4.1.2.6

#### 6.5.2.2	Procedures on receiving message segments targeting to an AS

##### 6.5.2.3.1	Reassemble all received message segments

Upon receiving all message segments from MSGin5G UE to an AS, the MSGin5G Server shall reassembles them into a single MSGin5G message and send it to the AS as specified in TS 29.538.

##### 6.5.2.3.2	Send message segments recovery request

If any segment from MSGin5G UE to an AS is not received within the expected time (based on configuration), the MSGin5G Server shall send CoAP POST to the originating MSGin5G Client in the MSGin5G UE for requesting message recovery, in the CoAP POST request, the MSGin5G Server:

a)	shall set the "T" field in the CoAP header to 0 to indicate this request is the type of Confirmable;

b)	shall include the MSGin5G Client address in an CoAP Option, e.g. if the MSGin5G Client address is a URI, include a Uri-Path Option with the value of the URI;

c)	shall set the CoAP Content-Format to "50", i.e. application/json; and

d)	shall include the following information elements in the CoAP payload encoded in JSON format:

1)	an "MSGin5G service identifier" element to indicate that this CoAP POST request is used for MSGin5G service;

2)	an "Message Type" element set to indicate this request is for segments recovery;

3)	a "Segmentation Set Identifier" element copied from one of the previous received segments; and

4)	a "List of Segment range" element to indicate the segments range which the client wants to recover, each segment range consist of start and end sequence number of missing segments e.g. (5-7, 10-10, 15-19);

6.5.2.3.3	Send messgage segments received confirmation

If all segments from a MSGin5G UE to a AS are received within the expected time (based on configuration) or some segments are considered recovery failed, the MSGin5G Server shall send the message segements received information to the originating MSGin5G Client in the MSGin5G UE by CoAP POST request, in the CoAP POST request, the MSGin5G Server:

a)	shall set the "T" field in the CoAP header to 0 to indicate this request is the type of Confirmable;

b)	shall include the MSGin5G Client address in an CoAP Option, e.g. if the MSGin5G Client address is a URI, include a Uri-Path Option with the value of the URI;

c)	shall set the CoAP Content-Format to "50", i.e. application/json; and

d)	shall include the following information elements in the CoAP payload encoded in JSON format:

1)	an "MSGin5G service identifier" element to indicate that this CoAP POST request is used for MSGin5G service;

2)	an "Message Type" element set to indicate that this request is for sending the message received information;

3)	a "Segmentation Set Identifier" element copied from one of the previous received segments; and

4)	a "Result" element to indicate the segments are received successfull or failed.

#### 6.5.2.3	Procedures on receiving messgage segment recovery request to a MSGin5G UE

Upon receiving a CoAP POST request containing the MSGin5G service identifier and containing the Message Type indicating the request is for messgage segment recovery, if the request is targeted to a MSGin5G UE, the MSGin5G Server shall construst a new CoAP POST request to the targeted UE, in the request, the MSGin5G Server:

a)	shall construct the new CoAP POST request with the corresponding value in the recevied CoAP POST request emessage except the Option header; and

b)	shall include the MSGin5G Client address in the Option header of the CoAP message and set the Option header to a corresponding value, e.g. if the MSGin5G Server address is a URI, the Uri-Path Option is set to the value of such URI.

#### 6.5.2.4	Procedures on receiving messgage segments received confirmation to a MSGin5G UE

Upon receiving a CoAP POST request containing the MSGin5G service identifier and containing the Message Type indicating the request is for message segments received confirmation, if the request is targeted to a MSGin5G UE, the MSGin5G Server shall construst a new CoAP POST request to the targeted UE, in the request, the MSGin5G Server:

a)	shall construct the new CoAP POST request with the corresponding value in the recevied CoAP POST request message except the Option header; and

b)	shall include the MSGin5G Client address in the Option header of the CoAP message and set the Option header to a corresponding value, e.g. if the MSGin5G Server address is a URI, the Uri-Path Option is set to the value of such URI.

* * * Next Change * * * *

* * * Next Change * * * *

<Proposed change in revision marks>