3GPP TSG-CT WG1 Meeting #133-e	C1-216975_revdraft1

- E-meeting, 11-19 November 2021

Source:	Huawei, HiSilicon

Title:	MSGin5G UE Configuration procedures

Spec:	3GPP TS 24.538 v0.1.0

Agenda item:	17.2.30

Document for:	Agreement

1. Introduction

This pCR is to propose MSGin5G UE Configuration procedures.

2. Reason for Change

As specified in TS 23.554, the MSGin5G UE Configuration is based on the configuration management functionality specified in TS 23.434.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.538 v0.1.0.

* * * First Change * * * *

## 6.2	Configuration

### 6.2.1	MSGin5G UE Configuration

MSGin5G UE Configuration is based on the Configuration management functionality specified in TS 23.434 [3] and TS 24.546 [y1].

* * * Next Change * * * *

#### 6.2.1.1	Procedure at MSGin5G UE

The MSGin5G UE should support the Configuration management client functionality as specified in 3GPP TS 23.546 [y1] in or outside the MSGin5G Client.The configuration management client may be collocated with MSGin5G Client or it can be separate as per 3GPP TS 23.554 [2].

If the Configuration management client function is not collocated with the MSGin5G client, the MSGin5G Client should use SEAL-C interface to interact with Configuration management client function for MSGin5G UE configuration.

The MSGin5G UE configuration procedures at the Configuration management client function is based on the procedures in clause 6.2.3.1 of 3GPP TS 24.546 [y1], in the procedures:

a)	the Configuration management client function on the MSGin5G UE acts as SCM-C;

b)	the Configuration management server function at the server-side acts as SCM-S;

c)	in the Request-URI, the "XCAP Root" is the URI of Configuration management server function;

d)	in the Request-URI, the "auid" is the unique service identifier of MSGin5G service; and

e)	the parameters serialized into a JavaScript Object Notation (JSON) structure includes:

1)	MSGin5G UE ID as specified in TS 23.554 [2]; and

2)	optionally, other related informations (e.g. device type, device vendor, etc).

Upon receiving the requested MSGin5G UE configuration data, the Configuration management client function shall submit the configuration data to MSGin5G Client by SEAL-C interface. The MSGin5G Client shall store the configuration data, including MSGin5G UE Service ID, the address of MSGin5G Server and other available MSGin5G Service specific informations.

* * * Next Change * * * *

#### 6.2.1.2	Procedure at MSGin5G Server

The Configuration management server functionality as specified in 3GPP TS 23.546 [y1] may be collocated with MSGin5G Server or it can be separate as per 3GPP TS 23.554 [2].

The MSGin5G UE configuration procedures at the Configuration management server function is based on the procedures in clause 6.2.3.2 of 3GPP TS 24.546 [y1], in the procedures, the configuration management server function act as SCM-S.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2;".

[3]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals".

[y1] 	3GPP TS 24.546: "Configuration management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".