3GPP TSG-CT WG1 Meeting #133-e	C1-21xxxx

- E-meeting, 11-19 November 2021											Revision of C1-216824

Source:	China Mobile

Title:	Update to Solution#3

Spec:	3GPP TR 23.700-10 v1.1.0

Agenda item:	17.3.3

Document for:	Agreement

1. Introduction

The solution#3 provides separate sub-solutions for different cases. The following clarifications are suggested:

- The impacts on URSP, not the PCF functionality;

- The description of the information of IMS subscription;

- The applications in Case#1 are third party applications to 5G network.

- The DNN may be used as a traffic descriptor in the URSP for Case#1;and

- The information of IMS service is mentioned only to address combined scenario 1 and scenario 2.

In addition, a new case the information of IMS subscription provided by both application and 5G network is added.

2. Reason for Change

The above clarifications are suggested to make the descriptions of the cases clear.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-10 v1.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.502: "Procedures for the 5G System".

[3]	3GPP TR 23.700-11: "Study on enhanced IP Multimedia Subsystem (IMS) to 5GC integration Phase 2; CT WG3 aspects".

[x]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[y]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

* * * Next Change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.503 [x] apply:

UE local configuration

* * * Next Change * * * *

## 6.3	Solution 3: Selecting an appropriate slice by IMS subscription

### 6.3.1	Description

This solution addresses Key Issue #1 for scenario 1.

In scenario 1, the IMS traffic of the UE to the separate IMS networks is routed to different 5GC network slices. For the separate IMS networks, the UE has information of IMS subscriptions (i.e. public and private user identities, home network domain name, used for IMS registration.) retrieved from ISIM, derived from USIM, configured in the ME , provided by third party application servers or the 5GC network.

NOTE 1:	 Home network domain name identifies the IMS network to which the IMS subscriber belongs.

Case#1: The information of IMS subscriptions is provided by third party application servers for the UE to connect to third party IMS networks.

In this case, each third party has its own IMS network to provide third party IMS services. The specific application provides the UE the information of IMS subscription for the UE to connect to the corresponding IMS network.

The URSP or UE local configuration can be used to indicate the UE appropriate slice to route the IMS traffic of the specific application to the separate IMS network. The current traffic descriptors (e.g. DNN) in the URSP can be reused to match the IMS traffic of the specific application.

Case#2: The information of IMS subscriptions is retrieved from ISIM or configured in the ME for the UE to connect to operator’s IMS networks

In this case, an operator has multiple IMS networks to provide its diverse IMS services. The traffic of these IMS services are usually routed to IMS networks via the default slice and default IMS DNN configured on the UE.

The USRP with home network domain name as a new traffic descriptor can indicate the UE an appropriate slice to route the IMS traffic of an IMS network identified by the home network domain name if each of the IMS clients associated with the IMS subscriptions provide a home network domain name.

The examples of URSP rules with a new traffic descriptor for Case#2 are as follows:

URSP rule 1:

-	Traffic Descriptor:

-	[new Type for the information of IMS network]: IMS home network domain name x

-	Route Selection Descriptor:

-	Network Slice Selection: S-NSSAI 1 (dedicated slice for IMS network 1)

-	DNN selection: IMS DNN

URSP rule 2:

-	Traffic Descriptor:

-	[new Type for the information of IMS network]:  IMS home network domain name y

-	Route Selection Descriptor:

-	Network Slice Selection: S-NSSAI 2 (dedicated slice for IMS network 2)

-	DNN selection: IMS DNN

Case#3: The information of IMS subscriptions is provided by both the third party application servers and 5GC network for the UE to connect to operator’s IMS networks.

In this case, a specific application server provides user identities without a home network domain name, while the operator provides the UE the home network network domain name of its IMS network serving the application.

URSP and UE Local Configuration of UE policy can be used to provide an appropriate slice to route the IMS traffic of a specific application, as described in the Case#1.

In addition, URSP can be used by the operator to indicate the home network domain name for the specific application to the corresponding IMS client in the UE. In this way, the third party need not configure home network domain names of different operator's IMS networks. The home network domain name is included in the route selection descriptor of a URSP rule.

Editor's note:	It is not clear how the traffic descriptors can be used to different IMS clients in the UE. For example, if there are two URSP rules, namely URSP rule X (associated with an RSD including IMS home network domain name A) and URSP rule Y (associated with with an RSD including IMS home network domain name B), the traffic descriptor of URSP rule X should be identifying the IMS client for the IMS network with IMS home network domain name A and the traffic desctiptor of URSP rule Y should be identifying the IMS client for the IMS network with IMS home network domain name B.

### 6.3.2	Impacts on existing nodes and functions

For the cases #2 and #3, the PCF and UE need to be updated to accommodate new added fields in the traffic descriptor and route selection descriptor, respectively.

Editor's note:	Impact analysis on Case#3 is incomplete.

6.3.3	Solution Evaluation

Editor's note:	This clause provides an evaluation of the solution.