3GPP TSG-CT WG1 Meeting #133-e	C1-21xxxx

- E-meeting, 11-19 November 2021											Revision of C1-216824

Source:	China Mobile

Title:	Update to Solution#3

Spec:	3GPP TR 23.700-10 v1.1.0

Agenda item:	17.3.3

Document for:	Agreement

1. Introduction

The solution#3 provides separate sub-solutions for different cases. The following clarifications are suggested:

- The impacts on URSP, not the PCF functionality;

- The description of the information of IMS subscription;

- The applications in Case#1 are third party applications to 5G network.

- The DNN may be used as a traffic descriptor in the URSP for Case#1;and

- The information of IMS service is mentioned only to address combined scenario 1 and scenario 2.

In addition, a new case the information of IMS subscription provided by both application and 5G network is added.

2. Reason for Change

The above clarifications are suggested to make the descriptions of the cases clear.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-10 v1.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.502: "Procedures for the 5G System".

[3]	3GPP TR 23.700-11: "Study on enhanced IP Multimedia Subsystem (IMS) to 5GC integration Phase 2; CT WG3 aspects".

[y]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

* * * Next Change * * * *

## 6.3	Solution 3: Selecting an appropriate slice by IMS subscription

### 6.3.1	Description

This solution addresses Key Issue #1 for scenario 1.

In scenario 1, the IMS traffic of the UE to the separate IMS networks is routed to different 5GC network slices. For the separate IMS networks, the UE has information of IMS subscriptions (i.e. public and private user identities, home network domain name, used for IMS registration.) retrieved from ISIM, derived from USIM, configured in the ME , provided by third party application servers or the 5GC network.

In order for each of IMS clients in the UE to access an IMS network via a desired 5GC network slice, a PDU session associated with the desired 5GC network slice needs to be established. 3GPP TS 24.526 [y] describes how PDU session attributes (e.g., S-NSSAI) is determined based on information provided by an application in the UE (i.e., an IMS client in this case).

According to the encoding of traffic descriptor in URSP, the information which can be provided by an application in the UE is defined as follows:

-	OS Id + OS App Id type

-	IPv4 remote address type

-	IPv6 remote address/prefix length type

-	Protocol identifier/next header type

-	Single remote port type

-	Remote port range type

-	IP 3 tuple type

-	Security parameter index type

-	Type of service/traffic class type

-	Flow label type

-	Destination MAC address type

-	802.1Q C-TAG VID type

-	802.1Q S-TAG VID type

-	802.1Q C-TAG PCP/DEI type

-	802.1Q S-TAG PCP/DEI type

-	Ethertype type

-	DNN type

-	Connection capabilities type

-	Destination FQDN

-	Regular expression

-	OS App Id type

-	Destination MAC address range type

In some cases, IMS clients in a UE:

-	cannot provide distinguished information regarding the parameters listed above ; and

-	can provide different IMS home network domain names;

where a new parameter "IMS home network domain name" should be introduced in traffic descriptor.

6.3.2	Impacts on existing nodes and functions

The PCF and UE need to be updated to accommodate the newly added parameter in the traffic descriptor.

6.3.3	Solution Evaluation

The solution allows a UE to assign appropriate S-NSSAIs to PDU sessions providing different IMS networks for IMS clients in the UE if the IMS clients in the UE provide the same existing traffic descriptor parameters and different IMS home network domain names.