3GPP TSG-CT WG1 Meeting #137-e	C1-225249

# E-Meeting, 18th – 26th August 2022

# Title:	Reply LS on AS-NAS layer interactions for MBS

# Response to:	LS (C1-224524/R2-2206629) on LS on AS-NAS layer interactions for MBS from RAN2

# Release:	Rel-17

# Work Item:	5MBS

Source:	3GPP CT WG1

To:	3GPP RAN WG2, 3GPP SA WG2

Cc:	-

Contact Person:

Name:	Christian Herrero-Veron

E-mail Address:	Christian.Herrero at Huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

1. Overall Description:

CT1 thanks RAN2 for their LS on AS-NAS layer interactions for MBS and CT1 would like to provide the following information:

1. CT1 confirms that the TMGI is needed in the NAS.

2. 	CT1 confirms that indication of establishment of user plane resources is needed in the NAS for multicast MBS sessions. Upper layers (i.e., the related application) may use this indication towards the user.

3. CT1 confirms that indication of release of user plane resources is needed in the NAS for multicast MBS sessions. Upper layers (i.e., the related application) may use this indication towards the user.

CT1 does not see the need of more indications at this moment in time.

2. Actions:

To 3GPP RAN WG2, 3GPP SA WG2 group.

ACTION: 	CT1 would like to request RAN2, RAN2 to take the information provided in this LS into account.

3. Date of Next CT1 Meetings:

CT1#138e	10th – 14th October 2022	e-meeting

CT1#139	14th – 18th November 2022	TBD