3GPP TSG-CT WG1 Meeting #141-e	C1-232426

# E-Meeting, 17th – 21st April 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.380 | CR | 0342 | rev | - | Current version: | 14.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction and clarification to floor control SSRC in MCPTT – media plane |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Research America |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCPTT-CT |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The SSRC of granted floor participant is sent in the both Floor Granted message and Floor Taken message. If the floor is granted for implicit floor request as a part of call initialization then the SSRC of granted floor participant value needs to be aware of before populating the field. This SSRC field value should be same as that of the SSRC of the floor participant in the Floor Control messages to identify the sender of the Floor Control messages. In this CR, a unqiue value of SSRC’s are generated and assigned to each floor participant are communicated during call establishment. This value will be used in the Floor Control messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New "mc_floorp_ssrc" fmtp attribute is introduced to provide the SSRC value generated by the floor control server to be used by the floor participants in Floor Control messages. New SSRC of floor participant field is introduced for the MCPC connect message to carry the the SSRC value generated by the floor control server to be used by the floor participants in Floor Control messages when using pre-established session. The state machine is updated to manage the SSRC value distribution. The Connect message structure is updated to include the newly introduced field. The Semantics and Syntax are added for the newly added “mc_floorp_ssrc" fmtp attribute |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be clear which value to be used in the Floor Control messages for the floor participant identifier (SSRC) and can result into implementation errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.3.1, 8.3.4, 9.2.2.3.2, 9.2.2.4.2, 9.3.2.3.3, 9.3.2.3.5, 9.3.2.4.10, 12.1.2.2, 12.1.2.3, new (8.3.3.A, 14.2.X, 14.3.Y) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.3.3.1	Introduction

This subclause describes the pre-established session call control specific data fields.

The pre-established session call control specific data fields are contained in the application-dependent data of the pre-established session call control message. The pre-established session call control specific data fields follow the syntax specified in subclause 8.1.3.

Table 8.3.3.1-1: Void

Table 8.3.3.1-2 lists the available data fields including the assigned field ID.

Table 8.3.3.1-2: Pre-established session call control data fields

| Field name | Field ID |  | Reference |
| --- | --- | --- | --- |
|  | Decimal | Binary |  |
| Media Streams | 000 | 00000000 | Subclause 8.3.3.2 |
| MCPTT Session Identity | 001 | 00000001 | Subclause 8.3.3.3 |
| Warning Text | 002 | 00000010 | Subclause 8.3.3.4 |
| MCPTT Group Identity | 003 | 00000011 | Subclause 8.3.3.5 |
| Answer State | 004 | 00000100 | Subclause 8.3.3.6 |
| Inviting MCPTT User Identity | 005 | 00000101 | Subclause 8.3.3.7 |
| Reason Code | 006 | 00000110 | Subclause 8.3.3.8 |
| SSRC of floor participant | 008 | 00001000 | Clause 8.3.3.A |
| PCK I_MESSAGE | 192 | 11000000 | Subclause 8.3.3.10 |

The following subclauses describe the coding of each data field.

* * * Next Change * * * *

#### 8.3.3.A	SSRC of floor participant field

The content of the SSRC of floor participant field is coded as specified in IETF RFC 3550 [3]. An SSRC of floor participant field can also have a Field ID and a length value. This clause specifies an SSRC of floor participant field including a Field ID and a length value.

Table 8.3.3.A-1: SSRC of floor participant field coding

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|SSRC of floor  |SSRC of floor  |SSRC value of floor            |

|participant    |participant    |participant                    |

|field ID value |length value   |                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|          SSRC value of floor  |Spare                          |

|          participant                                          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

The <SSRC of floor participant field ID> value is a binary value and is set according to table 8.3.3.1-2.

The <SSRC of floor participant length> value is a binary value and has the value '6' indicating the total length in octets of the <SSRC of floor participant length> value item and the spare bits.

The <SSRC of floor participant> value is coded as the SSRC specified in IETF RFC 3550 [3].

The spare bits are set to zero.

* * * Next Change * * * *

### 8.3.4	Connect message

The Connect message is sent by the participating MCPTT function on the originating side to the MCPTT client to confirm the establishment of an MCPTT call or sent on the terminating side to initiate an MCPTT call. The Connect message is only used in the on-network mode and only sent over the unicast bearer.

Table 8.3.4-1 shows the content of the Connect message.

Table 8.3.4-1: Connect message

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=2|P| Subtype |   PT=APP=204  |          Length               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|               SSRC of participating MCPTT function            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                          name=MCPC                            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   MCPTT Session Identity field                |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                    MCPTT Group Identity field                 |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                      Media Streams field                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                      Warning Text field                       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                      Answer State field                       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                 Inviting MCPTT User Identity field            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                       PCK I_MESSAGE field                     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                 SSRC of floor participant field               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

With the exception of the three first 32-bit words the order of the fields are irrelevant.

Subtype:

The subtype shall be coded according to table 8.3.2-1.

Length:

The length shall be coded as specified in to subclause 8.1.2.

SSRC:

The SSRC field shall carry the SSRC of the participating MCPTT function.

The SSRC field shall be coded as specified in IETF RFC 3550 [3].

MCPTT Session Identity:

The MCPTT Session Identity field is coded as described in subclause 8.3.3.3.

MCPTT Group Identity:

The MCPTT Group Identity field is coded as described in subclause 8.3.3.5.

Media Streams:

The Media Streams field is coded as described in subclause 8.3.3.2.

Warning Text:

The Warning Text field is coded as described in subclause 8.3.3.4.

Answer State:

The Answer State field is coded as described in subclause 8.3.3.6.

When the Answer State field is not included the value "confirmed" shall be assumed.

Inviting MCPTT User Identity:

The Inviting MCPTT User Identity field is coded as described in subclause 8.3.3.5.

When the inviting MCPTT user requested privacy, the <sip:anonymous@invalid.invalid> identity shall be used.

PCK I_MESSAGE:

The PCK I_MESSAGE is used to transport the PCK and PCK-ID for use in private call.

This field is used when the terminating participating MCPTT function sends the Connect message to the terminating client for a private call.

SSRC of floor participant:

The SSRC of floor participant field carries the SSRC value assigned by the floor control server to be used in the Floor Control messages by the receiving floor participant.

The SSRC of floor participant field is coded as described in clause 8.3.3.A.

* * * Next Change * * * *

##### 9.2.2.3.2	Receive Connect message (R: Connect)

Upon reception of a Connect message:

1.	if the MCPTT client accepts the incoming call the MCPTT client:

a.	shall send the Acknowledgement message with Reason Code field set to 'Accepted';

b.	shall use only the media streams of the pre-established session which are indicated as used in the associated call session Media Streams field, if the Connect contains a Media Streams field;

c.	shall store the SSRC of floor participant and use it in the Floor Control messages to indicate the SSRC of floor participant who is sending the Floor Control message;

d.	shall create an instance of the 'Floor participant state transition diagram for basic operation' as specified in clause 6.2.4; and

e. shall enter the 'U: Pre-established session in use' state; or

2.	Otherwise the MCPTT client:

a.	shall send the Acknowledgement message with the Reason Code field set to 'Busy' or 'Not Accepted'; and

b.	shall remain in 'U: Pre-established session not in use' state.

* * * Next Change * * * *

##### 9.2.2.4.2	Receive Connect message (R: Connect)

Upon reception of a Connect message the MCPTT client:

1.	if the first bit in the subtype of the Connect message is set to '1' (acknowledgement is required), shall send Acknowledgement message with the Reason Code field set to 'Accepted';

2.	shall store the SSRC of floor participant and use it in the Floor Control messages to indicate the SSRC of floor participant who is sending the Floor Control message; and

3.	shall remain in the 'U: Pre-established session in use' state.

* * * Next Change * * * *

##### 9.3.2.3.3	Receive SIP INVITE request (R: SIP INVITE)

Upon receiving a SIP INVITE request from the controlling MCPTT function, if in automatic answer mode, the participating MCPTT function:

1.	shall send the Connect message to the invited MCPTT client using the media plane control transport channel defined for this call. The Connect message:

a.	shall include the SIP URI received in the Contact header field of the SIP INVITE request in the <MCPTT Session Identity> value in the MCPTT Session Identity field;

b.	if the SIP INVITE request included an "application/vnd.3gpp.mcptt-info+xml" MIME body with the <session-type> element set to a value of "prearranged" or "chat":

i.	shall include the content of the <mcptt-calling-group-id> element in the MCPTT Group Identity field;

ii.	shall include the value 'prearranged' or 'chat' in the <Session Type> value in the MCPTT Session Identity field;

iii.	shall include the content of the <mcptt-calling-user-id> element in the Inviting MCPTT User Identity field if available from the SIP INVITE message and privacy of the inviting MCPTT ID is not required; and

iv.	shall include 'anonymous@anonymous.invalid' in the Inviting MCPTT User Identity field if the MCPTT ID of the inviting MCPTT user is not available from the SIP INVITE message or privacy of the inviting MCPTT ID is required;

c.	if the SIP INVITE request includes an "application/vnd.3gpp.mcptt-info+xml" MIME body with the <session-type> element set to a value of "private":

i.	shall include the content of the <mcptt-calling-user-id> element in the Inviting MCPTT User Identity field if available from the SIP INVITE message and privacy of the inviting MCPTT ID is not required;

ii.	shall include 'anonymous@anonymous.invalid' in the Inviting MCPTT User Identity field if the MCPTT ID of the inviting MCPTT user is not available from the SIP INVITE message or privacy of the inviting MCPTT ID is required;

iii.	shall include the value 'private' in the <Session Type> value in the MCPTT Session Identity field; and

iv.	if the MIKEY-SAKKE I_MESSAGE is included in the received SIP INVITE request, shall include the MIKEY-SAKKE I_MESSAGE as received, in the PCK I_MESSAGE field;

d.	shall include a Media Streams field, if the negotiated SDP in the pre-arranged session contains more media streams than required by the SDP in the SIP INVITE request;

NOTE 1:	For example if the call is a private call without floor control the Media Streams field needs to be included with the <Control Channel> value set to '0'.

e.	shall include the unique SSRC identifier of an invited MCPTT user received in the "mc_floorp_ssrc" fmtp attribute of the SDP offer of the SIP INVITE request in the <SSRC of floor participant> value in the SSRC of floor participant field field; and

f.	shall set the first bit in the subtype of the Connect message to '1' (Acknowledgment is required) as described in subclause 8.3.2;

2.	shall start timer T55 (Connect);

3.	shall initialize counter C55 (Connect) to 1;

4.	shall reserve the media channel resources if not yet reserved; and

5.	shall enter the 'G: Pre-established session in use' state.

NOTE 2: The case of manual answer is described in 3GPP TS 24.379 [2].

* * * Next Change * * * *

##### 9.3.2.3.5	Receive SIP 200 (OK) response to the SIP re-INVITE request (R: 200 OK)

Upon receiving the SIP 200 (OK) response to the SIP re-INVITE request of the pre-established session a call as specified in 3GPP TS 24.379 [2] (call setup with manual answer with pre-established session) the participating MCPTT function:

1.	shall send the Connect message to the invited MCPTT client(s) using the media plane control transport channel defined for this call. The Connect message:

a.	shall include the SIP URI received in the Contact header field of the SIP INVITE request in the <MCPTT Session Identity> value in the MCPTT Session Identity field;

b.	if the SIP INVITE request included an "application/vnd.3gpp.mcptt-info+xml" MIME body with the <session-type> element set to a value of "prearranged" or "chat":

i.	shall include the content of the <mcptt-calling-group-id> element in the MCPTT Group Identity field; and

ii.	shall include the value 'prearranged' or 'chat' in the <Session Type> value in the MCPTT Session Identity field;

c.	if the SIP re-INVITE request included an "application/vnd.3gpp.mcptt-info+xml" MIME body with the <session-type> element set to a value of "private":

i.	may include the content of the <mcptt-calling-group-id> element in the Inviting MCPTT User Identity field, if available from the SIP re-INVITE message and privacy of the inviting MCPTT ID is not required;

ii.	may include 'anonymous@anonymous.invalid' in the Inviting MCPTT User Identity field if the MCPTT ID of the inviting MCPTT user is not available from the SIP re-INVITE message or privacy of the inviting MCPTT ID is required; and

iii.	shall include the value 'private' in the <Session Type> value in the MCPTT Session Identity field;

d.	shall include a Media Streams field, if the negotiated SDP in the pre-arranged session contains more media streams than required by the SDP in the SIP INVITE request;

NOTE:	For example if the call is a private call without floor control the Media Streams field needs to be included with the <Control Channel> value set to '0'.

e.	shall include the unique SSRC identifier of an invited MCPTT user received in the "mc_floorp_ssrc" fmtp attribute of the SDP offer of the SIP INVITE request in the <SSRC of floor participant> value in the SSRC of floor participant field field; and

f.	may set the first bit in the subtype of the Connect message to '1' (Acknowledgment is required) as described in subclause 8.3.2;

2.	shall start timer T55 (Connect);

3.	shall initialise counter C55 (Connect) to 1;

4.	shall reserve the media channel resources if not yet reserved; and

5.	shall enter the 'G: Pre-established session in use' state.

* * * Next Change * * * *

##### 9.3.2.4.10	Receive SIP 200 (OK) response (R: 200 OK)

Upon receiving a SIP 200 (OK) response from the controlling MCPTT function, the participating MCPTT function:

1.	shall send the Connect message to the MCPTT client which initiated this call using the media plane control transport channel defined for this call. The Connect message:

a.	shall include the SIP URI received in the Contact header field of the SIP 200 (OK) response in the <MCPTT Session Identity> value in the MCPTT Session Identity field;

b.	if an "application/vnd.3gpp.mcptt-info+xml" MIME body with an <session-type> element is included in the SIP 200 (OK) response, shall include the <Session Type> value in the MCPTT Session Identity field with a value corresponding to the <session-type> element;

c.	if an "application/vnd.3gpp.mcptt-info+xml" MIME body with an <session-type> element is not included in the SIP 200 (OK) response, shall include the <Session Type> value set to 'no session type' in the MCPTT Session Identity field;

d.	if a Warning header field with set to the value "399" is received in the SIP 200 (OK) response, shall include the content of the "warn-text" parameter in the Warn Text header field;

e.	if a P-Answer-State header field is included in the SIP 200 (OK) response, shall include an Answer State field with a value corresponding to the "answer-type" parameter;

f	should set the first bit in the subtype of the Connect message to '1' (Acknowledgment is required) as described in subclause 8.3.2; and

g.	shall include the unique SSRC identifier of an inviting MCPTT user received in the "mc_floorp_ssrc" fmtp attribute of the SDP answer of the SIP 200 (OK) response in the <SSRC of floor participant> value in the SSRC of floor participant field field;

2.	shall start timer T55 (Connect);

3.	shall increment counter C55 (Connect) by 1; and,

4.	shall remain in the 'G: Pre-established session in use' state.

* * * Next Change * * * *

#### 12.1.2.2	Semantics

In an SDP offer and answer, the "mc_queueing" fmtp attribute is used to indicate support of the Floor Request message queueing mechanism, as defined in the present specification.

In an SDP offer, the "mc_priority" fmtp attribute indicates (using an integer value between '1' and '255') the maximum floor priority that the offerer requests to be used with Floor Request messages sent by the offerer. In an SDP answer, the attribute parameter indicates the maximum priority level that the answerer has granted to the offerer. The value must be equal or less than the value provided in the associated SDP offer.

NOTE 1: If the "mc_priority" fmtp attribute is not used within an SDP offer or answer, a default priority value is assumed.

In an SDP offer, the "mc_granted" fmtp attribute parameter indicates that the offerer supports the procedure where the answerer indicates, using the fmtp attribute in the associated SDP answer, that the floor has been granted to the offerer.

NOTE 2: When the "mc_granted" fmtp attribute is used in an SDP offer, it does not indicate an actual request for the floor. The SDP "mc_implicit_request" fmtp attribute can be used to request the floor. In an SDP answer, the attribute indicates that the floor has been granted to the offerer.

NOTE 3: Once the offerer has been granted the floor, the offerer has the floor until it receives a Floor Revoke message, or until the offerer itself releases the floor by sending a Floor Release message, as described in the present specification.

In an SDP offer, the "mc_implicit_request" fmtp attribute indicates that the offerer implicitly requests the floor (without the need to send a Floor Request message). In an SDP answer, the attribute parameter indicates that the answerer has accepted the implicit floor request. Once the answerer grants the floor to the offerer, the answerer will send a Floor Granted message.

NOTE 4: The usage of the "mc_implicit_request" fmtp attribute in an SDP answer does not mean that the answerer has granted the floor to the offerer, only that the answerer has accepted the implicit floor request.

In an SDP answer, the "mc_ssrc" fmtp attribute indicates ssrc value of the offerer. The value may be equal to the value provided in the associated SDP offer or different if a collision is detected. If the associated SDP offer does not indicate the ssrc value then server can determine an appropriate value. This value is returned in an SDP answer only if the answerer accepts the implicit floor request offered in SDP.

In an SDP offer, the "mc_no_floor_ctrl" fmtp attribute indicates that the offerer wants to use pre-established session call control during the session without floor control over the offered media plane control channel. In an SDP answer, the attribute parameter indicates that the answerer has accepted the request to use the offered media plane control channel for pre-established session call control during the session without floor control.

In an SDP offer, the "mc_floorp_ssrc" fmtp attribute indicates the ssrc of the floor participant (server generated and assigned ssrc value to be used) that the offerer requests to be used with Floor Control messages by the receiving floor participant. In SDP answer, the "mc_floorp_ssrc" fmtp attribute indicates the ssrc of the floor participant (server generated and assigned ssrc value to be used) that the answerer requests to be used with Floor Control messages by the receiving floor participant.

* * * Next Change * * * *

#### 12.1.2.3	Syntax

Table 12.1.2.3-1: SDP "fmtp" attribute for the MCPTT media plane control channel

fmtp-attr-mpcp      =  "a=fmtp:" "MCPTT" SP attr-param-list

attr-param-list     =  attr-param *(COLON attr-param)

attr-param          =  mc_queueing / mc_priority / mc_granted / mc_implicit_request / mc_ssrc / mc_no_floor_ctrl / mc_floorp_ssrc

mc_queueing         =  "mc_queueing"

mc_priority         =  "mc_priority=" 1*2(DIGIT)

mc_granted          =  "mc_granted"

mc_implicit_request =  "mc_implicit_request"

mc_ssrc		        =  "mc_ssrc=" 1*(DIGIT)

mc_no_floor_ctrl    =  "mc_no_floor_ctrl"

mc_floorp_ssrc      =  "mc_floorp_ssrc=" 1*(DIGIT)

* * * Next Change * * * *

### 14.2.X	"mc_floorp_ssrc" fmtp attribute

When inviting an MCPTT user to an MCPTT session, the MCPTT server shall assign unique SSRC identifier to be used in the media floor control messages by an invited MCPTT user and shall include in the "mc_floorp_ssrc" fmtp attribute with the assigned unique SSRC identifier of an invited MCPTT user in the SDP offer.

* * * Next Change * * * *

### 14.3.Y	"mc_floorp_ssrc" fmtp attribute

The MCPTT server shall assign unique SSRC identifier to be used in the media floor control messages by an inviting MCPTT user of an MCPTT session and shall include in the "mc_floorp_ssrc" fmtp attribute with the assigned unique SSRC identifier of an inviting MCPTT user in the SDP answer.

* * * End of Changes * * * *