3GPP TSG-CT WG1 Meeting #146	C1-240abc

# Online, 22– 26 January 2024

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on CoAP definition of the Sdd_RegularTransmissionConnection API

Spec:	3GPP TS 24.543 v1.0.0

Agenda item:	18.2.16

Document for:	Agreement

1. Reason for Change

The current specification misses to define the CoAP definition of the Sdd_RegularTransmissionConnection API described by 3GPP TS 23.433. Hence, it is proposed to add the necessary information for the CoAP resource representation and encoding.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 24.543 v1.0.0.

3. Revision history

-

* * * First Change * * * *

## A.3.1	Sdd_RegularTransmissionConnection API

### A.3.1.1	API URI

The CoAP URIs used in CoAP requests from SDDM-S towards the SDMM-C shall have the Resource URI structure as defined in Annex C.1.1 of 3GPP TS 24.546 [6] with the following clarifications:

a)	the <apiName> shall be "sdd-rtc-s";

b)	the <apiVersion> shall be "v1"; and

c)	the <apiSpecificSuffixes> shall be set as described in clause A.3.1.2.

### A.3.1.2	Resources

#### A.3.1.2.1	Overview

Figure A.3.1.2.1.1: Resource URI structure of the Sdd_RegularTransmissionConnection API provided by SDDM-S

Table A.3.1.2.1.1 provides an overview of the resources and applicable CoAP methods.

Table A.3.1.2.1.1: Resources and methods overview

| Resource name | Resource URI | CoAP method | Description |
| --- | --- | --- | --- |
| SDD Regular Transmission Connection | val-services/{valServiceId}/sdd-regular-transmission-connection | POST | Establish an SDDM regular transmission connection. |
|  |  | DELETE | Release an SDDM regular transmission connection |

#### A.3.1.2.2	Resource: SDD Regular Transmission Connection

##### A.3.1.2.2.1	Description

The SDD regular transmission connection resource represents an SDD regular transmission connection to be created at a given SDDM-S and SDDM-C.

##### A.3.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/sdd-rtc-s/<apiVersion>/val-services/{valServiceId}/establishment-request

This resource shall support the resource URI variables defined in the table A.3.1.2.2.2.1.

Table A.3.1.2.2.2.1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause C.1.1 of 3GPP TS 24.546 [6]. |
| apiVersion | string | See clause A.3.1.1. |
| valServiceId | string | Identifier of a VAL service. |

##### A.3.1.2.2.3	Resource Standard Methods

A.3.1.2.2.3.1	POST

This operation allows to establish an SDDM regular transmission connection.

This method shall support the request data structures the data structures, request codes and response codes specified in table A.3.1.2.2.3.1.1 and A.3.1.2.2.3.1.2.

Table A.3.1.2.2.3.1.1: Data structures supported by the POST Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EstablishmentRequest | M | 1 | The information of request of establishment of an SDDM regular transmission connection. |

Table A.3.1.2.2.3.1.2: Data structures supported by the POST Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EstablishmentResponse | M | 1 | 2.01 Created | SDDM regular transmission connection created successfully. |
| NOTE: The mandatory CoAP error status codes for the GET Request listed in table C.1.3-1 of 3GPP TS 24.546 [31] shall also apply. |  |  |  |  |

A.3.1.2.2.3.2	DELETE

This operation releases an SDDM regular transmission connection.

This method shall support the request data structures the data structure, request codes and response codes specified in table A.3.1.2.2.3.2.1 and A.3.1.2.2.3.2.2.

Table A.3.1.2.2.3.2.1: Data structures supported by the DELETE Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ReleaseRequest | M | 1 | The information of request of release of an SDDM regular transmission connection. |

Table A.3.1.2.2.3.2.2: Data structures supported by the DELETE Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 2.02 Deleted | SDDM regular transmission connection released successfully. |
| NOTE: The mandatory CoAP error status codes for the DELETE method listed in table C.1.3-1 of 3GPP TS 24.546 [31] shall also apply. |  |  |  |  |

### A.3.1.3	Data Model

#### A.3.1.3.1	General

Table A.3.1.3.1.1 specifies the data types defined specifically for the SDD_RegularTransmissionConnection API service provided by SDDM-S.

Table A.3.1.3.1.1: SDD_RegularTransmissionConnection API provided by SDDM-S specific data types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |

Editor’s note:	The table is FFS.

Table A.3.1.3.1.2 specifies the simple data types defined specifically for the SDD_RegularTransmissionConnection API service provided by SDDM-S.

Table A.3.1.3.1.2: SDD_RegularTransmissionConnection API provided by SDDM-S specific simple data types

| Data type | Section defined | Description |
| --- | --- | --- |

Editor’s note:	The table is FFS.

Table A.3.1.3.1.3 specifies the enumerations defined specifically for the SDD_RegularTransmissionConnection API service provided by SDDM-S.

Table A.3.1.3.1.3: SDD_RegularTransmissionConnection API provided by SDDM-S specific enumeration

| Data type | Section defined | Description |
| --- | --- | --- |

Editor’s note:	The table is FFS.

#### A.3.1.3.2	Structured data types

#### A.3.1.3.3	Simple data types and enumerations

None.

### A.3.1.4	Error Handling

General error responses are defined in Annex C.1.3 of 3GPP TS 24.546 [6].

### A.3.1.5	CDDL Specification

#### A.3.1.5.1	Introduction

The data model described in clause A.3.1.3 shall be binary encoded in the CBOR format as described in IETF RFC 8949 [17].

Clause A.3.1.5.2 uses the concise data definition language described in IETF RFC 8610 [r8610] and provides corresponding representation of the SDD_RegularTransmissionConnection API provided by SDDM-S data model.

#### A.3.1.5.2	CDDL document

Editor’s note:	The CDDL document is FFS.

### A.3.1.6	Media Types

Editor’s note:	The media types are FFS.

* * * Next Change * * * *

## A.4.1	Sdd_RegularTransmissionConnection API

### A.4.1.1	API URI

The CoAP URIs used in CoAP requests from SDDM-C towards the SDMM-S shall have the Resource URI structure as defined in Annex C.1.1 of 3GPP TS 24.546 [6] with the following clarifications:

a)	the <apiName> shall be "sdd-rtc-c";

b)	the <apiVersion> shall be "v1"; and

c)	the <apiSpecificSuffixes> shall be set as described in clause A.4.1.2.

### A.4.1.2	Resources

#### A.4.1.2.1	Overview

Figure A.4.1.2.1.1: Resource URI structure of the Sdd_RegularTransmissionConnection API provided by SDDM-C

Table A.4.1.2.1.1 provides an overview of the resources and applicable CoAP methods.

Table A.4.1.2.1.1: Resources and methods overview

| Resource name | Resource URI | CoAP method | Description |
| --- | --- | --- | --- |
| SDD Regular Transmission Connection | val-services/{valServiceId}/sdd-regular-transmission-connection | POST | Establish an SDDM regular transmission connection. |
|  |  | DELETE | Release an SDDM regular transmission connection |

#### A.4.1.2.2	Resource: SDD Regular Transmission Connection

##### A.4.1.2.2.1	Description

The SDD regular transmission connection resource represents an SDD regular transmission connection to be created at a given SDDM-C and SDDM-S.

The establishment request resource allows a SDDM-C to request the SDDM-S to establish an SDDM regular transmission.

##### A.4.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/sdd-rtc-c/<apiVersion>/val-services/{valServiceId}/establishment-request

This resource shall support the resource URI variables defined in the table A.4.1.2.2.2.1.

Table A.4.1.2.2.2.1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause C.1.1 of 3GPP TS 24.546 [6]. |
| apiVersion | string | See clause A.4.1.1. |
| valServiceId | string | Identifier of a VAL service. |

##### A.4.1.2.2.3	Resource Standard Methods

A.4.1.2.2.3.1	POST

This operation allows to establish an SDDM regular transmission connection.

This method shall support the request data structures the data structures, request codes and response codes specified in table A.4.1.2.2.3.1.1 and A.4.1.2.2.3.1.2.

Table A.4.1.2.2.3.1.1: Data structures supported by the POST Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EstablishmentRequest | M | 1 | The information of request of establishment of an SDDM regular transmission connection. |

Table A.4.1.2.2.3.1.2: Data structures supported by the POST Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EstablishmentResponse | M | 1 | 2.01 Created | SDDM regular transmission connection created successfully. |
| NOTE: The mandatory CoAP error status codes for the GET Request listed in table C.1.3-1 of 3GPP TS 24.546 [31] shall also apply. |  |  |  |  |

A.4.1.2.2.3.2	DELETE

This operation releases an SDDM regular transmission connection.

This method shall support the request data structures the data structures, request codes and response codes specified in table A.4.1.2.2.3.2.1 and A.4.1.2.2.3.2.2.

Table A.4.1.2.2.3.2.1: Data structures supported by the DELETE Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ReleaseRequest | M | 1 | The information of request of release of an SDDM regular transmission connection. |

Table A.4.1.2.2.3.2.2: Data structures supported by the DELETE Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 2.02 Deleted | SDDM regular transmission connection released successfully. |
| NOTE: The mandatory CoAP error status codes for the DELETE method listed in table C.1.3-1 of 3GPP TS 24.546 [31] shall also apply. |  |  |  |  |

### A.4.1.3	Data Model

#### A.4.1.3.1	General

Table A.4.1.3.1.1 specifies the data types defined specifically for the SDD_RegularTransmissionConnection API service provided by SDDM-C.

Table A.4.1.3.1.1: SDD_RegularTransmissionConnection API provided by SDDM-C specific data types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |

Editor’s note:	The table is FFS.

Table A.4.1.3.1.2 specifies the simple data types defined specifically for the SDD_RegularTransmissionConnection API service provided by SDDM-C.

Table A.4.1.3.1.2: SDD_RegularTransmissionConnection API provided by SDDM-C specific simple data types

| Data type | Section defined | Description |
| --- | --- | --- |

Editor’s note:	The table is FFS.

Table A.4.1.3.1.3 specifies the enumerations defined specifically for the SDD_RegularTransmissionConnection API service provided by SDDM-C.

Table A.4.1.3.1.3: SDD_RegularTransmissionConnection API provided by SDDM-C specific enumeration

| Data type | Section defined | Description |
| --- | --- | --- |

Editor’s note:	The table is FFS.

#### A.4.1.3.2	Structured data types

#### A.4.1.3.3	Simple data types and enumerations

None.

### A.4.1.4	Error Handling

General error responses are defined in Annex C.1.3 of 3GPP TS 24.546 [6].

### A.4.1.5	CDDL Specification

#### A.4.1.5.1	Introduction

The data model described in clause A.4.1.3 shall be binary encoded in the CBOR format as described in IETF RFC 8949 [17].

Clause A.4.1.5.2 uses the concise data definition language described in IETF RFC 8610 [r8610] and provides corresponding representation of the SDD_RegularTransmissionConnection API provided by SDDM-C data model.

#### A.4.1.5.2	CDDL document

Editor’s note:	The CDDL document is FFS.

### A.4.1.6	Media Types

Editor’s note:	The media types are FFS.

* * * End of Changes * * * *