3GPP TSG-CT WG1 Meeting #153	C1-250211

# Athens, Greece, 17-21 February 2025

Source:	Huawei, HiSilicon

Title:	IANA registration requests related to TS 24.543, 24.545, and 24.548

Agenda item:	17.54

Document for:	DISCUSSION

# Introduction

The stage-3 standardization work of the Rel-18 version of the 3GPP specifications was considered frozen by TSG CT#104. Back when the Rel-18 was frozen, CT1 checked whether CT1 specifications require new IANA registration requests from 3GPP, and therefore a number of registration requests were sent to IANA.

This paper provides information about the result of the IANA expert review of registration requests sent because of new MIME types added in TS 24.543 [1], 24.545 [2] and 24.548 [3]. The paper shows that CT1 needs to update the previously mentioned specifications so that appropriate level of granularity for media types is provided following IANA recommendations. After approval of necessary updates, 3GPP needs to proceed with revised registration requests of MIME types.

# 2	IANA expert review of 3GPP requests related to TS 24.543, 24.545, and 24.548

## 2.1	General

Back in 2024, 3GPP sent registration request related to new MIME types added in the Rel-17 (eSEAL WID related) and Rel-18 (SEALDD WID related) versions of CT1 specifications, e.g., TS 24.543 [1], 24.545 [2] and 24.548 [3].

IANA expert review took place and a number of new MIME types were assigned by IANA under the vendor-specific MIME type, i.e., "vnd.3gpp".

However, the IANA expert review indicated that registration request related to TS 24.543 [1], 24.545 [2] and 24.548 [3] are not accepted. The main reason is that the majority of the media types are direct mappings to a data type that is either a CBOR payload or an XML element. Moreover, IANA recommends following a different way, quote:

"Therefore unless an intermediary needs to identify the datatype without parsing the body, then including the type identifier in the media type is redundant.  A mirror structure could be creating in CBOR to identify the contained type using a single root member, or a media type parameter could contain the type in a header".

Note that the goal of the IANA registration process is not only to make it easy to register new media types, but also to provide some sanity checking to make sure the new types are well thought out. Hence, we have found that the 3GPP MIME definitions for TS 24.543 [1], 24.545 [2] and 24.548 [3] are not the most appropriate ones.

## 2.2	Analysis of IANA expert review and proposal

In light of the IANA expert review response to 3GPP, let’s consider TS 24.543 [1] and see how CT1 can amend the new MIME types so that a better and more appropriate MIME type design can be defined.

Considering the Rel-18 version of TS 24.543 [1], one can see that the specification uses a distinct media type for request and response payloads for accessing a specific data type.

Subset of requested media types includes:

- for XML payload "vnd.3gpp.seal-data-delivery-info+xml"; and

- for CBOR payload many are defined, as follows:

vnd.3gpp.seal-data-delivery-data-storage-creation-req-info+cbor

vnd.3gpp.seal-data-delivery-data-storage-creation-res-info+cbor

vnd.3gpp.seal-data-delivery-data-storage-mgt-req-info+cbor

vnd.3gpp.seal-data-delivery-data-storage-query-res-info+cbor

vnd.3gpp.seal-data-delivery-data-storage-reservation-req-info+cbor

vnd.3gpp.seal-data-delivery-data-storage-reservation-res-info+cbor

vnd.3gpp.seal-data-delivery-data-storage-status-notification-info+cbor

vnd.3gpp.seal-data-delivery-establishment-req-info+cbor

vnd.3gpp.seal-data-delivery-establishment-res-info+cbor

vnd.3gpp.seal-data-delivery-measurement-notification-info+cbor

vnd.3gpp.seal-data-delivery-measurement-subscription-req-info+cbor

vnd.3gpp.seal-data-delivery-measurement-subscription-res-info+cbor

vnd.3gpp.seal-data-delivery-release-req-info+cbor

vnd.3gpp.seal-data-delivery-tx-quality-mgt--res-info+cbor

vnd.3gpp.seal-data-delivery-tx-quality-mgt-req-info+cbor

vnd.3gpp.seal-data-delivery-urllc-establishment-req-info+cbor

vnd.3gpp.seal-data-delivery-urllc-establishment-res-info+cbor

vnd.3gpp.seal-data-delivery-urllc-release-req-info+cbor

vnd.3gpp.seal-data-delivery-urllc-update-req-info+cbor

In view of the IANA expert review, doing the above way (of defining MIME types) within a large data model is not the appropriate level of granularity for media types. Note that the current way of defining MIME types results in lots of them and it is not scalable.

A way forward could be to define a structure as proposed by the reply from the IANA expert review. Hence, based on the data model described in the clause A.3.2.3 of TS 24.543 [1], an appropriate design would be a single media type registration with a required parameter to identify the type. For example, for the case of CBOR payload a general and single (root) MIME type can be defined and then this can use a media type parameter to identify a particular data type, e.g., "vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-creation-req" and "vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-creation-res".

The above proposal provides a significantly more manageable way of working with media types as a single one should be registered and at the very same time, it avoids the need to register more and more (new) media types when new types are introduced in a new release of the specification, e.g., the current one; Rel-19. Note that the proposal is in fact easily scalable.

Similarly, for TS 24.545 [2], and 24.548 [3] a new MIME type structure should be created so that for CBOR payload, a single root member, and then a parameter will contain the type in a header as has been previously suggested.

# 3	Conclusion

The authors of this paper provide information about the result of the IANA expert review of registration requests sent because of new MIME types added in TS 24.543 [1], 24.545 [2] and 24.548 [3].

The present paper shows that CT1 needs to update the new MIME types added in TS 24.543 [1], 24.545 [2] and 24.548 [3] considering the information provided by IANA. A proposal is outline so that a single (root) media type registration is required for a particular payload and then a media type parameter is used to identify a data type. An example can be "vnd.3gpp.seal-data-delivery-info+cbor;modeltype=data-storage-creation-req".

The authors of this paper have submitted a number of change requests (CRs) based on the proposal outlined so that if approved, 3GPP can proceed with revised registration request of MIME types. Registration of MIME types is needed for the functionality (APIs) defined by TS 24.543 [1], 24.545 [2] and 24.548 [3] (all SEAL-related protocol specifications) can be adopted by the market.

# References

[1]	3GPP TS 24.543: "Data Delivery Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol Specification

[2]	3GPP TS 24.545: "Location Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol Specification ".

[3]	3GPP TS 24.548: "Network Resource Management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol Specification".