3GPP TSG-CT WG1 Meeting #153	C1-250281

# Athens, Greece, 17-21 February 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0694 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G ProSe multi-hop U2N relay based on Model A discovery: adding hop count |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 23.304 version 19.2.0, for Layer-2 link management over PC5 reference point for 5G ProSe multi-hop UE-to-Network relay based on Model A discovery, hop count for the remote UE is added in the 5G ProSe remote UE info: The link modification procedure is enhanced to inform the parent relay, e.g. another 5G ProSe Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay, of the User Info of the child relay(s) or the Remote UE(s). The Link Modification Request message contains a list of 5G ProSe Remote UE (Remote User ID, Remote UE Info, hop-count) and the corresponding PC5 QoS Flow associated with each 5G ProSe Remote UE. The Remote User ID and Remote UE Info are as defined in clause 6.5.1.1. The hop-count corresponds to the PC5 hops of the Remote UE from the network. This information element is corresponding to the list of multi-hop UE parameters in clause 11.3.58 of TS 24.554. In addition to that, the 5G ProSe multi-hop remote UE needs to trigger the link modification procedure to its 5G ProSe intermediate UE-to-network relay UE after obtaining the IP address/prefix from the 5G ProSe multi-hop UE-to-network relay UE. After that, the intermediate UE-to-network relay UE initiates the link modification procedure to its parent intermediate relay toward the 5G ProSe multi-hop UE-to-network relay UE to provide the list of remote UE information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding step for the 5G ProSe multi-hop remote UE to initiate direct link modification procedure to inform its remote UE context. Removing listing parameters of the List of remote UE parameters IE in the procedure clause and clarifying in 11.3.58. Changing the title of 11.3.58 to have consistent terminology. Adding hop count in the List of remote UE parameters IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe multi-hop UE-to-network relay UE cannot determine the hop count to reach the 5G ProSe multi-hop remote UE being connected, so the link management based on the hop count cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8b.2.2.3, 11.3.58 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**********First change**********

#### 8b.2.2.3	5G ProSe direct link modification procedure for multi-hop UE-to-network relay

The 5G ProSe direct link modification procedure for multi-hop UE-to-network relay reuses the procedure for 5G ProSe direct link modification procedure for UE-to-network relay communication described in clause 7.2.3, with considering the same roles defined in bullet a) and b) of clause 8b.2.2.2.1.

When the 5G ProSe direct link establishment procedure for multi-hop UE-to-network relay is based on model A discovery, after the 5G ProSe multi-hop remote UE establishes the 5G ProSe direct link with the first 5G ProSe intermediate UE-to-network relay UE:

a0)	the 5G ProSe multi-hop remote UE may initiate a 5G ProSe direct link modification procedure for multi-hop UE-to-network relay toward the 5G ProSe intermediate UE-to-network relay UE to inform its remote UE context after obtaining IP address/prefix from the 5G ProSe multi-hop UE-to-network relay UE;

Editor's note (CR0694, 5G_ProSe_Ph3): The information element for remote UE context in the PROSE DIRECT LINK MODIFICATION REQUEST message is FFS.

a)	each 5G ProSe intermediate UE-to-network relay UE, starting from the first 5G ProSe intermediate UE-to-network relay UE, shall initiate a 5G ProSe direct link modification procedure for multi-hop UE-to-network relay towards its 5G ProSe multi-hop parent UE-to-network relay UE, where the PROSE DIRECT LINK MODIFICATION REQUEST message created by each 5G ProSe intermediate UE-to-network relay UE towards its 5G ProSe multi-hop parent UE-to-network relay UE:

1)	shall additionally include the List of remote UE parameters IE and the associated PC5 QoS flow of the 5G ProSe multi-hop remote UEs; and

b)	each 5G ProSe multi-hop parent UE-to-network relay UE shall complete the 5G ProSe direct link modification procedure for multi-hop UE-to-network relay with its 5G ProSe multi-hop child UE-to-network relay UE before triggering the same procedure for the next hop.

Editor's note (CR 0670, 5G_ProSe_Ph3): The details of the associated PC5 QoS flow are FFS.

When the 5G ProSe direct link establishment procedure for multi-hop UE-to-network relay is based on model B discovery, after the 5G ProSe multi-hop remote UE establishes the 5G ProSe direct link with the first 5G ProSe intermediate UE-to-network relay UE:

a)	the 5G ProSe multi-hop remote UE may trigger the 5G ProSe direct link modification procedure for multi-hop UE-to-network relay towards the first 5G ProSe intermediate UE-to-network relay UE to provide PC5 QoS Info and PC5 QoS rule(s);

Editor's note (CR 0670, 5G_ProSe_Ph3): The details of that step including any potential changes in the PROSE DIRECT LINK MODIFICATION REQUEST message to deliver the PC5 QoS Info and PC5 QoS rule(s) are FFS.

b)	then, each 5G ProSe intermediate UE-to-network relay UE, starting from the first 5G ProSe intermediate UE-to-network relay UE, may initiate a 5G ProSe direct link modification procedure for multi-hop UE-to-network relay towards its 5G ProSe multi-hop parent UE-to-network relay UE, where the PROSE DIRECT LINK MODIFICATION REQUEST message created by each 5G ProSe intermediate UE-to-network relay UE towards its 5G ProSe multi-hop parent UE-to-network relay UE:

1)	shall additionally include the split the PC5 QoS; and

Editor's note (CR 0670, 5G_ProSe_Ph3): The details of the split PC5 QoS hop-by-hop and any corresponding updates in the PROSE DIRECT LINK MODIFICATION REQUEST message are FFS.

b)	each 5G ProSe multi-hop parent UE-to-network relay UE shall complete the 5G ProSe direct link modification procedure for multi-hop UE-to-network relay with its 5G ProSe multi-hop child UE-to-network relay UE before triggering the same procedure for the next hop.

**********Second change**********

### 11.3.58	List of multi-hop remote UE parameters

The list of multi-hop remote UE parameters is used to indicate the identity, address information, hop count, and the associated PC5 QoS flow for the 5G ProSe multi-hop remote UE(s).

Editor's note (CR 0671, 5G_ProSe_Ph3):	The naming of this IE and whether other parameters are included in it (such as any parameters related to the 5G ProSe intermediate UE-to-network relay UEs) are FFS.

Editor's note (CR 0671, 5G_ProSe_Ph3):	The details of the associated PC5 QoS flow are FFS. Also based on that, the minimum length of the IE is FFS.

The list of multi-hop UE parameters information element is a type 6 information element with the minimum length of TBD octets and the maximum length of 65538 octets.

The list of multi-hop UE parameters information element is coded as shown in figure 11.3.z.1, figure 11.3.z.2 and table 11.3.z.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| List of multi-hop UE parameters IEI |  |  |  |  |  |  |  | octet 1 |
| Length of multi-hop UE parameters list contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
| Number of remote UE parameters |  |  |  |  |  |  |  | octet 4 |
| Remote UE parameters 1 |  |  |  |  |  |  |  | octet 5 |
|  |  |  |  |  |  |  |  | octet a |
| … |  |  |  |  |  |  |  | octet a+1* octet b* |
| Remote UE parameters k |  |  |  |  |  |  |  | octet b+1* |
|  |  |  |  |  |  |  |  | octet c* |

Figure 11.3.58.1: List of multi-hop UE parameters

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Remote UE context |  |  |  |  |  |  |  | octet 5 |
|  |  |  |  |  |  |  |  | octet d |
| Hop count |  |  |  |  |  |  |  | octet d+1 |
| Remote UE associated PC5 QoS flow |  |  |  |  |  |  |  | octet d+2 |
|  |  |  |  |  |  |  |  | octet a |

Figure 11.3.58.2: Remote UE parameters

Table 11.3.58.1: List of remote UE parameters IE

Remote UE parameters (octet 5 to a)

The contents of remote UE parameters field are applicable for one individual UE and contains the remote UE context and the remote UE associated PC5 QoS flow.

Remote UE context (octet 5 to d)

The remote UE context field contains the identity and address information of the UE and is coded as the remote UE context field specified in figure 9.11.4.29.2 in 3GPP TS 24.501 [11],starting from octet 5 till octet j+k+3.

Hop count (octet d+1)

The hop count field is coded as specified in figure 11.2.21.1 and table 11.2.21.1. It is set to 1 by the first 5G ProSe Multi-hop intermediate UE-to-Network relay UE and is incremented by 1 every time the message is forwarded by a 5G ProSe Multi-hop Intermediate UE-to-Network relay UE(s) toward the 5G ProSe Multi-hop UE-to-network relay UE.

Remote UE associated PC5 QoS flow (octet d+1 to a)

TBD

**********End of change**********