3GPP TSG-CT WG1 Meeting #153	C1-250449

# Athens, Greece, 17-21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.257 | CR | 0051 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NTZ activation procedure for NTZ support over UAE layer |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP_Ph3 |  |  |  |  |  | Date: |  | 2025-02-09 |
| Category: | B |  |  |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  | As per the clause 7.1 and 7.11 of TS 23.255 v19.4.0, UAE layer support for NTZ has the following procedure agreed in Stage-2, which require Stage-3 implementation: - Registration procedure update with NTZ capability - Management of NTZ support configuration - NTZ configuration - NTZ activation by UAE client - UAE-layer/SEAL/LMD assisted NTZ enforcement - New USS NTZ policy |  |  |  |  |  |  |
| Summary of change: |  |  | This CR proposes the following: - UAE layer support for NTZ activation by UAE client |  |  |  |  |  |  |
| Consequences if not approved: |  |  | Support for NTZ not specified in stage 3. |  |  |  |  |  |  |
| Clauses affected: |  | 6.12.1.2 (new) |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

#### 6.12.1.2	NTZ activation by UAE client procedure

Upon detection of approaching area for deactivating transmission based on their NTZ configuration, the UAE-C shall generate an HTTP POST request message according to IETF RFC 9110 [5]. In the HTTP POST request message, the UAE-C:

a)	shall set the Request-URI to the URI corresponding to the identity of the UAE-S;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.uae-info+xml";

c)	shall include an application/vnd.3gpp.uae-info+xml MIME body with a <NTZ-notification> element in the <UAE-info> root element which:

1)	shall include a <UAS-id> element set to the identifier of the UAS, a list of UAV or UAV-C identifier or both or a group identifier, for which the NTZ notification applies;

2)	shall include a <NTZ-status> element indicating NTZ enforcement status e.g. "NTZ imminent entry" or "NTZ exit";

3)	shall include a <NTZ-transmission-information> element indicating whether the transmission is switched on or off and includes the frequency band information;

4)	shall include a <time> element set as date and time (with an offset from the UTC) of the instance when the notification is sent;

5)	shall include a <location-information> element set to location and altitude, i.e., 3-dimentional geographical coordinates, of the UAE client when the notification is sent; and

6)	may include a <NTZ-enforcement-time> element indicating the presence of NTZ enforcement time information;

i)	may include a <estimated-start-time> element set as date and time (with an offset from the UTC) of the instance when the NTZ enforcement is expected to start;

ii)	may include a <estimated-completion-time> element set as date and time (with an offset from the UTC) of the instance when the NTZ enforcement is expected to end; and

iii) may include a <actual-completion-time> element set as date and time (with an offset from the UTC) of the instance when the NTZ enforcement has ended; and

d)	shall send the HTTP POST request towards the UAE-S.

* * * * End of changes * * * *