3GPP TSG-CT WG1 Meeting #153	C1-250573

# Athens, Greece, 17-21 February 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.502 | CR | 0318 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting wrong clause number and wrong reference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | Non3GPPMob_Sec |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The newly introduced clause due to the agreed CR0313 in C1-247164 (in CT1#152) is still numbered as 7.3A.x after implementing the CR in the spec. This shall be corrected to be 7.3A.7. Also same correction is needed in the reference that refers to this clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting the wrong clause number and wrong reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong clause number and reference stay in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.1, 7.3A.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

### 7.3A.1	General

A trusted non-3GPP access network (TNAN) includes a trusted non-3GPP access point (TNAP) and a trusted non-3GPP gateway function (TNGF). The TNAN and a UE initiate an exchange of EAP-Request and EAP-Response messages including Identity as specified in IETF RFC 3748 [9] for link layer authentication of the UE by the TNAP. Upon completion of the EAP-Request/Response messages, an exchange of the EAP-5G messages are initiated once the UE receives an EAP-Request/5G-Start from the TNGF. The UE also at that time informs the upper layers that the access stratum connection is established.

An exchange of the NAS messages which are encapsulated in EAP-5G messages occur until the UE is authenticated by the 5GCN. Upon completion of the UE authentication and reception of the EAP-Success by the UE, the UE and the TNAP employs the TNAP key (or FT key, see clause 7.3A.7) to establish access specific layer-2 security such as 4-way handshake in case IEEE 802.11 [19] is used between the TNAP and the UE.

Upon completion of successful establishment of access specific layer-2 security, the UE is configured with an IP address by TNAN by e.g. DHCP and the UE initiates an IKE_SA_INIT exchange as specified in IETF RFC 7296 [6].

The UE establishes the IP based secure connection by establishing an IKE SA and first child SA for NAS signalling traffic to the TNGF over NWt. Once the UE establishes the IKE SA and the signalling IPsec SA with the TNGF, the UE initiates establishment of a TCP connection for transport of NAS message with TNGF, secured using the signalling IPsec SA. The UE and the TNGF exchanges NAS messages over the TCP connection once it is established. Additional child SAs (user plane IPsec SAs) can be established to transfer user plane traffic (see clause 7.5).

An example of an IKE SA and first child SA establishment procedure is shown in figure 7.3A.1-1.The figure illustrates that EAP messages are employed for the communication between the UE and the TNAP while the TNAP is transparent to the communication between the UE and the TNGF when employing EAP-5G messages. Link layer protocol is used to exchange these messages between the UE and the TNAN. The internal protocol used for the communications between the TNAP and the TNGF, is illustrated as dashed lines in this figure and is out of the scope of 3GPP.

Figure 7.3A.1-1: IKE SA and first child SA establishment procedure for UE registration over trusted non-3GPP access

***** Next changes *****

### 7.3A.7	Re-authentication procedure for the UE mobility between two TNAPs connected to the same TNGF

When the UE supporting FT procedure connects to another TNAP that supports FT procedure within the same mobility domain (see IEEE 802.11 [19]) of the old TNAP, the UE can perform re-authentication on the new TNAP based on FT procedure as specified in 3GPP TS 33.501 [5]. In that case the TNAN and the UE shall not initiate the establishment of the EAP session over non-3GPP access specified in clause 7.3A.2.

***** End of changes *****