3GPP TSG-CT1 Meeting #153	C1-251009

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.558 | CR | 0120 | rev | -1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service provisioning with Common EAS Bundle |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 3GPP TS 23.558 CR#0656(see S6-242655) agreed in SA6#61, proposed Common EAS Bundle as part of ECS Service provisioning. TS 24.558 needs to be aligned with the above SA6 changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The ECS Service Provisioning procedure is updated for identifying the Common EAS Bundle scenario and determine the EES(s) accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Common EAS bundle discovery and selection remains unsupported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

##### 7.2.2.2.2	EEC requesting service provisioning information using Eecs_ServiceProvisioning_Request operation

To request for the one-time service provisioning information, the EEC shall send an HTTP POST request (custom operation: "Request") to the ECS with the request URI set to"{apiRoot}/eecs-serviceprovisioning/<apiVersion>/request". And the body including the ECSServProvReq data structure, as specified in clause 8.1.5.2.2.

Upon receiving the HTTP POST message from the EEC, the ECS shall:

a)	process the EEC service provisioning request information;

b)	verify and check if the EEC is authorized to request service provisioning information from ECS;

c)	if the EEC is authorized to request service provisioning information from ECS, then the ECS:

1)	may obtain the UE's location as specified in clause 5.3 of 3GPP TS 29.122 [3];

2)	if the EdgeApp_2 feature is supported and if "plmnId" attribute is not provided within the "connInfo" attribute, the ECS may obtain the UE roaming status and serving PLMN identifier by invoking NEF monitoring API using the 3GPP core network capabilities. In case of UE is roaming, the ECS may use the serving PLMN identifier to identify the partner ECS information like ECS end point, DNN and S-NSSAI to update the "redirectedECS" attribute of the ECSServProvResp data type;

i)	if the EdgeApp_3 feature is supported and if "predictExpTime" attribute is provided by which the UE moves to the predicted location, the ECS may also identify those EES with the non instantiated EAS that can be instantiated before "predictExpTime" based on the EAS deployment time information received from ADAES (as specified in clause 8.8.2 of 3GPP TS 23.436 [9]) or locally available information;

3)	if the "acProfs" attribute is provided by the EEC without the "appGroupProfile" attribute in the "appInfo" attribute, the ECS identifies the EES(s) based on the provided "acProfs" attribute and the UE location, and if the enNB1 feature is supported, the "userLocation" attribute may be provided in the "locInf" attribute within the ECSServProvReq data type;

i)	if acSvcContSupp information is included in the AC Profile, the matching EES has to support ACRScenario indicated in the acSvcContSupp information; and

ii)	for each AC Profile, if eass information is included in the AC Profile, the ECS identifies the matching EES such that the EES profile matches easId information. ECS may also include EAS instantiation information using "easInstInfos" attribute in eass information;

4)	if the EdgeApp_2 feature is supported and the EEC provided the "appGroupProfile" attribute in the "appInfo" attribute;

i)	then the ECS identifies the matching EES(s) based on the application group identity shared in "appGrpId" attribute and may also utilize the location information in the "expectedSvcArea" attribute if provided; or

ii)	the ECS retrieves the EES(s) information corresponding to the application group ID as specified in clause 6.4 of 3GPP TS 29.558 [4];

NOTE 1:	How the EES determines the validity of the application group is implementation specific.

5)	if neither application group profile nor AC profiles(s) are provided:

i.	if available, the ECS identifies the EES(s) based on the UE-specific service information at the ECS and the UE location; and

ii.	ECS identifies the EES(s) by applying the ECSP policy (e.g. based on the UE location);

6)	if the EdgeApp_2 feature is supported:

i.	the ECS may identify the EES based on the EEC service continuity support information and EES service continuity support information; and

ii.	if the EEC provided the list of desired ECSP identifiers within the "ecspIds" attribute, the ECS shall identify the matching EES(s) based on the registered ECSP identifier in EES profile and the received list of desired ECSP identifiers; and

iii)	in case of no matching EES(s), the ECS identifies those partner ECS that satisfies the requirements. If required by the ECSP policies, the ECS may:

A)	trigger ECS discovery procedure as specified in clause 6.6 of 3GPP TS 29.558 [4] to discover the partner ECS(s); and

B)		trigger service provisioning information retrieval procedure as specified in clause 6.5 of 3GPP TS 29.558 [4] to obtain service provisioning information from the partner ECS; and

iv)	if the ECS is provisioned with authentication methods supported by matching EES(s) as specified in clause 6.3 of 3GPP TS 33.558 [7], then the ECS may include the "eesAuthMethods" attribute for each candidate EES(s) as specified in clause 8.1.5.2.9 in the ECSServProvResp and if multiple authentication methods are supported by the EES, then it is left to the EEC implementation to choose any method it supports when it communicates with the EES; and

7)	the ECS also determines other information that needs to be provisioned, e.g. identification of the EDN, EDN service area, EES endpoints; and

8)	if the EdgeApp_3 feature is supported:

i)	if the EEC provided "appInfo" attribute includes both the "appGroupProfile" attribute and the "easBundleInfos" attribute, then the ECS consider this as common EAS bundle scenario and identifies the EES(s) by using these information; and

d)	if the ECS is able to determine service provisioning information using the inputs in service provisioning request, UE-specific service information at the ECS or the ECSP's policy, then the ECS returns an HTTP "200 OK" status code response with the response body including the ECSServProvResp data structure which may include the lifetime of the provided EDN configuration information.

If the EdgeApp_2 feature is supported and in case of roaming, if the partner ECS(s) information has been identified in step c.2), the ECS sends a successful response including the "redirectedECS" attribute containing the list of ECS(s) configuration information to which the EEC shall redirect the service provisioning request.

If the inputs in service provisioning request do not match any EDN configuration information (i.e. there is no client side error), the ECS sends an HTTP "204 No Content" status code response code.

Otherwise, the ECS shall reject the service provisioning request and respond with an appropriate failure cause.

The EEC may cache the service provisioning information (e.g. EES endpoint). If the lifeTime attribute is included in the service provisioning response, then the EEC may cache and reuse the service provisioning information only for the duration specified by the lifeTime attribute.

The EEC may initiate service provisioning procedure with ECS(s) provided in the "redirectedECS" attribute of the "ECSServProvResp" data type. If the UE is roaming to a V-PLMN, then EEC may establish a connection with the partner ECS that can be a HR-SBO PDU session or an LBO PDU session.

The EEC may select one or more EES to perform EAS discovery, for multiple EES(s) case, if instantiable EAS information using "easInstInfos" attribute for an EAS is not available, or the instantiable EAS information using "easInstInfos" attribute is set to instantiated or instantiable.

The EEC may consider the instantiable EAS information using "easInstInfos" attribute and the associated instantiation criteria to mitigate the waste of EDN resources for EAS discovery. The EEC selects one EES, if the EAS instantiation status corresponding to the EASID requested by AC/EEC is instantiable but not yet instantiated (i.e., no instantiated EAS).

NOTE 2:	If the EAS instantiation fails based on the selected EES, the EEC retries the EAS discovery request to another EES ((e.g. selecting another one EES based on the instantiable EAS information).

NOTE 3:	How EEC maintains the service provisioning information is implementation specific.

# *** End of Changes ***