3GPP TSG-CT WG1 Meeting #159	C1-260452

# Goa, India, 9th – 13th February 2026													Revision of C1-260148

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.369 | CR | 0022 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cause#3 in Read Command Reject |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2026-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature) C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 5.3.3.5 specifies AIoT device functionality when receiving Write request and if the device is so low on energy, that is can only sent back a reject message. The same functionality is missing in the Read request, which is a critical flaw. The following statement in clause 5.3.3.5 is technically wrong, as this contradicts to requirements in clause 6.3. - Quote from 5.3.3.5: "Upon receiving a WRITE COMMAND message, if the write command procedure cannot be implemented because the energy will be run out, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #3 "Low energy". - Quote from 6.3: If the AIoT device receives an AIoT NAS message with a message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall return a STATUS message with cause #97 "message type non-existent or not implemented". Respective changes should apply also to Annex A.1 Backward compatibility analysis: The changes are backwards compatible, as the highly hypothetical existing AIOTF will correctly handle READ COMMAND REJECT message, even if it is not expecting the cause value #3 "Low energy". |  |  |  |  |  |  |  |  |
| Summary of change: |  | Technically incurrent statement in 5.3.3.5 is corrected. Missing functionality is added to Read command procedure execution in clause 5.3.2.5. The cause#3 can also be included in the Write Command Reject message. The respective statement in Annex A.1 now applies to all command messages generic. Editorial errors are fixed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Critical deficiencies remain in the spec, which likely will lead to interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.5, 5.3.3.5, A.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

#### 5.3.2.5	Read command procedure not accepted by AIoT device

Upon receiving a READ COMMAND message, if the command type specific parameter (e.g. the offset to read application data from the AIoT Device user memory, the length of the application data to read.) is invalid, the AIoT device shall send a READ COMMAND REJECT message with a cause set to #1 "Command type specific parameters invalid".

Upon receiving a READ COMMAND message, if the read command procedure cannot be performed because the energy will run out, the AIoT device shall send a READ COMMAND REJECT message with a cause set to #3 "Low energy" (i.e. the AIoT device has sufficient energy only for sending the reject message).

Upon receiving the READ COMMAND REJECT message, the AIOTF shall consider the read command procedure is unsuccessful and stop timer T1.

* * * * Next Change * * * *

#### 5.3.3.5	Write command procedure not accepted by AIoT device

Upon receiving a WRITE COMMAND message, if the command type specific parameter (e.g. the offset where to write the application data, the length of the application data to write) is invalid, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #1 "Command type specific parameters invalid".

Upon receiving a WRITE COMMAND message, if the write command procedure cannot be performed because the energy will run out, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #3 "Low energy" (i.e. the AIoT device has sufficient energy only for sending the reject message).

Upon receiving the WRITE COMMAND REJECT message, the AIOTF shall consider the write command procedure is unsuccessful and stop timer T2.

* * * * Next Change * * * *

Annex A (informative):
Cause values

# A.1	Cause values for the AIoT NAS protocol

Cause #1 – Command type specific parameters invalid

This cause is used to indicate that the command type specific parameter (e.g. the offset to read application data from the AIoT device user memory, the length of the application data to read, the offset where to write the application data, or the length of the application data to write) is invalid.

Cause #3 – Low energy

This cause is used to indicate that the command procedure cannot be performed because the energy will run out.

Cause #96 – Invalid mandatory information

This cause is used to indicate that the equipment sending this cause has received a message with a non-semantical mandatory IE error.

Cause #97 – Message type non-existent or not implemented

This cause is used to indicate that the equipment sending this cause has received a message with a message type it does not recognize either because this is a message not defined, or defined but not implemented by the equipment sending this cause.

Cause #111 – Error, unspecified

This cause is used to indicate that the requested read or write command was not executed successfully.

* * * * End of Changes * * * *